3GPP TSG-RAN WG2 Meeting #120	                                  R2-2212139

Toulouse, France, 14th November – 18th November, 2022

Agenda item:		8.5.4.1

Source:		Lenovo

Title:		Discussion on UE Feedback enhancements

Document for:		Discussion and Decision

# Introduction

- In the RAN2#120 meeting, RAN2 discussed UE feedback enhancement for XR traffic and reached the following agreement:

- introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). - RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. - Delay information consists of at least “remaining time”. - If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

In this contribution we further discuss UE feedback enhancements for XR capacity, e.g. how BSR can enhance capacity for XR.

# Discussion

In previous RAN1/2 meetings it was concluded that XR-awareness information provided from the core network is helpful for the XR-specific power saving enhancements and capacity enhancements.

In TR 38.838, a model for XR traffic was agreed for evaluating capacity and power saving aspects in a 5G NR system. The model considers the multi-flow nature of XR traffic, which includes for example video, audio and pose flows, in DL and/or UL directions. The flows described in the TR have different periodicity and packet delay budget (PDB) constraints.

SA2 has introduced the terminology of a PDU set. A PDU set is a set of packets (e.g. IP packets) that have dependency to each other and are important to the application for correct behaviour of the application (e.g. packets of a video frame that are required to decode the video). Such PDU set must have some common QoS treatment within the 3GPP network. This is particularly needed considering that individual IP packets within an XR PDU set (e.g., a video frame) are dependent on each other and must be all received within the expected PDB to be of any use by the end user application. The implication of PDU set concept is that IP packets should no longer be treated independently in the RAN. The concept of a PDU-Set enables enhancements to efficient resource management in 5GS, e.g. in NG-RAN. One such example enables cell capacity increase. In this example NG-RAN may take a decision to not deliver any PDU of a given PDU-Set when NG-RAN can assess that not all PDUs constituting that PDU set are feasible to be delivered within a required time.

### Delay-Aware scheduling

The gNB can take knowledge of PDU set delay into account in scheduling transmissions, e.g., by giving priority to transmissions close to their delay budget limit, and by not scheduling (e.g., UL) transmissions exceeding a PDU set delay budget. The UE can also take advantage of such knowledge to save UE’s power by determining if an UL transmission (e.g., PUCCH in response to PDSCH, UL pose, or PUSCH) corresponding to a transmission that exceeds its delay budget can be dropped. Additionally, UE does not need to wait for re-transmission of a PDSCH that will never occur (e.g., DRX retransmission timers can be stopped). For DL transmissions it is assumed that gNB is aware of the Remaining delay budget of the data pending for transmission, e.g., based on information provided by the SMF, and takes such knowledge into account in scheduling decisions.

For UL resource allocation, it would be necessary that UE provides some assistance information regarding the remaining delay budget of the data pending in its buffer to the gNB. For example, information of a remaining valid duration for retransmission of a TB, a request of no further retransmission of a TB can be transmitted along with HARQ-ACK feedback or other uplink control information (UCI). We think that UE should also provide information on the delay budget of the data for which UL resources are requested when sending a buffer status report. We think that the PSDB information provided to the RAN is not sufficient. Since NW is not aware of the exact arrival time of UL data in the buffer and hence can also not be sure about the remaining (valid) time of data being pending in the buffer for transmission, UE should provide this information within the BSR. In addition, for less signalling overhead, UE can report the remaining PDU set delay budget of the available PDU set with the lowest PSDB value if the remaining PDU set delay budget has not run out.

Similarly, gNB would benefit from a notification by UE about UL data - which were reported in a BSR previously - for which the delay budget is exceeded. gNB can take such knowledge into account for an efficient resource allocation, e.g. avoiding to issue further UL grants for data pending in UE’s buffer which is of no use for the user.

Proposal 1: UE should include delay information, e.g. remaining delay budget, associated with data being available for transmission within a BSR.

Proposal 2: UE should notify gNB when the delay budget is exceeded for data which has been previously reported in a BSR.

### BSR triggering and format enhancements

As mentioned above, IP packets belonging to the same PDU set are correlated to each other, and the end-user performance depends on if all these IP packets belonging to a single PDU set are successfully delivered. Providing the amount of data of the PDU set is beneficial for gNB to schedule the suitable UL grant size to schedule them as a whole. However, the legacy BSR procedure does not well support the PDU set level buffer size reporting.

- Current BSR format indicates all the available data volume with LCHs in an LCG. If there’s more than one PDU set available for transmission for an LCH, NW cannot identify the buffer size for each PDU set in the BSR and also cannot know the amount of data corresponding to the remaining delay budget information. RAN2 can discuss how to enhance the PDU set buffer size report format.

- There are 4 type of current BSR trigger event: 1) UL data with an LCH in an LCG with higher priority becomes available or UL data with an LCH in an LCG becomes available again after no UL data in an LCG is available; 2) number of padding bits is larger enough to include an BSR; 3) retxBSR-Timer expires and data is available in an LCG ; 4) periodicBSR-Timer expires. The event 1), 2) and 3) is not designed for the period arrival data. Due to the UL arrival jitter caused by variable coding delay depending on the variable packet size, it is difficult to set a suitable value of periodicBSR-Timer to adapt the periodical arrival with jitter. RAN2 can discuss whether the legacy BSR trigger events support the PDU set level buffer status report, if not, how to enhance the BSR trigger for a PDU set buffer size report.

Observation 1: Providing the buffer size of a PDU set is beneficial for gNB to schedule the suitable UL grant size to schedule the complete PDU set.

Proposal 3: RAN2 to discuss enhancements to the BSR trigger conditions and BSR format to support PDU set buffer size report.

Last RAN2 meeting agreed to introduce new BS table to reduce the quantisation errors. We analyse the quantisation errors for example of frame rate having 60 fps according to the statement in RAN1 TR 38.838 in the following table against the current BS 8-bit table in TS 38.321.

Table 1 quantisation errors analysis for traffic with frame rate= 60FPS, data rate =10/20 Mbps, GOP size=8

| Quantization error | Data Rate (Mbps) | a | Mean I size | Mean P size | I-frame size(byte) |  | P-frame size(byte) |  | Buffer Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Max | Min | Max | Min |  |
| 2415 | 10 | 1.5 | 30840 | 20560 | 46261 | 15420 | 30840 | 10280 | 48676 |
| 4700 | 20 | 1.5 | 61680 | 41120 | 92521 | 30840 | 61681 | 20560 | 97221 |
| 530 | 10 | 2 | 38836 | 19418 | 58254 | 19418 | 29127 | 9709 | 58784 |
| 899 | 20 | 2 | 77672 | 38836 | 116508 | 38836 | 58254 | 19418 | 117407 |

wherein:

- Data Rate: average data rate of a single stream video

- a: average size ratio between one I-frame/slice and one P-frame/slice

Even for one frame rate, the frame size varies with different data rate, rate of I-frame, rate of P-frame, group of size or size ratio between I-frame and P-frame, and the variable value range between maximum value and minimum value for I-frame will cover tens of thousands of bytes. Therefore, it is challenging to define a limited set of new buffer size tables which cover all the possible packet sizes to decrease the quantisation error even for one data rate. Therefore, RAN2 n need to discuss further the detailed design of the BS tables for XR traffic.

Observation 2: Since the frame size varies with the different data rate, rate of I-frame, rate of P-frame, group of size or size ratio between I-frame and P-frame, it is challenging to design a limit set of 8-bit buffer size tables which cover all the possible packet sizes and addressing the quantization errors.

Based on the description in TR 26.926, the IP packets for the XR traffic have importance parameter, which is assigned relative importance information (higher number means higher importance). Generally, I-frame is higher importance than P-frame, it’d better have different scheduling handling at NW based on the importance level. Currently, the I-frame and P-frames are in same QoS flow, and both are mapped to same DRB, in order to enable an efficient uplink scheduling RAN2 should discuss whether the importance parameter needs to be reflected in the scheduling procedure, e.g., in the BSR procedure.

Proposal 4: RAN2 to discuss the need of BSR enhancements to consider the importance of PDU set in case of I-frames and P-frames being mapped to the same QoS flow and same DRB.

# Conclusion

In this contribution, we discuss UE feedback enhancements for XR capacity. We have the following proposals:

Proposal 1: UE should include delay information, e.g. remaining delay budget, associated with data being available for transmission within a BSR.

Proposal 2: UE should notify gNB when the delay budget is exceeded for data which has been previously reported in a BSR.

Observation 1: Providing the buffer size of a PDU set is beneficial for gNB to schedule the suitable UL grant size to schedule the complete PDU set.

Proposal 3: RAN2 to discuss enhancements to the BSR trigger conditions and BSR format to support PDU set buffer size report.

Observation 2: Since the frame size varies with the different data rate, rate of I-frame, rate of P-frame, group of size or size ratio between I-frame and P-frame, it is challenging to design a limit set of 8-bit buffer size tables which cover all the possible packet sizes and addressing the quantization errors.

Proposal 4: RAN2 to discuss the need of BSR enhancements to consider the importance of PDU set in case of I-frames and P-frames being mapped to the same QoS flow and same DRB

# References

RAN1#110-e chairman note.

RAN2#119-e chairman note.