3GPP TSG-RAN WG2 Meeting #120                                                               R2-2212140

Toulouse, France, November 14-18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1486 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on HARQ buffers flushing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When timeAlignmentTimers are expired, only uplink HARQ buffers are required to be flushed in sec 5.2. When the MAC reset is performed, timeAlignmentTimers are considered as expired and sec 5.2. is referred to for uplink HARQ buffers flushing, while ‘soft buffers’ for the downlink HARQ processes are flushed in sec 5.12. That is, there is a separation for uplink and downlink HARQ buffers flushing at MAC reset. However, present specification in sec 5.2 mentions “flush all HARQ buffers” for timealignmentTimers expiry which causes ambiguity. During recent discussion in R2#119e, it was also found ambiguous for MBS broadcast HARQ buffer operation which has no impact of timeAlignmentTimers expiry. To make description unambiguous, it is reasonable to clarify with explcitly stating as “flush all uplink HARQ buffers” in sec 5.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify in sec 5.2 that when a timealignmentTimer expires, the MAC entity shall “flush all uplink HARQ buffers”. Impacted 5G architecture options: NR SA, NR-DC, (NG)EN-DC Impacted functionality: NR HARQ buffer handling Inter-operability: There are no inter-operability issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is potential ambiguity to implementers about HARQ buffer flushing upon timeAlignmentTimers expiry. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

#### <Start of Change>

## 5.2	Maintenance of Uplink Time Alignment

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	start or restart the timeAlignmentTimer associated with the indicated TAG.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for this TAG;

3>	start or restart the timeAlignmentTimer associated with this TAG.

2>	else if the timeAlignmentTimer associated with this TAG is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5; or

3>	when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE:

4>	stop timeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE as specified in clause 5.1.4a:

2>	apply the Timing Advance Command for PTAG;

2>	start or restart the timeAlignmentTimer associated with PTAG.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all uplink HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG:

3>	flush all uplink HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running. Furthermore, when the timeAlignmentTimer associated with the PTAG is not running, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell.

#### <End of Change>