3GPP TSG-RAN WG2 Meeting #120                                   R2-2212142

Toulouse, France, November 14-18, 2022

Agenda item:		6.21.1

Source:				Samsung

Title:					Discussion on per-FR gap

Document for:		Discussion & Decision

# 1 Introduction

Per-FR gap assumption of having separate resources for FR1 and FR2 in the UE implementations is no longer valid. To address this, RAN plenary #97e tasked RAN2 to progress the issue and select a fitting solution among the more fine-granular capability solutions or other solutions like needforgap to address the per-FR gap. In this contribution, we further discuss this issue and provide a way-forward.

# 2 Discussion

In RAN2#119bis-e meeting [1], per-FR gap issue was further discussed and RAN2 considered the alternatives, which came from RAN plenary.

Alt 1.1 (More fine grained capability for Per-FR-Gaps, 1 bit per BC),

Alt 1.3 (more fine grained capability for Per-FR-Gaps - limited by number of carriers), and

Alt 2 (Use similar framework/procedure as for ”NeedForGap”).

Among the initial set of solutions as above, RAN2 decided to exclude Alt 1.1 due to excessive signalling overhead. RAN2 made the following agreements for further progress:

Exclude Alt 1.1 for now.

On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

Alt 1.3 per BC considers introducing the independent gap and maximum number of CCs per BC instead of per FR gap capability for maximum number of CCs for the Alt 1.3. Whereas Alt 2 requires UE to indicate the need for gap based on the existing configuration provided to the UE in the RRC reconfiguration message from the network. We compare these solutions in the table 1.

TABLE 1: Comparison of different approaches for per-FR gap

| Approach | Pros | Cons |
| --- | --- | --- |
| Alt 1.3 | Low signalling overhead. Backward compatible. Simplicity. Less spec impact | Misses factors like MIMO layers/BW. Basic solution. |
| Alt 1.3 per BC | More flexibility. High granularity | High signalling overhead. High spec impact with more ASN.1 changes |
| Alt 2 | Existing mechanism, Low signalling overhead, More flexibility, High granularity | Support for DC scenario is not existing in Needforgap approach. Spec impact with procedural changes and ASN.1 changes |

Based on the comparative analysis, Alt 1.3 seems to be simple, efficient and can be quite readily incorporated. Alt 2 i.e. Needforgap approach seems quite flexible and high granularity solution, however, the support for DC scenarios is not existing due to MN-SN coordination complexity and it may also be difficult to address all the considerations and work out the DC solution for Alt 2 given the limited time in hand for Rel-17. Considering this, Alt 1.3 seems the best choice.

Proposal: RAN2 to agree on Alt 1.3 based solution for per FR-gap capability and adopt it from Rel-17.

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposal:

Proposal: RAN2 to agree on Alt 1.3 based solution for per FR-gap capability and adopt it from Rel-17.

# 4 Reference

[1] RAN2#119bis-e meeting report v1