3GPP TSG-RAN WG2 Meeting#120		       					R2-2212183

Toulouse, France, November 2022

Source: 			ZTE corporation, Sanechips

Title: 	Consideration on cell selection and reselection related to NES for NR

Agenda item:		8.3.4

Document for: 	Discussion and Decision

# Introduction

For 5G, the BS power consumption accounts for about 58% of cellular network power consumption. In RAN2#119e meeting, the following agreements have been made for network energy power saving:

Solution groups:

- Adaption of MIB/SSB/SIB

-  partial/simplified SSB

2	Increase of SSB/SIB periodicity

3	On demand SSB/SIB1 (FFS if there are enhancements for other SIBs)

- FFS for on-demand MIB

4	Receiving SSB/SIB on one carrier/cell and performing access to another carrier/cell

5	Handover/Fast PCell change for NES

- CHO or new configuration

- group HO

6	Resource adaptation (frequency and time domain)

- Including PRACH, SRS, PUSCH, PUCCH resources and periodicities

- cell DTX/DRX

- measurement

- reference signal type and configuration of reference signal pattern for connected mode

- BWP adaptation

7	Any Cell activation/re-activation or UE wake up request signal (connected/idle)

8	Paging enhancements (includes paging-less solutions)

9	Cell selection/reselection (ie. cell prioritization also including legacy UEs)

Things to study

- Study group configuration and signalling for transitions for different solutions

- pre-configuration and L1/L2 signaling to trigger change of configuration

2	Identify/capture RAN2 impact to legacy for the different solutions

3	Awareness of the NES states at the UE side for the different solutions

4	Aim to minimize DL signalling for NES

5	Consider UE complexity and energy consumption

6	UE assistance information for the specific network energy technique, it’s benefits and impact to UE/NW

In RAN2#119bis-e meeting, an offline email discussion for [NES] Cell Selection/Reselection and SSB/SIB-less has been performed and the following agreements have been made further for network energy power saving:

Agreements:

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

In this paper, we will share our understandings on cell selection and re-selection related to NES for NR, and give our proposals.

# Discussion

## NES cell differentiation

Based on the RAN2 119-e and RAN2#119bis-e meeting, the following features may be supported for NES purposes:

- SSB-less and SIB-less cell

- Cell supporting cell specific DTX and/or DRX

- Cell supporting mobility enhancements, including CHO and group mobility etc

Based on the current specification, after cell selection and/or cell re-selection, UE will perform frame synchronization with cell and read the SIB of the cell. If the UE is unable to acquire the SIB1, it will consider the cell as barred. So, UE shall not select or re-select a SSB-less and SIB-less cell during cell selection or cell re-selection procedure.

Observation 1: During cell selection or cell re-selection procedure, UE shall not select or re-select a SSB-less and SIB-less cell.

Proposal 1: For cell selection and re-selection issue related to NES for NR, SSB-less and SIB-less cell is not considered.

Since cell specific DTX/DRX and mobility enhancements for NES etc may be supported, which can save gNB energy and can only be used for UEs supporting these features. If the UE capability can match the cell capability properly, the gNB energy saving can be performed well. Thus, cell specific DTX/DRX and mobility enhancements for NES etc can be considered in cell selection and re-selection.

Proposal 2: For cell selection and re-selection issue related to NES for NR, the cell supporting cell specific DTX/DRX and mobility enhancements for NES etc can be considered.

In the following discussion, the features of cell specific DTX/DRX and mobility enhancements for NES etc are called NES features, the cells supporting cell specific DTX/DRX and mobility enhancements for NES etc is called NES cells, and the UEs supporting cell specific DTX/DRX and mobility enhancements for NES etc is called NES-capable UEs.

## Network load consideration

In network with light load, e.g. one cell/frequency can bear all the traffic load in the area, the UEs can be converged to one legacy cell/frequency and switch off the other cell/frequency in the area for network power saving.

In network with middle or high load, load balance may be necessary, e.g. the UEs are distributed among multiple cells/frequencies for better QoE. In which case, if the legacy UEs are distributed in the legacy cells/frequencies, and NES-capable UEs are distributed in the NES cells, the NES features can be used as soon as possible for power saving. The UEs distribution between cells/frequencies are usually performed in cell re-selection procedure.

Observation 2: Whether the NES-capable UEs need to prioritize NES cells for cell re-selection depends on the network load.

Considering that load balance usually is inter-frequency load balance, and legacy cell/NES cell can be deployed in different frequencies, only frequency level prioritization need to be supported for NES capable UEs prioritizing NES cells for cell re-selection.

To prioritize NES frequency for cell re-selection, two ordinary ways are used:

Alt 1: the NES-capable UEs shall always consider the NES cells to be the highest priority.

Alt 2: add a Qoffet to the NES frequency, whenever the serving cell belongs to the NES frequency or the neighbor cell belongs to the NES frequency. E.g. enhance the cell re-selection procedure:

Rs = Qmeas,s +Qhyst - Qoffsettemp + QoffsetNES-Freq Rn = Qmeas,n -Qoffset - Qoffsettemp + QoffsetNES-Freq

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |
| QoffsetNES-Freq | Qoffset applied to NES frequency for NES-capable UE. If the UE supports NES features(e.g. cell specific DTX/DRX and mobility enhancements for NES etc), and QoffsetNES-Freq is configured for the frequency of the cell, QoffsetNES-Freq is applied; else, QoffsetNES-Freq is not applied. |

Since UE in RRC_IDLE or RRC_INACTIVE state can not be aware of the network load, based on the observation 3, it is necessary for network to control whether to prioritize NES frequency for NES-capable UE cell re-selection based on the network load, whatever the alt 1 or alt 2 is used. For load balance flexibility(e.g. not all NES-capable UEs will be balanced to NES cells), it is suggest to add a Qoffet to the NES frequency to prioritize NES frequency for NES-capable UE cell re-selection.

Proposal 3: Whether to prioritize NES frequency for NES-capable UE cell re-selection depends on the whether QoffsetNES-Freq is configured for the frequency, and if configured, QoffsetNES-Freq is used to prioritize NES frequency for NES-capable UE cell re-selection.

Since Qoffsetfrequency is already used for cell re-selection, which can prioritize the UEs to re-select configured frequency. Thus, the legacy Qoffsetfrequency can be used to prioritize the legacy UEs to re-select a legacy frequency, and the new QoffsetNES-Freq (or new QoffsetNES-Freq plus legacy Qoffsetfrequency) can be used to prioritize the NES-capable UEs to re-select a NES frequency.

Observation 3: The legacy Qoffsetfrequency can be used to prioritize the legacy UEs to re-select a legacy frequency. The new QoffsetNES-Freq (or new QoffsetNES-Freq plus legacy Qoffsetfrequency) can be used to prioritize the NES-capable UEs to re-select a NES frequency.

Based on the observations and proposals, the TP for TR 38.864 is also provided in the appendix.

# Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are provided:

Observation 1: Observation 1: During cell selection or cell re-selection procedure, UE shall not select or re-select a SSB-less and SIB-less cell.

Observation 2: Whether the NES capable UEs need to prioritize NES cells for cell re-selection depends on the network load.

Observation 3: The legacy Qoffsetfrequency can be used to prioritize the legacy UEs to re-select a legacy frequency. The new QoffsetNES-Freq (or new QoffsetNES-Freq plus legacy Qoffsetfrequency) can be used to prioritize the NES-capable UEs to re-select a NES frequency.

Proposal 1: For cell selection and re-selection issue related to NES for NR, SSB-less and SIB-less cell is not considered.

Proposal 2: For cell selection and re-selection issue related to NES for NR, the cell supporting cell specific DTX/DRX and mobility enhancements for NES etc can be considered.

Proposal 3: Whether to prioritize NES frequency for NES-capable UE cell re-selection depends on the whether QoffsetNES-Freq is configured for the frequency, and if configured, QoffsetNES-Freq is used to prioritize NES frequency for NES-capable UE cell re-selection.

Based on the observations and proposals, the TP for TR 38.864 is also provided in the appendix.

# References

[1] RP-220297, Revised SI: Study on network energy savings for NR, March, 2022

[2] R2-2210995，Report of [Offline-302][NES] Cell Selection/Reselection and SSB/SIB-less (Huawei)

[3] R2_119bis-e_Notes_R18 UAV NES (Diana)_18-10-22_14-40.docx

# Appendix

---------------------------------------------------------Start of text proposal-------------------------------------------------------

6.X	Techniques in cell selection and re-selection domain

6.X.1 NES cell differentiation for cell selection and re-selection

During cell selection and re-selection procedure, UE shall not select or re-select a SSB-less and SIB-less cell to camp.

During cell selection and re-selection procedure, the cell supporting cell specific DTX/DRX and mobility enhancements for NES etc can be prioritized per frequency level for NES capable UEs. Whether NES capable UEs will prioritize the cell supporting cell specific DTX/DRX and mobility enhancements for NES etc for cell re-selection is indicated by network.

6.X.2 NES frequency prioritization for NES-capable UE

To prioritize NES frequency for cell re-selection, an QoffsetNES-Freq can be indicated for the NES frequency in SIB.

During cell re-selection, the QoffsetNES-Freq is used as follows:

Rs = Qmeas,s +Qhyst - Qoffsettemp + QoffsetNES-Freq Rn = Qmeas,n -Qoffset - Qoffsettemp + QoffsetNES-Freq

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |
| QoffsetNES-Freq | Qoffset applied to the NES frequency for NES-capable UEs. If the UE supports NES features(e.g. cell specific DTX/DRX and mobility enhancements for NES etc), and QoffsetNES-Freq is configured for the frequency of the cell, QoffsetNES-Freq is applied; else, QoffsetNES-Freq is not applied. |

---------------------------------------------------------End of text proposal-------------------------------------------------------