3GPP TSG-RAN WG2 Meeting#120		       					   R2-2212184

Toulouse, France, November 2022		  	 	        Update of R2-2210666

Source: 			ZTE corporation, Sanechips

Title: 	Techniques in various domains and UE assistance information for NES

Agenda item:		8.3.6

Document for: 	Discussion and Decision

# Introduction

For 5G, the BS power consumption accounts for about 58% of cellular network power consumption. Besides the impact on climate change, the network OPEX tends to account for around 25% of the operator cost base, wherein over 90% of network costs are spent on energy. In RAN#95-e, the SI of Study on network energy savings for NR has been approved [1] with the following RAN2 related objectives:

Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

In RAN1#109, some potential energy saving techniques have been discussed, and the following agreement has been reached:

Agreement

Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects:

- potential methods of reducing/adapting transmission/reception of common channels/signals, e.g. SSB, SIB1, other SI, paging, PRACH, and its impact to initial access procedure, cell (re)selection, handover, synchronization and measurements performed by the idle/inactive/connected UE;

  - potential methods of reducing transmission/reception of common channels/signals can include no- or reduced-transmission/reception, increased periodicity, enablement of on-demand transmission/reception of common channels/signals, or offloading of common channels/signals to other carriers or use of light or relaxed versions of common channels /signals

- potential methods of reducing/adapting transmission/reception of periodic and semi-persistent signals and channels configuration such as CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, positioning RS (PRS), etc.

- semi-static and/or dynamic cell on/off in one or more granularity, e.g. /subframe/slot/symbol; some examples are:

  - Cell/network node activation request by the UE, for example using signal/channel from UE for gNB’s wake-up request

  - enhancements to L1/L2 based mobility to efficiently enable a network node (e.g. TRP, repeater) on/off operation within a cell (within network energy saving SI scope)

  - signaling enhancements for indication of semi-static and/or dynamic cell/subframe/slot/symbol on/off duration

- support of periodic and/or on-demand reference signal(s) from the gNB to aid discovery of a cell;

- dynamic adaptation of UE C-DRX configurations in a UE-group or cell-specific manner

- Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities

  - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal

  - including technique to allow discovery and measurement of cells in sleep or dormant states

- UE assistant information facilitating BS time domain adaptation

Note: For all techniques above, study of time domain techniques is applicable for single component carrier and multi-component carrier cases. Use of UE grouping and its interaction with proposed techniques can be considered.

In this paper, we will share our understanding on some potential techniques in time, frequency and spatial domain and UE assistance information for network energy savings.

# Discussion

## Time and frequency domain energy saving techniques

Dynamic cell on/off

When the load is distributed in multiple cells and there is light load in each cell, the network can consider to converge load to some cells and turn off the others dynamically to save power.

To make wise decision on which cell(s) can be shutdown, the load information of the cells should be exchanged. In DC and/or CA cases, the Scell can be easily deactivated and shutdown without service handover, but the Pcell shutdown will impacts the UEs on-line much. And based on the current specification, the load information of the cell and the neighbor cell/carrier cannot differentiate Pcell load information and Scell load information, which is difficult for gNB to decide to whether a cell can be shut down and which cell can be shutdown. Thus, we propose to differentiate PCell and SCell when exchange the load information between network nodes.

Proposal 1: In DC and/or CA cases, the load information can be measured and exchanged between network nodes differentiating Pcell and Scell.

## Spatial domain energy saving techniques

Beam on/off operation

In NR multiple carriers network or NR/LTE network, when the load of a NR beam is light, the service in the beam can be converged to some of the other beams or carriers and shutdown the beam to save gNB power. E.g. one or more of beam can switch their service to other cell/carrier/beam and then shut down their own power to save energy.

To shutdown beam for energy saving, the following aspects should be considered:

- The load information of the beam and the neighbor cell/carrier/beam, which are used for gNB to determine whether the beam can be shut down and which beam can be shutdown should be exchanged between network nodes

- When the gNB decides to shut down a beam, the gNB should switch the service of the beam to another other cell/carrier/beam, and restrict the new users from accessing to the beam.

Proposal 2: Dynamic beam on-off can be considered for network energy saving with the following enhancement supported:

- Beam level load information measurement and exchange between network nodes;

- Beam level access control.

## UE assistance information for cell and beam on/off

For potential network energy saving techniques, it should balance between the network energy saving and UE performance. If the UE can report some information to help network, the network can adjust the energy saving techniques to reduce the impact on user experience.

We understand the following types of information would be useful for network energy saving:

The traffic related information:

- Maximal delay within which a given data packet could be transmitted;

- Expected total traffic volume;

The mobility related information:

- Visited cell records with visited cell ids and camped time duration;

Other assistance information form UE:

- Indication of large latency for traffic

The traffic related information would be helpful for NW to identify the traffic pattern in the time domain and decide when to turn off some symbols or slots. The mobility related information can help NW predict the UE distribution and decide how to turn on/off some cells, carriers or beams. While the indication of a large latency for traffic, can report bad user experience from the UE side so that NW can relax some energy saving techniques to improve user experience as requested.

Proposal 3: The following assistance information provided from UE can be considered to help with network energy saving techniques:

The traffic related information:

- Maximal delay within which a given data packet could be transmitted;

- Expected total traffic volume;

The mobility related information:

- Visited cell records with visited cell ids and camped time duration;

Other assistance information from UE:

- Indication of large latency for traffic

# Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: In DC and/or CA cases, the load information can be measured and exchanged between network nodes differentiating Pcell and Scell.

Proposal 2: Dynamic beam on-off can be considered for network energy saving with the following enhancement supported:

- Beam level load information measurement and exchange between network nodes;

- Beam level access control.

Proposal 3: The following assistance information provided from UE can be considered to help with network energy saving techniques:

The traffic related information:

- Maximal delay within which a given data packet could be transmitted;

- Expected total traffic volume;

The mobility related information:

- Visited cell records with visited cell ids and camped time duration;

Other assistance information from UE:

- Indication of large latency for traffic.

Based on the proposals, the TP for TR 38.864 is also provided in the appendix.

# References

[1] RP-220297, Revised SI: Study on network energy savings for NR, March, 2022

# Appendix

---------------------------------------------------------Start of text proposal-------------------------------------------------------

6.X	Techniques in cell and beam on/off enhancement

6.X.1 Dynamic cell on/off enhancement

When the network load is light load in each cell, the network can consider to converge load to some cells and turn off the others dynamically to save power.

To make wise decision on which cell(s) can be shutdown, the load information of the cells should be exchanged. In DC and/or CA cases, the Scell can be easily deactivated and shutdown without service handover, but the Pcell shutdown will impacts the UEs on-line much. And based on the current specification, the the load information of the cell and the neighbor cell/carrier cannot differentiate Pcell load information and Scell load information, which is difficult for gNB to decide to whether a cell can be shut down and which cell can be shutdown.

For efficient cell on/off dynamically, cell load information should be measured (and exchange between network nodes) differentiating Pcell and Scell.

6.X.2 Dynamic beam on/off enhancement

In NR multiple carriers network or NR/LTE network, when the load of a NR beam is light, the service in the beam can be converged to some of the other beams or carriers and shutdown the beam to save gNB power. E.g. one or more of beam can switch their service to other cell/carrier/beam and then shut down their own power to save energy.

When the gNB decides to shut down a beam, the gNB should switch the service of the beam to another other cell/carrier/beam, and restrict new users from accessing to the beam.

Thus, to shutdown beam for energy saving, the following enhancements are supported:

- Beam level load information measurement(and exchange between network nodes);

- Beam level access control.

6.X.3 UE assistance information for cell and beam on/off

For cell and beam on/off techniques, it should balance between the network energy saving and UE performance. The UE can report the following information to help network to decide and/or adjust the energy saving techniques to reduce the impact on user experience:

The traffic related information:

- Maximal delay within which a given data packet could be transmitted;

- Expected total traffic volume;

UE mobility related information:

- Visited cell records with visited cell ids and camped time duration;

Other assistance information from UE:

- Indication of large latency for traffic.

---------------------------------------------------------End of text proposal-------------------------------------------------------