3GPP TSG RAN WG2 Meeting #120	                                    R2-2212194

# Toulouse, France, 14th – 18th November, 2022

Agenda item:		6.6.3

Source:			Huawei, HiSilicon

Title:		HARQ process offset configuration and repetition capability for SDT

Document for:	 	Discussion and decision

# Introduction

In [1], it was proposed to disallow the configuration of harq-ProcID-Offset by employing the following change in RRC specifications:

This issue was discussed during RAN2#119 meeting and the views were gathered as part of the offline discussion summarized in [2], as follows:

Question 11: Do you agree that the field harq-ProcID-Offset can be configured for SDT? <SOME PARTS OMITTED> Summary - LG, Lenovo, NEC, Intel, Nokia, Qualcomm, CATT, Apple, China Telecom (9 companies) think it is not needed to add this field for CG-SDT configuration - ZTE, Xiaomi, HW, OPPO, Sharp, IDC, VIVO (7 companies) think it can be configured for CG-SDT Proposal11: R2 to further discuss whether the field harq-ProcID-Offset can be configured for CG-SDT. (9/16)

After some further discussion during an online session, it was concluded to further investigate this issue:

FFS to check in this meeting - Allow the field harq-ProcID-Offset can be configured for CG-SDT (add the case for CG-SDT).  Double confirm that there is nothing that breaks in the MAC when CG retx timer is configured.

Eventually, no conclusion was achieved during RAN2#119 meeting due to differing opinions between the companies.

Furthermore, according to ANS.1, the signaling in TS 38.331 reuses the existing ConfiguredGrantConfig and hence the signalling allows configuration of parameters related to repetition for the CG-SDT, which means that repetition is supported for CG-SDT. However, the repetition related issue is still discussed in RAN1 because the resource validation of CG PUSCH repetition transmission is not exactly the same as in legacy specifications. Additionally, for non-SDT CG configurations, whether to support repetition is optional with capability while there is no such capability for SDT currently.

In this contribution we present our view on harq-ProcID-Offset configuration and repetition capability for SDT.

# Discussion

## HARQ process offset configuration

In the current specifications, the harq-ProcID-Offset and harq-ProcID-Offset2 fields are part of ConfiguredGrantConfig IE which is reused already for CG-SDT configuration. harq-ProcID-Offset is used for operation in unlicensed spectrum and is always configured together with cg-RetransmissionTimer-r16 while harq-ProcID-Offset2 is used for licensed spectrum and is never configured together with cg-RetransmissionTimer-r16. Their field descriptions are as follows:

harq-ProcID-Offset For operation with shared spectrum channel access configured with cg-RetransmissionTimer-r16, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)].

harq-ProcID-Offset2 Indicates the offset used in deriving the HARQ process IDs, see TS 38.321 [3], clause 5.4.1. This field is not configured together with cg-RetransmissionTimer-r16. If the field harq-ProcID-Offset2-v1700 is present, the UE shall ignore the harq-ProcID-Offset2-r16.

As can be seen, there is nothing that prevents either harq-ProcID-Offset or harq-ProcID-Offset2 to be configured for CG-SDT.

Observation 1: According to current RRC specifications, either harq-ProcID-Offset or harq-ProcID-Offset2 can be configured for CG-SDT.

Observation 2: In case multiple CG-SDT configurations are configured, harq-ProcID-Offset would be used for CG-SDT configuration on unlicensed spectrum while harq-ProcID-Offset2 would be used in case CG-SDT is configured for licensed spectrum.

From RRC specifications alone, both harq-ProcID-Offset and harq-ProcID-Offset2 could be used without any additional specification change, which is aligned with the assumption that no specific changes are introduced to make SDT work in unlicensed spectrum.

However, one more thing that has to be checked, according to the agreement above, is whether configuring cg-RetransmissionTimer-r16 will not break SDT procedures in MAC layer, since another retransmission timer, i.e. cg-SDT-RetransmissionTimer, was agreed for SDT operation. This parameter is defined as follows in RRC specifications:

cg-SDT-RetransmissionTimer Indicates the initial value of the configured grant retransmission timer used for the initial transmission of CG-SDT with CCCH message (see TS 38.321 [3]) in multiples of periodicity.

In MAC, the retransmission timers are defined as follows:

RRC configures the following parameter when retransmissions on configured uplink grant is configured: - cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process; - cg-SDT-RetransmissionTimer: the duration after a configured grant (re)trasnmission of a HARQ process of the initial CG-SDT transmission with CCCH message when the UE shall not autonomously retransmit the HARQ process.

We can see that both timers are used to control the autonomous retransmissions over the configured grant, but the cg-SDT-RetransmissionTimer is used only for initial SDT transmissions while cg-RetransmissionTimer is used for all transmissions over the CG, but only in unlicensed spectrum. These timers are used in the current MAC specifications in several places. In most of the places the timers can work together and do not interfere with each other. However, in section 5.4.1 of MAC specifications, there is a procedure which may not work as intended in case both timers are configured at the UE:

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall: 1> if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or 1> if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell: 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; 2> if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured and cg-SDT-RetransmissionTimer is not configured (i.e. new transmission): 3> if there is an on-going CG-SDT procedure and PDCCH addressed to the MAC entity's C-RNTI has been received; or 3> if there is no on-going CG-SDT procedure: 4> consider the NDI bit for the corresponding HARQ process to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 2> else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process: 3> if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission): 4> consider the NDI bit to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 3> else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant): 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 2> else if the cg-SDT-RetransmissionTimer is configured and not running for the corresponding HARQ process; 3> if the configured uplink grant is for the initial transmission for the CG-SDT with CCCH message (i.e., initial new transmission); or 3> if the configuredGrantTimer is not running or not configured, and PDCCH addressed to the MAC entity's C-RNTI has been received after the initial transmission of the CG-SDT with CCCH message (i.e., subsequent new transmission): 4> consider the NDI bit to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 3> else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant for initial transmission of CG-SDT with CCCH message or for its retransmission; and 3> if PDCCH addressed to the MAC entity's C-RNTI has not been received (i.e., retransmission for initial CG-SDT transmission): 4> consider the NDI bit to have not been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

We can see that the second condition, highlighted in blue, will never be triggered in case cg-RetransmissionTimer for the corresponding HARQ process is configured and not running. In other words, the part highlighted in blue will only be triggered in case cg-RetransmissionTimer has a value higher than cg-SDT-RetransmissionTimer.

Observation 3: In case both cg-RetransmissionTimer and cg-SDT-RetransmissionTimer are configured, cg-SDT-RetransmissionTimer will only be utilized in case it has value higher than cg-RetransmissionTimer. Otherwise, retransmissions for initial CG-SDT transmission will be controlled by cg-RetransmissionTimer.

It could be argued that this was not the original intended behaviour and some amendments to MAC specification could be done, e.g. it could be clarified that cg-RetransmissionTimer is not used for initial CG-SDT transmission. On the other hand, there seems to be nothing wrong with the current procedure, other than cg-SDT-RetransmissionTimer being not useful in some SDT configurations in unlicensed spectrum. Hence, such changes are non-essential and there is nothing broken in the procedures even without such changes. Based on this, we propose the following:

Proposal 1: No specification changes are needed for support of SDT over NR-U, in particular there is no need to clarify field descriptions or configuration conditions of harq-ProcID-Offset and harq-ProcID-Offset2 as well as cg-RetransmissionTimer and cg-SDT-RetransmissionTimer fields.

## Repetition capability for SDT

In RAN1#108-e, after several meetings’ discussion, RAN1 could not reach consensus on whether to support repetitions or not, so in RAN1 LS in [3], RAN1 asked RAN2 to decide on repetitions for CG-SDT. RAN2 provided the following reply on repetitions in RAN2 reply LS in [4]: “With regards to the above issue about repetition for CG-SDT, the signalling in TS 38.331 reuses the existing ConfiguredGrantConfig and hence the signalling allows configuration of parameters related to repetition (i.e. repK, repK-RV (including repK-r17), pusch-RepTypeIndicator-r16 and frequencyHoppingPUSCH-RepTypeB-r16) within this IE”.

In consequence, CG-SDT may be configured with repetitions. However, as mentioned in the introduction section RAN1 has still not worked out the details of how repetitions work for CG-SDT. In R15 and R16, data is transmitted only in RRC_Connected mode and there is no association between CG Type 1 resources and SSBs. Once configured, Type 1 configured grant resources are considered activated/valid. While for R17 SDT procedure in RRC_Inactive, considering that configured grant resources of the CG-SDT are associated with SSB, the UE needs to determine whether each configured grant is valid when performing the CG-SDT. RAN1 discussed how to understand the repetition term if the repetition is configured for SDT. From the perspective of RAN1, there could be two possibilities, one is that the multiple PUSCH repetitions are regarded as one PUSCH occasion, the other is that multiple PUSCH repetitions are regarded as separate PUSCH occasions. Currently, no consensus can be reached in RAN1. Anyway, we think that the CG PUSCH repetition supported by R17 SDT is not exactly the same as the existing PUSCH repetition supported by R15/16.

Observation 4: RAN1 has still not worked out the details of repetitions for CG-SDT.

Furthermore, the usefulness of repetitions for CG-SDT is unclear as the UE determines that the downlink pathloss reference RSRP is higher than the sdt-RSRP-Threshold before initiating SDT. This ensures that the SDT can be performed only when the signal quality is good, i.e., the UE is close to the cell center. Therefore, the probability of successful uplink CG PUSCH transmission of the UE performing SDT is relatively high. In this case, repetitions lead to a waste of radio resources and power consumption of the UE while overhead and power consumption reduction were the main goals of SDT work. This is unfriendly especially to UEs that support the SDT function and have high power consumption requirements, e.g. IoT sensors.

Observation 5: Usefulness of CG-SDT repetitions is doubtful in most scenarios while it negatively impacts radio resources overhead and UE’s power consumption.

In Rel-15 and Rel-16, the capability of repetitions is defined as follows.

| type1-PUSCH-RepetitionMultiSlots Indicates whether the UE supports Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value equal to 2, 4, or 8 with a single repetition of the transport block within each slot, and redundancy version pattern as indicated by UL-TWG-RV-rep. A UE supporting this feature shall also support Type 1 PUSCH transmissions with configured grant as specified in TS 38.214 [12] with UL-TWG-repK value of one. This applies only to non-shared spectrum channel access. For shared spectrum channel access, type1-PUSCH-RepetitionMultiSlots-r16 applies. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| pusch-RepetitionTypeB-r16 Indicates whether the UE supports PUSCH repetition type B, as specified in 6.1.2 of TS 38.214 [12]. | FS | No | N/A | N/A |

Based on the above analysis, we think that a separate repetition capability should be introduced to indicate whether the repetition can be supported by UE for SDT, e.g.:

| - Definitions for parameters | - Per | - M | - FDD-TDD - DIFF | - FR1-FR2 - DIFF |
| --- | --- | --- | --- | --- |
| pusch-Repetition-SDT Indicates whether the UE supports PUSCH repetition typeA and typeB for SDT, as specified in TS 38.214 [12]. | UE | No | N/A | N/A |

Based on this capability, the network may decide whether CG-SDT with repetitions can be configured to the UE. However, this does not affect the repetition capability of data transmission when the UE is in RRC Connected mode.

Proposal 2: Introduce a new separate PUSCH repetition capability for CG-SDT.

In the example above, we propose to introduce a single capability covering both typeA and typeB repetitions. In case we would like to allow one of typeA and typeB repetitions to be supported by the UE during SDT, we can consider also other options, i.e.:

- Introduce separate capability bits for typeA and typeB repetitions for CG-SDT, e.g. type1-PUSCH-RepetitionMultiSlots-SDT and pusch-RepetitionTypeB-SDT

- Link the new SDT capability with the existing repetition capabilities, i.e. if the UE indicates pusch-Repetition-SDT, it means the UE supports either typeA or typeB or both for SDT, depending on indication of the existing capabilities (type1-PUSCH-RepetitionMultiSlots and pusch-RepetitionTypeB).

Proposal 3: In case a single PUSCH repetition capability for CG-SDT covering both typeA and typeB repetitions is not preferred by RAN2, the following options can be considered:

- Introduce separate capability bits for typeA and typeB repetitions for CG-SDT, e.g. type1-PUSCH-RepetitionMultiSlots-SDT and pusch-RepetitionTypeB-SDT

- Link the new SDT capability with the existing repetition capabilities, i.e. if the UE indicates pusch-Repetition-SDT, it means the UE supports either typeA or typeB or both for SDT, depending on indication of the existing capabilities (type1-PUSCH-RepetitionMultiSlots and pusch-RepetitionTypeB).

# Conclusion

Based on the discussion in this paper, the following is proposed:

HARQ process offset configuration

Observation 1: According to current RRC specifications, either harq-ProcID-Offset or harq-ProcID-Offset2 can be configured for CG-SDT.

Observation 2: In case multiple CG-SDT configurations are configured, harq-ProcID-Offset would be used for CG-SDT configuration on unlicensed spectrum while harq-ProcID-Offset2 would be used in case CG-SDT is configured for licensed spectrum.

Observation 3: In case both cg-RetransmissionTimer and cg-SDT-RetransmissionTimer are configured, cg-

SDT-RetransmissionTimer will only be utilized in case it has value higher than cg-RetransmissionTimer. Otherwise, retransmissions for initial CG-SDT transmission will be controlled by cg-RetransmissionTimer.

Proposal 1: No specification changes are needed for support of SDT over NR-U, in particular there is no need to clarify field descriptions or configuration conditions of harq-ProcID-Offset and harq-ProcID-Offset2 as well as cg-RetransmissionTimer and cg-SDT-RetransmissionTimer fields.

Repetition capability for SDT

Observation 4: RAN1 has still not worked out the details of repetitions for CG-SDT.

Observation 5: Usefulness of CG-SDT repetitions is doubtful in most scenarios while it negatively impacts radio resources overhead and UE’s power consumption.

Proposal 2: Introduce a new separate PUSCH repetition capability for CG-SDT.

Proposal 3: In case a single PUSCH repetition capability for CG-SDT covering both typeA and typeB repetitions is not preferred by RAN2, the following options can be considered:

- Introduce separate capability bits for typeA and typeB repetitions for CG-SDT, e.g. type1-PUSCH-RepetitionMultiSlots-SDT and pusch-RepetitionTypeB-SDT

- Link the new SDT capability with the existing repetition capabilities, i.e. if the UE indicates pusch-Repetition-SDT, it means the UE supports either typeA or typeB or both for SDT, depending on indication of the existing capabilities (type1-PUSCH-RepetitionMultiSlots and pusch-RepetitionTypeB).

# References

- R2-2207416	Analysis on remaining issues for SDT	CATT	discussion	Rel-17

- R2-2208912	Summary of [AT119-e][302][Sdata] UP open issues and CR to 38.321 (Huawei), Huawei, HiSilicon

- R2-2204445	Reply LS on the physical layer aspects of small data transmission	RAN1

- R2-2206828	Reply LS on the L1 related agreements for SDT	RAN2