3GPP TSG-RAN WG2 Meeting #120	R2-2212210

Toulouse, France, 14th – 18th Nov, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on slice based cell reselection

Agenda Item:	6.8

Document for:	Discussion and decision

- Introduction

During the RAN2 #119-e meeting [1] , issues related to slice based cell reselection were discussed and partial agreements are as below.

Come back in next meeting once CT1/SA2 have finished their work. No reply to CT1 sent from this meeting.

If the sentence needs changes, those should be considered from the earliest possible release (e.g. Rel-15 if there are problems with the sentence already there, or Rel-17 specific issues with allowed/configured slices).

11: The NSAG information provided from NAS to AS (i.e.1) in R2-2207934 and 1) in R2-2207953 and proposal in R2-2208517) is postponed to be considered with further CT1/SA2 progress.

13: whether to improve the wording of all instances of ‘NSAG’s received from NAS’ and similar wording (i.e P3 in R2-2208143) is postponed to be considered with CT1/SA2 progress.

19: Postpone the coexistence of HSDN and slice specific cell reselection (FFS how/if that works in this release).

This contribution focuses on some remaining issues of slice based cell reselection.

- Discussion

  - Interaction between NAS and AS

In the latest TS 23.501, the interaction between NAS and AS for slice based cell reselection are as follows.

The UE NAS provides to the UE AS the NSAG Information as received from the AMF and the S-NSSAIs in the Allowed NSSAI as input to cell reselection, except when the UE intends to register with a new set of S-NSSAIs with a Requested NSSAI different from the current Allowed NSSAI, in which case the UE NAS provides to the UE AS layer the NSAG Information as received from the AMF and the S-NSSAIs in the Requested NSSAI, and this may trigger a cell reselection, before sending the Registration Request including the new Requested NSSAI.

It is obvious that the UE NAS only needs to transparently provide the UE AS with NSAG information received from the CN and the Allowed NSSAI or Requested NSSAI. Then the UE AS is responsible for tailoring the appropriated NSAG information as input for slice based cell reselection.

Observation 1: The UE NAS only needs to transparently provide the UE AS with NSAG information received from the CN and the Allowed NSSAI or Requested NSSAI. Then the UE AS is responsible for tailoring the appropriated NSAG information as input for slice based cell reselection.

In [2] and [3], there remains some postponed issues relating to cross layer interaction between the UE NAS and the UE AS, like whether:

- it should be specified that the mapping relationship between slices and NSAGs is provided to UE via NAS signalling in TS 38.300

- the description of “NSAG and their priorities” is changed into “the NSAG information (as specified in TS 24.501 [14])” or explicitly indicates the “NSAG ID + NSAG priority + valid TAC” pair

Hence, it is proposed that,

Proposal 1: RAN2 should align with the progress of SA2/CT1 for the interaction between the UE NAS and the UE AS.

  - Cell reselection frequency prioritization

In the last RAN2 #119bis-e meeting, the details of cell reselection frequency priority handling was triggered in [4] and agreements are as below.

- P1, P3, P4 are agreed: P1: For the cell reselection frequency prioritization, the MBS frequency may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation). P3’: For the cell reselection frequency prioritization, the frequency providing V2X/NR sidelink may be considered as the highest priority even if the slice specific frequency priority is configured (i.e., it is up to UE implementation). P4: For the cell reselection frequency prioritization, the HSDN cell shall be always considered as the highest priority, i.e., higher than the slice specific cell reselection priority even if configured and higher than MBS/V2X sidelink/NR sidelink frequency priority even if considered as the highest priority. It’s only applicable when the HSDN-capable UE is in High-mobility state, as it is today. Chair: Postpone finalization (checking of TS impact) to next meeting. If issues are found with the agreements, we can come back.

According to P4, it is clarified that considering the coexistence of HSDN, MBS, V2X/NR sidelink and slice, if the HSDN-capable UE is in High-mobility state, the HSDN cell shall be always be the highest priority, which can be directly supported via the highlighted sentence in TS 38.304.

If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). …

Therefore, it is proposed that,

Proposal 2: If the HSDN-capable UE is in High-mobility state, the HSDN cell shall be always considered as the highest priority as specified in TS 38.304. No spec impacts are observed.

- Conclusion

In this contribution, we mainly discuss details of slice based cell reselection. We have the following observations and proposals:

Observation 1: The UE NAS only needs to transparently provide the UE AS with NSAG information received from the CN and the Allowed NSSAI or Requested NSSAI. Then the UE AS is responsible for tailoring the appropriated NSAG information as input for slice based cell reselection.

Proposal 1: RAN2 should align with the progress of SA2/CT1 for the interaction between the UE NAS and the UE AS.

Proposal 2: If the HSDN-capable UE is in High-mobility state, the HSDN cell shall be always considered as the highest priority as specified in TS 38.304. No spec impacts are observed.

- References

- R2-2208702, Report on LTE/NR17 legacy, XR, MUSIM, WUS, QoE and NW energy.

- R2-2208729, [AT119-e][240][Slicing] RRC, MAC and Stage-2 CRs to RAN slicing (Huawei).

- R2-2208733, [AT119-e][241][Slicing] Cell reselection corrections to RAN slicing (Qualcomm).

- R2-2211025, Report of [AT119bis-e][005][NR17] Cell Reselection Frequency Prioritization (Kyocera).