3GPP TSG-RAN WG2 Meeting #120          	          R2-2212216

Toulouse, France, 14th – 18th November, 2022

Agenda Item:	6.13.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on capturing L2M agreements in TS 38.314

Document for:	Discussion and decision

Introduction

At RAN2#119-e meeting, the following agreements were made for L2M:

Agreements:

1	RAN2 agree that for a consistent calculation of the total RAN delay per split DRB per UE, a set of formulas for total RAN delay calculation are needed. RAN2 has already agreed that the total RAN delay calculation can be used for QoS monitoring and MDT purposes.

2	Total RAN delay calculation formula for split DRB in RAN node is to be specified in Rel 17, for the following scenarios:

PDCP duplication is not enabled during the delay measurement period

PDCP duplication was enabled during the delay measurement period

3	Total RAN delay calculation formula for split DRB is to be discussed in Rel-18, for the following scenario:

PDCP duplication was enabled per packet basis i.e., some packets transmitted with duplication and some packets without dupplication in a delay measurement period.

=>	RAN2 discuss whether to specify the total RAN delay formula in TS 38.314 or liaise SA5 to consider the formulas in their specification.

In this paper, we discuss how to progress on the above topic.

Discussion

For L2 measurements, TS 38.314 has defined some measurements. For example, for the measurement Average over-the-air interface packet delay in the UL per DRB per UE, it has been captured in section 4.2.1.2.2 in TS 38.314 and it is for QoS verification of MDT or for the QoS monitoring. In addition, SA5 has also defined some measurements, e.g. TS 28.552.

##### 4.2.1.2.2 Average over-the-air interface packet delay in the UL per DRB per UE The objective of this measurement is to measure air interface UL packet delay for OAM performance observability or for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4]. Protocol Layer: MAC

For total RAN delay formula, we think either RAN2 or SA5 can capture the definitions. We slightly prefer RAN2 to define them, and then we can send a LS to SA5 for information.

Proposal 1: It is proposed RAN2 to capture total RAN delay formula in TS 38.314.

At RAN2#119-e meeting, some papers [2][3] provided draft changes for total RAN delay formula. On top of these inputs, we provide more analysis and the draft changes can be found in section 5.

Proposal 2: It is proposed RAN2 to discuss and agree on draft changes as listed in section 5.

Conclusion

In this paper, we discuss how to progress on L2M discussions, and it is proposed:

Proposal 1: It is proposed RAN2 to capture total RAN delay formula in TS 38.314.

Proposal 2: It is proposed RAN2 to discuss and agree on draft changes as listed in section 5.

References

- R2-2209301	RAN2#119-e meeting minutes

- R2-2207948	Discussion on capturing L2M agreements in TS 38.314	Huawei, HiSilicon

- R2-2208206	Total RAN Delay calculation	Ericsson

Draft changes

4.2.1.x Total RAN Delay in Split-DRB Scenario

4.2.1.x.1 DL Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period

The objective of this measurement is to calculate total DL RAN delay per split-DRB per UE when PDCP duplication is enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.1-1: Definition for total DL delay for DL PDCP duplicated packets per Split-DRB per UE

| Definition | Total DL delay for DL PDCP duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total DL delay for a Split-DRB when PDCP duplication is enabled for the entire measurement period. Detailed Definition: M_DL(T,drbid)=min(M_DLMCG(T,drbid) , M_DLSCG(T,drbid)) , where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for total DL delay for DL PDCP duplicated packets per Split-DRB per UE

| M_DL(T,drbid) | Total DL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_DLMCG(T,drbid) | Total DL Delay on the MCG per split DRB per UE during the time period T. |
| M_DLSCG(T,drbid) | Total DL Delay on the SCG per split DRB per UE during the time period T. |
| min(M_DLMCG(T,drbid) , M_DLSCG(T,drbid)) | Minimum of the total DL delays between MCG and SCG legs per Split-DRB per UE during the time period T. |

4.2.1.x.2 UL Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period

The objective of this measurement is to calculate total UL RAN delay per split-DRB per UE when PDCP duplication is enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.1-1: Definition for total UL delay for UL PDCP duplicated packets per Split-DRB per UE

| Definition | Total UL delay for UL PDCP duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total UL delay for a Split-DRB when PDCP duplication is enabled for the entire measurement period. Detailed Definition: M_UL(T,drbid)=min(M_ULMCG(T,drbid) , M_ULSCG(T,drbid)) , where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for total UL delay for UL PDCP duplicated packets per Split-DRB per UE

| M_UL(T,drbid) | Total UL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_ULMCG(T,drbid) | Total UL Delay on the MCG per split DRB per UE during the time period T. |
| M_ULSCG(T,drbid) | Total UL Delay on the SCG per split DRB per UE during the time period T. |
| min(M_ULMCG(T,drbid) , M_ULSCG(T,drbid)) | Minimum of the total UL delays between MCG and SCG legs per Split-DRB per UE during the time period T. |

4.2.1.x.3 DL Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period

The objective of this measurement is to calculate total DL RAN delay per split-DRB per UE when PDCP duplication is not enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.2-1: Definition for total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE

| Definition | Total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total DL delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M_DL(T,drbid)=I_DLMCG(T,drbid)×M_DLMCG(T,drbid)+I_DLSCG(T,drbid)×M_DLSCG(T,drbid)I_DL(T,drbid) I_DLT,drbid=I_DLMCG(T,drbid)+I_DLSCG(T,drbid), where explanations can be found in the table 4.2.1.x.2-2 below. |
| --- | --- |

Table 4.2.1.x.2-2: Parameter description for total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE

| M_DL(T,drbid) | Total DL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_DLMCG(T,drbid) | Total DL Delay on the MCG per split DRB per UE during the time period T. |
| M_DLSCG(T,drbid) | Total DL Delay on the SCG per split DRB per UE during the time period T. |
| I_DLT,drbid | Total number of DL PDCP non-duplicated packets sent per Split-DRB per UE during the time period T. |
| I_DLMCG(T,drbid) | Total number of DL PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |
| I_DLSCG(T,drbid) | Total number of DL PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |

4.2.1.x.4 UL Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period

The objective of this measurement is to calculate total UL RAN delay per split-DRB per UE when PDCP duplication is not enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.2-1: Definition for total UL delay for UL PDCP non-duplicated packets per Split-DRB per UE

| Definition | Total UL delay for UL PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total UL delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M_UL(T,drbid)=I_ULMCG(T,drbid)×M_ULMCG(T,drbid)+I_ULSCG(T,drbid)×M_ULSCG(T,drbid)I_UL(T,drbid) I_ULT,drbid=I_ULMCG(T,drbid)+I_ULSCG(T,drbid), where explanations can be found in the table 4.2.1.x.2-2 below. |
| --- | --- |

Table 4.2.1.x.2-2: Parameter description for total UL delay for UL PDCP non-duplicated packets per Split-DRB per UE

| M_UL(T,drbid) | Total UL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_ULMCG(T,drbid) | Total UL Delay on the MCG per split DRB per UE during the time period T. |
| M_ULSCG(T,drbid) | Total UL Delay on the SCG per split DRB per UE during the time period T. |
| I_ULT,drbid | Total number of UL PDCP non-duplicated packets sent per Split-DRB per UE during the time period T. |
| I_ULMCG(T,drbid) | Total number of UL PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |
| I_ULSCG(T,drbid) | Total number of UL PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |