3GPP TSG RAN WG2 Meeting #120	R2-2212223

# Toulouse, France, 14th – 18th November, 2022

Agenda item:		8.13.7

Source:			Huawei, HiSilicon

Title:		Discussion on SON/MDT enhancements for NPN

Document for:	 	Discussion and decision

Introduction

At RAN2#119b-e meeting, the following agreements were made [1]:

Agreements:

1	SNPN ID (e.g.,NID ID) checking is needed before sending the availability indication for corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking.

2	Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID related info should be included can be discussed per use case.

3	RAN2 prioritizes the use cases of RLF report and logged MDT enhancement for NPN.

In the related email discussion for RA report, the following remaining issues still need further discussions:

- Detail discussion for PNI-NPN ID checking before sending the availability indication for corresponding SON and MDT report.

- Whether SNPN ID or PNI-NPN ID related info should be included into SON/MDT report

In this paper, we focus on RAN2 areas on the above issues.

Discussion

In R16 Non-Public Network (NPN) was introduced, and there are two types of NPN:

- SNPN: Standalone NPN, which does not rely on functions provided by a PLMN. An SNPN is identified by a PLMN ID and NID.

- PNI-NPN: Public Network Integrated NPN, which relies on the functions provided by a PLMN. In PNI-NPN, a Closed Access Groups (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. A CAG is identified by a CAG identifier.

For SNPN, the UE is set to operate in SNPN access mode and only moves within cells if the PLMN and NID broadcast by the cell matches the serving SNPN. For PNI-NPN, mobility is supported between PNI-NPN and PLMN/PNI-NPN based on the UE mobility restriction information.

Discussion on RLF report for NPN

In the last RAN2 119b-e meeting, it has been agreed that NID checking is needed before sending the availability indication for corresponding SON reports. It is reasonable to support NID checking for SNPN. In SNPN, the UE operating in SNPN access mode is only allowed to support mobility within the same SNPN, usually in a restricted area. By combining PLMN with NID, there is no equivalent SNPN. It is possible that the UE moves to a different SNPN network when operating in SNPN access mode. Besides, if the UE is not set to operate the specific SNPN access mode, the UE can perform normal PLMN selection procedures and finds the suitable cell belongs to other network based on the mobility restriction information. Due to the lack of SNPN information checking, e.g., NID checking, the UE may wrongly report the SNPN related SON reports to the PLMN or another supported SNPN network. Since one SNPN network is usually isolated with other networks, the wrongly reported SON reports will not be able to be re-transmitted to the correct SNPN.

However for PNI-NPN, the mobility of the UE is based on the mobility restrictions in the UE context as legacy. In the view of network, PNI-NPN is a slice of a PN network, while specified with CAG. Therefore, in contrast to SNPN, even UE records some SON reports in a PNI-NPN cell and reports them to a PN node, the latter can help to re-transmit these reports. Thus, we observe that no enhancements on SON reports are needed for PNI-NPN.

Proposal 1: PNI-NPN ID checking on SON reports are not needed.

Currently in TS 38.331, the UE only check RPLMN before reporting the SON reports. Here we use RLF as an example, in the clause 5.7.10.3 of 38.331, the checking procedure has been specified as follows.

#### 5.7.10.3 Reception of the UEInformationRequest message Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation: 1> if rlf-ReportReq is set to true: 2> if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report: 3> set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR; 3> set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report; 3> discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

Therefore, to support NID checking, the NID-list should also be included in the stored VarRLF-Report in UE. As for PNI-NPN, though CAG checking is not needed, the NW side needs to know the PNI-NPN network corresponding to the RLF report, for more specific optimizations. Thus, both NID and CAG should be included in the RLF report.

Proposal 2: Include both NID and CAG in the RLF report.

Discussion on logged MDT for NPN

For immediate MDT, since the UE is transparent to the immediate configuration while the measurements are performed and reported immediately, we think this kind of MDT is applicable for both SNPN and PNI-NPN without any updates.

Observation 1: No enhancements on immediate MDT are needed for SNPN and PNI-NPN.

For logged MDT in NPN, RAN3 in its 117b-e meeting has achieved the following processes:

Introduce a CAG list for MDT area scope. In Rel-18, UE performs MDT only in registered SNPN. Open issues: 1:How to introduce CAG list in existing MDT Configuration-NR IE,e.g. new separate Area Scope IE, or to extend existing area scope choice structure. 2:Whether area scope enhancement for SNPN is needed? 3:Whether PN UEs are allowed to be configured with NPN frequencies for MDT data collection of Neighbour Cells.

Therefore, a CAG list for MDT area scope should be included in MDT area configuration.

AreaConfiguration-v1700 ::=      SEQUENCE {

areaConfig-r17                   AreaConfig-r16                                                      OPTIONAL, -- Need R

interFreqTargetList-r17          SEQUENCE(SIZE (1..maxFreq)) OF InterFreqTargetInfo-r16              OPTIONAL  -- Need R

cag-IdentityList-r16             SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16              OPTIONAL  -- Need R

}

For SNPN, we think RAN2 should wait for RAN3 agreements about whether area scope enhancement for SNPN is needed.

Proposal 3: A CAG list for MDT area scope should be included in MDT area configuration.

Observation 2: RAN2 can wait for RAN3 progress about whether area scope enhancement for SNPN is needed.

For logged MDT results reporting, the analysis in section 2.1 is also applicable. For SNPN case, NID checking is necessary and legacy RPLMN checking is enough for PNI-NPN case, since the nodes sharing the same PLMN with the original PNI-NPN cell can help to transfer the measurement results to the TCE. Besides, both NID and CAG should be included in the logged MDT reports, for the operator to form a more specific realization of NPN network coverage.

Proposal 4: PNI-NPN ID checking on logged MDT reports are not needed.

Proposal 5: Include both NID and CAG in logged MDT reports.

Discussion on other aspects

One more potential issue is about the UE measurement resource wastes in SNPN. Assuming that a UE has some MDT measurement results or SON reports for this SNPN network, once the UE camps on or is connected to other networks before sending these reports to the specific NPN network, the stored results anyhow cannot be transferred to the original SNPN network. One reason is that the later connected networks may not belong to the registered PLMN and NID lists. Besides, usually a SNPN network is totally isolated, which means there is no any interfaces between one SNPN network and other networks. Moreover, if the UE is not connected to the specific SNPN networks longer than 48 hours, the stored results will be automatically deleted. This mentioned issue will bring wastes of UE measurement resource.

In order to avoid the wastes of UE measurement resource, one solution can be considered and here are some details:

- The UE knows the accurate rest time to store MDT reports

- If the UE can just store related reports for less than a specific time, and if it camps on the SNPN network, it will enter RRC_CONNETED status and report results the SNPN network

This method may bring extra power consumption for the UE, but can somehow promote the reported ratio of the measurement results.

Proposal 6: RAN2 to discuss the potential issue about UE measurement resource wastes in SNPN.

Conclusion

In this contribution, we give our opinions for SON/MDT enhancements in NPN, based on RAN 2 aspects.

Observation 1: No enhancements on immediate MDT are needed for SNPN and PNI-NPN.

Observation 2: RAN2 can wait for RAN3 progress about whether area scope enhancement for SNPN is needed.

Proposal 1: PNI-NPN ID checking on SON reports are not needed.

Proposal 2: Include both NID and CAG in the RLF report.

Proposal 3: A CAG list for MDT area scope should be included in MDT area configuration.

Proposal 4: PNI-NPN ID checking on logged MDT reports are not needed.

Proposal 5: Include both NID and CAG in logged reports.

Proposal 6: RAN2 to discuss the potential issue about UE measurement resource wastes in SNPN.

References

[1] RAN2#119b-e minutes