3GPP TSG-RAN WG2 #120 	R2-2212235

Toulouse France, 14th –18th November 2022

Agenda Item:	8.5.4.1 Feedback enhancements

Source:	NEC

Title:	BSR Enhancements for XR

Document for:	Discussion, Decision

# Introduction

Regarding BSR enhancement, last meeting we made two main agreements:

- Agree to introduce new BS table(s) to reduce the quantisation errors.

- Consider a remaining delay information is useful for XR, but it is FFS whether dynamic report is needed or PSDB is sufficient.

This contribution we would like will share our opinion on following aspects:

- Whether it is needed to have dynamic report of remaining delay information

- Additional enhancements to BSR

# Discussion

## New BSR triggers

BSR is a very important input for gNB scheduler to make dynamic schedule decision. In one direction, as we agree we can make a BS report more precise by introducing new BS table(s), in another direction, we can introduce new BSR report trigger(s), then gNB would have more up to date information of the BS after more data arrival in the buffer and or more data transmitted from the buffer.

Current BSR report framework triggers were designed as a compromise between overhead and efficiency, it may be too conservative for XR type of traffic:

- Regular BSR will not be triggered when more data arrive in buffer for the same LCH/LCG. If the CG match the start of data burst well, when BSR MAC CE is transmitted, not all PDUs of the data burst has been in buffer yet. PDU(s) arriving later and mapping to the same LCG will not be reported in time to gNB.

- Padding BSR may be transmitted only if there are enough padding bits available, because padding BSR has lower priority then data. However very likely there will not be padding bit if not all data has been transmitted

- Periodical BSR may not be applicable for burst data arrival unless very short periodicity is configured which does not sound reasonable

- Even If we introduce new BS tables, it would be feasible to introduce granularity as fine as the lower range of the legacy BSR considering more overhead with more buffer size bits and bigger BS table.

To improve capacity, it is very important to make gNB scheduler have a precise BSR estimation especially when it is about to finish the scheduling of all data. From this sense, introducing new BSR report triggers would be most efficiency and easy direction. as we mentioned at beginning, since BSR will not always triggered and update when the buffer size changes e.g., due to new fresh data arrival or due to part of the data has been transmitted, the estimation of the buffer size at gNB become less precise especially if the previous BSR indicates a big BS value range.

Moreover, consider the big packet/data burst size, extra overhead due to more frequent BSR report will not be an issue especially when the grant size is very big.

Many companies proposed to introduce new BSR triggers:

- BSR is triggered if there is remaining data for the current PDU set being transmitted[1]

- BSR is triggered when all data of a whole burst or PDU set arrives [4]

- BSR is triggered when a new data burst starts arriving[2]

- New trigger based on the reception of first PDU of PDU set [8][9]

- BSR is triggered when subsequent data arrival[4]

- Starting on-duration should trigger a periodic BSR[5]

- PDU discard should trigger a regular BSR[5][6]

- BSR is triggered when packet approaching the PDB[7] [9][10]

- BSR MAC CE is always included for each TB transmitted with XR packet[3]

- BSR is triggered when the grant is big

Details of new BSR trigger(s) could be discussed in WI phase. But in general, adding new BSR trigger(s) would be useful for gNB to have up to date information of the buffer size when there is change

- Introduce new BSR trigger(s) would be useful for gNB to have up to date information of the buffer size when there is change

Consider to introduce new BSR trigger(s)

## Delta BSR

Last meeting, we agreed to introduce new BS table(s) to reduce the quantization errors. however there are more questions coming up:

- Will the new BSR table cover buffer size from 0byte to infinite? or will it only cover limited buffer size range from Bmin to Bmax?

- If the new table only cover limited buffer size range, how do we determine the range? Will the range be configured dynamically, or will the range be hard coded in specification? How will this work for diverse XR service with different packet sizes

- How will the new BSR table be used together with legacy BSR table if they cover the same buffer size range?

- how fine should the new BS table be?

On the other hand, current buffer table is well designed to provide very precise size when the buffer size is smaller. Hence anther solution is to report a delta BSR MAC CE especially when the buffer size is big.

Detailed explanation is as following, assume:

- UE report buffer size index M, which indicate X< buffer size ≤ Y

- B be the amount of data actually in the UE buffer X< B ≤ Y

- Define Delta buffered data volume as B-X.

- UE looks up the legacy BSR table for delta buffer data volume B-X, result in buffer size index N

- UE reports legacy BSR(M) and Delta BSR (N) to gNB

Because the delta buffered data volume will be likely small, so it will give gNB a much more precise information. This can reduce the quantization errors largely

Benefit of this solution is no need of new BS table(s), it can work for any XR service with different typical/average data packet size, and there is no need to sort out how to work among different BS tables.

Consider delta BSR as an alternative solution to reduce the quantisation errors

## Dynamic report of remaining delay information

It is confirmed that delay information would be useful for XR. Considering PSDB is already available, question is whether dynamic report of remaining delay information would be useful or not. In uplink, although gNB may be able to roughly estimate the remaining delay based on the BSR report time and the PSDB information, the precision of the estimation cannot be guaranteed because it is not efficient to configure a very short CG periodicity if CG is used for BSR. If CG is not used for BSR, BSR transmission will be delayed more due to SR/BSR procedure, beside the delay is unpredictable. Therefore, considering the very stringent requirement of PSDB, it is beneficial to allow UE also reports the remaining delay information.

- The remaining delay information is useful for capacity improvement and is not known by gNB accurately

Because remaining delay information might be per PDU/PDU set, and it is tightly linked to buffered data and scheduler, we think we should extend BSR to report this dynamic information rather than designing a new framework.

Consider UE reporting dynamic remaining delay information along with the BSR

# Summary

This contribution provided our initial proposals on BSR enhancements as follows:

- Observation 1 Introduce new BSR trigger(s) would be useful for gNB to have up to date   information of the buffer size when there is change

- Consider to introduce new BSR trigger(s)

- RAN2 kindly consider delta BSR as an alternative solution to reduce the quantisation errors

- Delta BSR is explained here after:

- UE reports buffer size index M via legacy BSR MAC CE, which indicate X< buffer size ≤ Y

- B be the amount of data actually in the UE buffer X< B ≤ Y

- Define Delta buffered data volume as B-X.

- UE looks up the legacy BSR table for B-X, result in buffer size index N

- UE reports legacy BSR(M) and Delta BSR (N) to gNB

- Observation 2	The remaining delay information is useful for capacity improvement and is not known by gNB accurately

- Consider UE reporting dynamic remaining delay information along with the BSR

# References

- R2-2209472	BSR enhancement for XR capacity	CATT	discussion	Rel-18	FS_NR_XR_enh

- R2-2210537	Discussion on BSR enhancement for XR-specific capacity improvement	Huawei, HiSilicon	discussion	Rel-18	FS_NR_XR_enh

- R2-2209591	BSR enhancement for XR capacity	MediaTek Inc.	discussion	Rel-18

- R2-2209490	Discussion on feedback enhancements for XR-specific capacity improvements	vivo	discussion	Rel-18	FS_NR_XR_enh

- R2-2209558	BSR for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_NR_XR_enh

- R2-2209636	Enhancements to Buffer Status Reporting for XR Traffic	Intel Corporation	discussion	Rel-18	FS_NR_XR_enh

- R2-2209672	Discussing on UE feedback enhancements for XR capacity	Xiaomi Communications	discussion

- R2-2209691	Discussion on XR-specific feedback enhancements 	InterDigital, Inc.	discussion	FS_NR_XR_enh

- R2-2210024	Discussion on feedback enhancement	OPPO	discussion	Rel-18	FS_NR_XR_enh

- R2-2210599	Discussion on BSR enahancement for timing information in XR	LG Electronics Inc.	discussion	Rel-18	FS_NR_XR_enh