3GPP TSG-RAN WG2 #120 	R2-2212236

Toulouse, France 14th –18th November 2022

Agenda Item:	8.5.4.2 scheduling enhancements

Source:	NEC

Title:	UE assistance information for CG configuration at gNB

Document for:	Discussion, Decision

# Introduction

At RAN2# 119bits-e meeting, following agreements were made regarding CG for XR:

1: Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting. 2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

This contribution provides our initial consideration on use cases of CG for XR and whether UE assistance information is needed for CG configuration.

# Discussion

## Configure CG for XR

In most cases, DG might be more efficient for XR traffic especially with varied and big packet size. However there are use cases of CG:

Use case 1: For traffic with fixed periodicity, fixed small packet size, and without jitter, e.g., uplink traffic for Cloud gaming, pose/control traffic.

Use case 2: For traffic with fixed periodicity, varied and big packet size and without much jitter, e.g., Uplink traffic for VR.  Even though CG configuration may not be so efficient for data transmission directly. However, since XR traffic is request low latency, and have certain periodicity. If the CG configuration match the data periodicity, it could be used to send BSR at least, this would shorten the time to request the very first dynamic grant upon new data burst arriving in UE buffer. otherwise, one option is that gNB must blindly schedule the data, this will waste the radio resource if the data does not arrival at the expected time, and effectively same as CG. Another option is to go through SR/BSR procedure which leads to longer latency.

Observation 1: there are still use cases to configure CG either for transmitting periodical and fixed packet size traffic or transmit BSR for periodical but varied packet size traffic.

Currently, NW already can configure up to 32 CG, and support to configure periodicity in slot granularity. in our understanding, it is sufficient to support XR traffic.

Observation 2: currently multiple CG configuration with periodicity of slot granularity can be reused for XR use cases mentioned in observation 1

Considering RAN1 is also looking to this area, we can wait for RAN1

Wait for RAN1 to discuss further enhancement on CG configuration

## Assistant information on UL XR traffic

However, to enable gNB to configure type 1 or type 2 Configured Grant in UL, Input of the XR service characteristics would be useful for gNB e.g., periodicity and start of periodicity, potential packet size, potential jitter range and so on. UE is aware of this information better than network side entities and can update gNB quickly if there is any change. we can consider that UE report the XR traffic characteristic relevant information to gNB for CG configuration

RAN2 consider to introduce UL XR traffic pattern relevant assistance information to gNB for CG configuration.

Two options to report UL XR traffic characteristics relevant information:

- explicit way: report XR traffic characteristics directly, may including periodicity, start offset, packet size and so on. This information can be sent as UL Assistance Information, gNB may use this to configure proper CG. But it is eventually up to gNB implementation how to use this information.

- Implicit way: UE could ask for CG based on the ongoing XR traffic pattern, it can clearly request a CG with certain periodicity, offset and TB size. this could be sent with a MAC CE. Again it is eventually up to gNB whether to configure CG and how.

Both options may have pros and cons, and it may also depends on how dynamic the assistance information will be sent to gNB, RAN2 can further discuss which direction to go:

RAN2 is kindly asked to consider one or both of following ways to report XR traffic relevant assistance information:

- a) explicitly report XR traffic characteristics/pattern

- b) implicitly report by asking for desired CG for UL XR transmission

# Summary

This contribution provided our initial proposals on CG for XR:

Observation 1: there are still use cases to configure CG either for transmitting periodical and fixed packet size traffic or transmit BSR for periodical but varied packet size traffic.

Observation 2: currently multiple CG configuration with periodicity of slot granularity can be reused for XR use cases mentioned in observation 1

- Wait for RAN to discuss further enhancement on CG configuration

- RAN2 consider to introduce UL XR traffic pattern relevant assistance information to gNB for CG configuration.

- RAN2 is kindly asked to consider one of following ways to report XR traffic relevant assistance information:

- a) explicitly report XR traffic characteristics/pattern

- b) implicitly report by asking for desired CG for UL XR transmission