3GPP TSG-RAN WG2 #120 	R2-2212240

Toulouse, France 14th –18th November 2022

Agenda Item:	8.7.2 Coverage Enhancement

Source:	NEC

Title:	Coverage Enhancements

Document for:	Discussion, Decision

# Introduction

This contribution provides our initial consideration on following enhancements relevant to NR NTN coverage enhancement:

- Initial Msg3 blind retransmission

- Msg3 repetition

- Codec bit rate adaptation

# Discussion

## Initial Msg3 blind retransmission

Since Msg3 coverage is the bottleneck, one solution is to support Initial Msg3 blind scheduling.

Initial Msg3 blink retransmission scheduling is not possible since the RAR reception window will be started after gNB-UE RTT. Different solutions have been proposed to support initial Msg3 blind retransmission, all solutions require UE to wake up extra time and hence gNB could send blind retransmission grants to UE.

On the other hand, the Msg3 repetition will achieve the same target but without extra wake up time or extra PDCCH(s). we think between Msg3 blind retransmission and Msg3 repetitions, there is clear advantage of Msg3 repetition then Msg3 blind retransmission.

- Initial Msg3 blind retransmission requires extra DCI overhead, and extra waking up time but achieve the same target as of Msg3 repetition feature

Msg3 repetitions is an additional capability of UE, UE indicates implicitly by choosing right random access resource set. but blind retransmission can be applied to all UEs without special UE capability indication currently, where gNB just use the current active time to do blind retransmission scheduling. This may motivate us to enhance initial Msg3 blind retransmission. However, if there is further enhancement e.g., introducing extra active time, a kind of capability indication likely through random access resource partitioning will be required.

On the other hand, since both Msg3 repetition and NTN are introduced in Rel17, it is likely most of NTN UE will support Msg3 repetition since it is very important to increase coverage.

No further enhancement to support initial Msg3 blind scheduling

## Msg3 repetition

From TS38.123 section 8.3

A UE can be provided in BWP-UplinkCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests repetitions for the PUSCH transmission [11, TS 38.321], the UE transmits the PUSCH over N_"PUSCH" ^repeat slots, where N_"PUSCH" ^repeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 from a set of four values provided by numberOfMsg3-RepetitionsList or from {1, 2, 3, 4} if numberOfMsg3-RepetitionsList is not provided. The UE determines an MCS for the PUSCH transmission by the 2 LSBs of the MCS field in the RAR UL grant or by the 3 LSBs of the MCS field in the DCI format 0_0, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the N_"PUSCH" ^repeat slots as the first N_"PUSCH" ^repeat slots starting from slot n+k_2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

From TS38.321

The MAC entity shall: 1> if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or 1> if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication: 2> assume MSG3 repetition is applicable for the current Random Access procedure. 1> else: 2> assume MSG3 repetition is not applicable for the current Random Access procedure.

- Msg3 repetition currently is not supported for contention free random-access procedure

As we can see from cited text above, for the rel-17 Msg3 repetition feature, network configure set(s) of Random-Access resources with MSG3 repetition indication and a RSRP threshold. UE can request Msg3 repetition if its RSRP lower than the configured threshold. Afterward, two bits in RAR grant is repurposed to indicate the repetition times, from UE point of view, it will read the two bits in RAR grant differently if it has requested for Msg3 repetition by chosen the corresponding random-access resource pool.

However there are potential issues for NTN:

- due to Smaller RSRP difference between cell edge and cell centre, it is difficult to configure the RSRP threshold as other RSRP based trigger e.g., for CHO

- due to satellite movement, RSRP of UEs will changes quickly, Amount of UE with RSRP<threshold will vary, and hence the preamble pool for requesting Msg3 repetition may became overloaded or underloaded along the time

- due to satellite movement and long gNB-UE RTT, for a given UE, its RSRP may change from below the threshold to above the threshold or vice versa after the time gap between Msg1 and Msg3 transmissions.

- RSRP-based threshold for Msg3 repetition random access resource may face issues in NTN scenarios.

RAN2 discuss necessary enhancement on Msg3 repetition feature for NTN scenario.

## Codec bit rate adaptation

RAN-assisted voice codec bitrate adaptation mechanism is supported in TN. The network may transmit the recommended bit rate MAC CE to the MAC entity to indicate the recommended bit rate for the UE for a specific logical channel and a specific direction (either uplink or downlink). Upon reception of a recommended bit rate MAC CE the MAC entity shall indicate the upper layers the recommended bit rate. Furthermore, upon receiving application layer message suggested higher downlink or uplink bit rate, the MAC entity may request the network to indicate the recommended bit rate. In response to the query, the network can send codec bitrate recommendation to UE via DL MAC CE. However, in NTN scenario, due to the much longer propagation distance, recommending codec bit rate based on MAC CE and query of recommended bit rate will take longer than in TN.

- The MAC CE based codec bit rate adaptation may take longer in NTN.

In TR 36.750, a broadcast based scheme has been proposed. Network can decide the mapping relationship of the bit rates/highest bit rate corresponding to each channel condition scope considering the load in the cell and broadcast the mapping relationship to UEs. According to the mapping relationship, the UE performs the measurements, and determines the recommended bit rates based on the measurements. Obviously, this may mitigate the RTT delay due to MAC CE exchange and save the L2 signaling.

RAN2 to study broadcast based codec bit rate adaptation mechanism for NTN.

# Summary

For NR NTN coverage enhancement, this contribution provided our initial proposals regarding initial Msg3 blind retransmission, Msg3 repetition and codec bit rate adaptation as follows:

Observation 1  Initial Msg3 blind retransmission requires extra DCI overhead, and extra waking up time but achieve the same target as of Msg3 repetition feature

Proposal 1   No further enhancement to support initial Msg3 blind scheduling

Observation 2   Msg3 repetition currently is not supported for contention free random-access procedure

Observation 3   RSRP-based threshold for Msg3 repetition random access resource may face issues in NTN scenarios.

Proposal 2    RAN2 discuss necessary enhancement on Msg3 repetition feature for NTN scenario.

Observation 4   The MAC CE based codec bit rate adaptation may take longer in NTN.

Proposal 3      RAN2 to study broadcast based codec bit rate adaptation mechanism for NTN.