3GPP TSG-RAN WG2 Meeting #120                                               	                R2-2212246

Toulouse, France,  November 14 – 18, 2022

Agenda item:	    8.4.2.2

Source: 		Qualcomm Incorporated

Title: 	RRC Aspects of LTM

Document for:	Discussion

# 1	Introduction

The following agreements were achieved in the last meeting [1]:

RAN2 to use “LTM” as term for the L1/L2-triggered mobility. Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between. No security update support in Rel-18 with L1/L2 based mobility. FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified. For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: - MAC/RLC reset (when configured) - RF retuning (e.g. needed for inter-frequency), baseband retuning R2 assumes that the following items may be discussed by RAN1 and RAN4 (and may be scenario specific): - Whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. R2 assumes this is feasible at least for the case that the target cell is already an active serving cell. - Whether to support of performing TRS tracking and CSI measurement of candidate/target cell before/by cell switch command L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN. A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. - Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration - Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release). For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration. For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates. FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed. RAN2 assumes that RAN1 will drive discussions on L1 measurement enhancements, if any. If RAN1 identifies the need for e.g. event reporting, filtering etc, RAN2 can then be involved if needed. Inter-freq L1L2 mobility: R2 Confirms that For L1L2 mobility inter-freq scenarios in general should be supported (including mobility to inter-frequency cell that is not a current serving cell), including the support of inter-frequency L1 measurements, if feasible by R4 and R1. RAN2 assumes that whether to use the unified TCI framework as the baseline for beam indication for L1L2 mobility is up to RAN1 (RAN2 observes that L1/L2 mobility need to support inter-freq cases).

This contribution discusses RRC modelling for LTM, delta configuration, and L2 reset during LTM.

# 2	Discussion

## 2.1	RRC Modelling

RAN2-119bis-e agreed to continue discussing two RRC models for configuring the candidate cells for LTM:

- Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration

- Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

Model 1 provides more flexibility compared to Model 2 and is also future proof. Given that most features of an RRC Reconfiguration message are optional, the overhead of Models 1 and 2 is similar. Model 1 should therefore be supported.

Proposal 1: Support RRC Model 1 – one Reconfiguration message per candidate target configuration.

## 2.2	Delta Configuration

RAN2-119bis-e agreed that delta configuration will be supported for the candidate configurations on top of a reference configuration. There are two issues if the reference configuration is any of the current configuration or the candidate configurations:

- Issue 1: If this configuration is updated, all other candidate configurations defined as delta on top of the modified reference will also have to be updated.

- Issue 2: If this configuration is released, all other candidate configurations will have to be redefined with respect to a new reference configuration.

On the other hand, if the reference configuration is a separate configuration, it does not refer to actual configuration of candidate cells that may have to be occasionally updated or released. Therefore, a separate reference configuration should be supported.

Proposal 2: The delta configuration of a candidate configuration is defined with respect to a separate reference configuration.

For simplicity, a reference configuration shall not use delta signalling.

Proposal 3: A reference configuration does not use delta signalling on top of another reference configuration.

Whether RRC Model 1 or 2 is used for the candidate configurations, the reference configuration should have similar structure to enable proper delta signalling.

Proposal 4: The reference configuration uses the same format as the candidate RRC configuration.

## 2.3	Determination of L2 Reset

A UE may perform L2 reset at cell switch including MAC reset, RLC reestablishment and PDCP recovery if explicitly instructed by the network. RAN2-119bis-e assumed this is configured via RRC.

RAN2-119bis-e further agreed that subsequent LTM between candidates without RRC reconfiguration shall be supported.

| Figure 1a - Example 1 of subsequent LTM | Figure 1b - Example 2 of subsequent LTM |
| --- | --- |

Figure 1 shows an example of a subsequent LTM scenario where the UE is initially served on cell 1 and receives candidate RRC configurations for cells 2, 3 and 4. In this case, the RRC configurations corresponding to cells 3 and 4 indicate L2 reset to the UE since they are served by a different gNB-DU than cell 1, whereas the RRC configuration corresponding to cell 2 does not configure L2 reset since cells 1 and 2 are served by the same gNB-DU.

In Figure 1a, the UE switches cell 1  cell 3 and correctly performs L2 reset. The UE performs a subsequent switch cell 3  cell 4. Although the second switch is under the same gNB-DU, the RRC configuration of cell 4 unnecessarily indicates L2 reset.

In Figure 1b, the UE switches cell 1  cell 3 and correctly performs L2 reset. The UE performs a subsequent switch cell 3  cell 2. Although the second switch is under different gNB-DUs, the RRC configuration of cell 2 incorrectly does not indicate L2 reset.

In both examples, the information carried in the candidate RRC configurations on whether to perform L2 reset is outdated.

Observation 1: Information on whether to perform L2 reset as carried in the candidate RRC configurations may be outdated in the subsequent LTM scenario.

To avoid this issue, two options are possible:

- Option 1: The cell switch MAC CE indicates whether the UE should perform L2 reset

- Option 2: The UE receives in RRC a grouping of the candidate RRC configurations. The UE performs L2 reset when it performs inter-group switch of candidate configurations and does not perform L2 reset otherwise.

Option 1 implies that MAC CE will have to carry configuration that is normally provided in RRC. This may have security issues. Since the association of cells and gNB-DUs is stationary, providing this configuration in MAC CE is not necessary. Option 2 should therefore be supported.

Observation 2: Indicating whether to perform L2 reset in MAC CE is unnecessary and may be insecure.

Proposal 5: RRC configures a grouping of candidate LTM configurations, where UE performs L2 reset during inter-group cell/CG mobility.

# Conclusion

This contribution discussed RRC modelling for LTM, delta configuration, and L2 reset during LTM. The following observations and proposals have been made:

Observation 1: Information on whether to perform L2 reset as carried in the candidate RRC configurations may be outdated in the subsequent LTM scenario.

Observation 2: Indicating whether to perform L2 reset in MAC CE is unnecessary and may be insecure.

Proposal 1: Support RRC Model 1 – one Reconfiguration message per candidate target configuration.

Proposal 2: The delta configuration of a candidate configuration is defined with respect to a separate reference configuration.

Proposal 3: A reference configuration does not use delta signalling on top of another reference configuration.

Proposal 4: The reference configuration uses the same format as the candidate RRC configuration.

Proposal 5: RRC configures a grouping of candidate LTM configurations, where UE performs L2 reset during inter-group cell/CG mobility.

# References

- RAN2-119bis-e – Chairman Notes