3GPP TSG-RAN WG2 Meeting #120	R2-2212249

Toulouse, France, November 14th – 18th, 2022

Agenda item:		8.5.3.1

Source:		Futurewei

Title:	On DRX enhancements for handling non-integer traffic periodicity

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss the issue of handling non-integer XR traffic periodicity for the DRX operation. More specifically, we describe and compare two solutions to enhance DRX for handling non-integer XR traffic periodicity.

# Discussions

## Issue of handling non-integer XR traffic periodicity in DRX operations

A typical video CODEC produces video frames at a rate of 24, 30, 50, 60, 90, or 120 frames per second, corresponding to a frame periodicity of 41.667, 33.333, 20, 16.667, 11.111, or 8.333 msec, respectively. Except 20 msec, the other frame periodicities are not an integer multiple of 1 msec or 1 microsecond.

The existing standardized DRX cycle values don’t match very well with these video frame periodicities, except 20 msec [1]. When the configured DRX cycle value doesn’t perfectly match with the traffic periodicity, the transmission queuing delay for the video data keeps increasing for each subsequent frame until it approaches a limit capped by the configured DRX cycle value and snaps back to a small value. After that, the transmission queuing delay keeps increasing for each subsequent frame again until it approaches the limit and snaps back again, and the same pattern goes on and on. The transmission queuing delay caused by the mismatch takes valuable time away from the delay budget available for the scheduling and the actual transmission over the Uu link, and therefore reducing scheduling flexibility and spectral efficiency. Although one can shorten the DRX cycle to cap the transmission queuing delay to a smaller value, the UE power saving gain will be reduced. Using RRC or Layer 1/2 signaling (such as DCI or MAC CE) to dynamically adjust the starting time of the next DRX cycle is not desirable either because of the frequent signaling overhead.

Two solutions, referred to as the rounding based solution and the leap-cycle based solution, are more practical for handling non-integer periodicity in DRX operations. We describe them and compare them in the subsequent sections.

## Rounding based solution

In the rounding based solution, the gNB configures a nominal DRX cycle duration to be precisely equal to the traffic periodicity, no matter the traffic periodicity is an integer periodicity or a non-integer periodicity. Then, for each DRX cycle, the gNB and the UE respectively performs the following:

Step 1. Computes the sum of the nominal DRX cycle duration and the stored Nominal Starting Time of Next DRX cycle and then updates the stored Nominal Starting Time of Next DRX cycle to be equal to the computed sum, which is to be used for computing the nominal starting time of the next DRX cycle.

Step 2. Rounds the computed Nominal Starting Time of Next DRX cycle to the closest time unit with a rounding function, a ceiling function, or a floor function that is specified. The time unit can be specified or pre-configured as, e.g., one frame, one slot, or one OFDM symbol, etc.

Step 3. Uses the rounded starting time as the actual starting time of the next DRX cycle and acts accordingly.

## Leap-cycle based solution

In the leap-cycle based solution, the gNB configures the UE, using RRC signaling, with the following:

- A regular DRX cycle duration (T_regular) to be equal to a standardized DRX cycle value that closely matches with but not necessarily equal to the traffic periodicity (T_traffic),

- A number N, where every time when N number of regular DRX cycles are complete, a leap DRX cycle is to be inserted next, and

- A leap DRX cycle duration (T_leap) to be equal to another standardized DRX cycle value,

where (N x T_regular + T_leap) = (N+1) x T_traffic.

Therefore, T_regular is shorter than T_traffic when T_leap is longer than T_traffic and T_regular is longer than T_traffic when T_leap is shorter than T_traffic.

The gNB and the UE each maintains a counter for tracking the number of consecutive regular DRX cycles completed, and when reaching the number N, resets the counter and inserts a leap DRX cycle as the next DRX cycle. After the inserted leap DRX cycle is completed, the next N DRX cycles are regular DRX cycles again, then followed by another leap DRX cycle, and so on and so forth.

Therefore, the RRC signaling used for configuring the UE for the above operations can be a one-time signaling. No dynamic signaling is required.

## Comparison

Both the rounding based solution and the leap-cycle based solution can be used for handling non-integer traffic periodicity for DRX enhancements. Furthermore, both solutions can also be used for handling non-integer traffic periodicity for SPS or CG enhancements, as the issue of periodicity mismatch also exists for SPS and CG.

Compared to the leap-cycle based solution, the rounding based solution produces more uniform durations among all DRX (or SPS/CG) cycles. In a situation where multiple XR UEs are served by the same cell and inter-UE staggering is configured, if the leap-cycle based solution is used, once a while, the starting time of the next DRX cycle ( or the next SPS-O/CG-O) of a UE is determined based on its leap DRX (or SPS/CG) cycle duration while the starting time of the next DRX cycle (or the next SPS-O/CG-O) of the other UEs are determined based on their regular DRX (or SPS/CG) cycle durations, which could result in certain overlapping of the next DRX-On periods (or SPS-Os/CG-Os) between this UE and one or more of the other UEs. In the same situation, if the rounding based solution is used instead, the jitters introduced by the rounding to the actual starting time of the next DRX cycle are capped at plus and minus half of the rounding time unit (such as one slot), hence are small, comparing to the jitters in data arrival time.

When inter-UE staggering is configured, a small time gap between the staggered high-volume data bursts of the different UEs may have already been arranged to avoid inter-UE collision due to the jitters in data arrival time. Such time gaps can easily absorb the impact of the small jitters due to the rounding used in the rounding base solution. Therefore, the rounding based solution is more friendly to inter-UE staggering and hence has a slight advantage over the leap-cycle based solution.

Proposal 1: RAN2 study the rounding based solution and the leap-cycle based solution, as described, for handling non-integer XR traffic periodicity in DRX operations.

Proposal 2: RAN2 consider incorporating the text proposed in the Annex into draft TR 38.835.

# Conclusions

The followings are proposed:

Proposal 1: RAN2 study the rounding based solution and the leap-cycle based solution, as described, for handling non-integer XR traffic periodicity in DRX operations.

Proposal 2: RAN2 consider incorporating the text proposed in the Annex into draft TR 38.835.

# References

3GPP TS 38.331.

# Annex: Proposed Text for TR 38.835

Start of new text

### 5.2.2.x	DRX enhancements for handling non-integer XR traffic periodicity

#### 5.2.2.x.1	Issue of handling non-integer XR traffic periodicity in DRX operations

A typical video CODEC produces video frames at a rate of 24, 30, 50, 60, 90, or 120 frames per second, corresponding to a frame periodicity of 41.667, 33.333, 20, 16.667, 11.111, or 8.333 msec, respectively. Except 20 msec, the other frame periodicities are not an integer multiple of 1 msec or 1 microsecond.

The existing standardized DRX cycle values don’t match very well with these video frame periodicities, except 20 msec. When the configured DRX cycle value doesn’t perfectly match with the traffic periodicity, the transmission queuing delay for the video data keeps increasing for each subsequent frame until it approaches a limit capped by the configured DRX cycle value and snaps back to a small value. After that, the transmission queuing delay keeps increasing for each subsequent frame again until it approaches the limit and snaps back again, and the same pattern goes on and on. The transmission queuing delay caused by the mismatch takes valuable time away from the delay budget available for the scheduling and the actual transmission over the Uu link, and therefore reducing scheduling flexibility and spectral efficiency. Although one can shorten the DRX cycle to cap the transmission queuing delay to a smaller value, the UE power saving gain will be reduced. Using RRC or Layer 1/2 signaling (such as DCI or MAC CE) to dynamically adjust the starting time of the next DRX cycle is not desirable either because of the frequent signaling overhead.

Two solutions, referred to as the rounding based solution and the leap-cycle based solution, are more practical for handling non-integer periodicity in DRX operations. We describe them and compare them in the subsequent sections.

## 5.2.2.x.2	Rounding based solution

In the rounding based solution, the gNB configures a nominal DRX cycle duration to be precisely equal to the traffic periodicity, no matter the traffic periodicity is an integer periodicity or a non-integer periodicity. Then, for each DRX cycle, the gNB and the UE respectively performs the following:

Step 1. Computes the sum of the nominal DRX cycle duration and the stored Nominal Starting Time of Next DRX cycle and then updates the stored Nominal Starting Time of Next DRX cycle to be equal to the computed sum, which is to be used for computing the nominal starting time of the next DRX cycle.

Step 2. Rounds the computed Nominal Starting Time of Next DRX cycle to the closest time unit with a rounding function, a ceiling function, or a floor function that is specified. The time unit can be specified or pre-configured as, e.g., one frame, one slot, or one OFDM symbol, etc.

Step 3. Uses the rounded starting time as the actual starting time of the next DRX cycle and acts accordingly.

## 5.2.2.x.3	Leap-cycle based solution

In the leap-cycle based solution, the gNB configures the UE, using RRC signaling, with the following:

- A regular DRX cycle duration (T_regular) to be equal to a standardized DRX cycle value that closely matches with but not necessarily equal to the traffic periodicity (T_traffic),

- A number N, where every time when N number of regular DRX cycles are complete, a leap DRX cycle is to be inserted next, and

- A leap DRX cycle duration (T_leap) to be equal to another standardized DRX cycle value,

where (N x T_regular + T_leap) = (N+1) x T_traffic.

The gNB and the UE each maintains a counter for tracking the number of consecutive regular DRX cycles completed, and when reaching the number N, resets the counter and inserts a leap DRX cycle as the next DRX cycle. After the inserted leap DRX cycle is completed, the next N DRX cycles are regular DRX cycles again, then followed by another leap DRX cycle, and so on and so forth.

Therefore, the RRC signaling used for configuring the UE for the above operations can be a one-time signaling. No dynamic signaling is required.

## 5.2.2.x.4	Comparison

Both the rounding based solution and the leap-cycle based solution can be used for handling non-integer traffic periodicity for DRX enhancements.

Compared to the leap-cycle based solution, the rounding based solution produces more uniform durations among all DRX cycles. In a situation where multiple XR UEs are served by the same cell and inter-UE staggering is configured, if the leap-cycle based solution is used, once a while, the starting time of the next DRX cycle of a UE is determined based on its leap DRX cycle duration while the starting time of the next DRX cycle of the other UEs are determined based on their regular DRX cycle durations, which could result in certain overlapping of the next DRX-On periods between this UE and one or more of the other UEs. In the same situation, if the rounding based solution is used instead, the jitters introduced by the rounding to the actual starting time of the next DRX cycle are capped at plus and minus half of the rounding time unit (such as one slot), hence are small, comparing to the jitters in data arrival time.

When inter-UE staggering is configured, small time gaps between the staggered high-volume data bursts of the different UEs may have already been arranged to avoid inter-UE collision due to the jitters in data arrival time. Such time gaps can easily absorb the impact of the small jitters due to the rounding used in the rounding base solution. Therefore, the rounding based solution is more friendly to inter-UE staggering and hence has a slight advantage over the leap-cycle based solution.

End of new text