3GPP TSG-RAN WG2 Meeting #120	R2-2212260

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.7.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Cell Reselection Enhancements in Rel-18

WID/SID:	NR_NTN_enh – Rel-18

Document for:	Discussion and Decision

# 1	Introduction

The following WI objective has been agreed for Rel-18 NTN WI [1]:

4.1.4 NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

The following agreements were made at RAN#119bis [3]:

Agreements: 1. For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered) 2. To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available.

Agreements: 1. System information is the basic means for providing necessary parameters to assist UE to estimate when the serving cell stops providing coverage at the present UE location. 2. UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. 3. The method of detecting the transmission energy or SIB presence to determine the NTN coverage when a UE currently camps on a TN cell is not pursued. 4. In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

Agreements: 1. RAN2 confirms that at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge) thanks to the existence of predefined satellite orbits and negligible UE’s mobility in comparison to satellite’s motion (we can further discuss at the next meeting whether this applies to idle mode UEs as well) 2. New Proposal 2: RAN2 continues the discussion (e.g. at RAN2#120) on the solution with keeping the same PCI after switching of the satellites. Clarify at least the following: • RAN1 impact • The need to perform UL beam switching and/or RA • Applicability to hard or soft satellite switching

# 2	NTN-NTN cell reselection

In quasi-Earth-fixed cells in Rel-17 t-Service can be used by the UEs to perform cell reselection measurements. However, for Earth-moving cells, using a common cell-specific t-Service can be difficult because the cell stop time depends on the UE location.

As shown above, the following agreements were made in the last RAN2 meeting [3]

Agreements: - For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered)

The agreement addresses the evident issue of Earth-moving cells where the serving cell stop time is specific for each UE location. However, the agreement left FFS whether this feature should be UE optional.

Observation 1: RAN2 left FFS whether the serving cell signals cell-specific parameters for UE to estimate when the serving cell stops cellular coverage.

Proposal 1: For Earth-moving cells, RAN2 to discuss whether there exist alternatives to the UE estimating when the serving cell stops coverage.

In RAN2#119bis the following agreements were made:

Agreements: - System information is the basic means for providing necessary parameters to assist UE to estimate when the serving cell stops providing coverage at the present UE location. - In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE.

Considering that Earth-moving cells adjust continuously its reference location, each UE will require appropriate calculations to estimate when the serving cell coverage stops. The UE should perform these calculations, considering:

- Satellite’s ephemeris (to know cells direction and cells speed).

- Serving cell reference location.

- Serving cell distance threshold.

- UE’s location.

Proposal 2: UE performs individual estimation, considering satellite’s ephemeris, serving cell reference location, serving cell distance threshold and its own location to enable location-based reselections in Earth-moving scenario.

As agreed in the last meeting, for Earth-moving cells, the cell reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. However, it was left open, how the reference location and/or distance threshold are provided to the UE.

Such assistance information can be broadcast via system information or via dedicated signaling. Dedicated signaling is UE-specific and as we consider here the IDLE mode scenario, such information can be only provided in RRC Release message (when moving from CONNECTED to IDLE). For such cases system information is a better choice.

Proposal 3: Considering the agreements taken in the RAN2#119bis meeting, the reference location and distance threshold of serving cell shall be provided to the UE via system information.

With this information in place for the serving cell, it is only logical, to provide similar information for neighbor cells as well, to allow the UE to select the proper neighbor cell.

Proposal 4: The reference location and distance threshold can also be provided for neighbor cells.

# 3	TN-NTN cell-reselection

The following was agreed in the RAN2#119bis meeting [3]:

Agreement: - To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available. - UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. - The method of detecting the transmission energy or SIB presence to determine the NTN coverage when a UE - currently camps on a TN cell is not pursued.

Following previous RAN2 discussions, it has been recognised that in a scenario with UE which is under TN coverage, it would be beneficial to start searching for NTN cells only when needed. Vice versa, a UE under NTN coverage could benefit from re-selecting to a TN cell when the UE is located close enough to the TN coverage area. In both scenarios, the unnecessary UE cell search (for TN or NTN) should be avoided to minimize the UE power consumption.

Observation 2: It is beneficial for the power saving purposes if the UE can measure for NTN or TN coverage only when relevant.

As indicated above, it was also agreed, that the method of detecting the transmission energy or SIB presence to determine NTN coverage is not pursued. However, we believe that this approach has not been fully understood. Hence, we want to add more explanation in the following paragraph.

Let us consider the scenario, where there is an NTN user close to the TN coverage. The TN cell can detect this NTN user in its vicinity, e.g. based on transmission energy detection and/or reception/decoding of the NTN SIB. With this knowledge, the TN can notify the NTN that a NTN-connected UE is in TN coverage, thereby aiding the reselection/cell change. Hence, this method is not to determine the NTN coverage, but to find an NTN UE within/at the border of TN coverage.

Proposal 5: RAN2 is asked to re-consider the method of detecting the transmission energy or SIB presence to find NTN UEs at the border of TN coverage.

Alternatively, the assistance information from the NTN cell could be based on information about the location of the relevant TN cells. For example, the NTN cell can broadcast one or more distanceThresh values corresponding to relevant neighbouring TN cells, to indicate to the NTN UEs at which locations they should start searching for TN cells.

Proposal 6: location information can be included in the system information to indicate UEs when start searching for TN/NTN cells and the UE does not have to search for TN/NTN when not in the indicated area.

# 4	NTN-TN cell-reselection in RRC_INACTIVE state

A UE served by the NTN may perform RRC_INACTIVE state cell re-selection to a TN cell. As per the following SA2 agreements made in the context of NTN:

- Clause 5.4.11.2 [1]: In case of NR satellite access, the RAT Types values "NR(LEO)", "NR(MEO)", "NR(GEO)" and "NR(OTHERSAT)" are used in 5GC to distinguish the different NR satellite access.

- Clause 5.4.10 [1]: In order to enable efficient enforcement of Mobility Restrictions, cells of each NR satellite RAT Type (NR(LEO), NR(MEO), NR(GEO) or NR(OTHERSAT)) need to be deployed in TAs different from TAs for other NR satellite RAT Types as well as different from TAs supporting terrestrial access RAT Types.

The above agreements imply that, each NTN RAT Type e.g. NR(LEO), NR(GEO) etc. must be configured with non-overlapping Tracking Area (TA) or the CN Registration Area or simply Mobility Registration Area (MRA). Which means that for the NTN and TN overlapping scenarios, the TA or MRA of NTN and TN must be clearly defined without an overlap.

Observation 3: Each NTN RAT Type e.g. NR(LEO), NR(GEO) etc. must be configured with non-overlapping Tracking Area (TA) or the CN Registration Area or simply Mobility Registration Area (MRA).

Observation 4: For the NTN and TN overlapping scenarios, the TA or MRA of NTN and TN must be clearly defined without an overlap.

Following deployment scenarios may be possible for NTN:

- NTN deployed with PLMN different from TN

- NTN and TN deployed with Equivalent PLMN (EPLMN)

- NTN and TN deployed with common PLMN

If we consider NTN and TN overlapping scenarios for an RRC_INACTIVE state UE, the following scenarios are most likely:

- RRC_INACTIVE state UE served by NTN cell re-selects a TN cell,

- RRC_INACTIVE state UE served by TN cell re-selects an NTN cell

In both the above-mentioned likely scenarios, it may be desirable to allow a UE in RRC_INACTIVE state to re-select from NTN cell to TN cell or vice-versa without a RRC state transition i.e., either to RRC_CONNECTED or to RRC_IDLE while preserving the existing PDU sessions and configured DRBs.

Observation 5: It may be desirable to allow a UE in RRC_INACTIVE state to re-select from NTN cell to TN cell or vice-versa without a RRC state transition i.e., either to RRC_CONNECTED or to RRC_IDLE.

Due to SA2 agreements for NTN, as captured above, when an RRC_INACTIVE state UE re-selects from NTN cell to TN cell or vice-versa, UE must trigger Mobility Registration Area Update procedure to inform the AMF that it has entered a new Tracking Area (TA) or Mobility Registration Area (MRA).

Observation 6: When an RRC_INACTIVE state UE re-selects from NTN cell to TN cell or vice-versa, UE must trigger Mobility Registration Area Update procedure to inform the AMF that it has entered a new Tracking Area (TA) or Mobility Registration Area (MRA).

Proposal 3: RAN2 to discuss how to enable an RRC_INACTIVE state UE to re-select from NTN cell to TN cell or vice-versa without an RRC state transition to RRC_CONNECTED or to RRC_IDLE, while preserving the PDU sessions and configured DRBs.

# 5	Conclusion

This documents has made the following observations and proposals:

Observation 1: RAN2 left FFS whether the serving cell signals cell-specific parameters for UE to estimate when the serving cell stops cellular coverage.

Proposal 1: For Earth-moving cells, RAN2 to discuss whether there exist alternatives to the UE estimating when the serving cell stops coverage.

Proposal 2: UE performs individual estimation, considering satellite’s ephemeris, serving cell reference location, serving cell distance threshold and its own location to enable location-based reselections in Earth-moving scenario.

Proposal 3: Considering the agreements taken in the RAN2#119bis meeting, the reference location and distance threshold of serving cell shall be provided to the UE via system information.

Proposal 4: The reference location and distance threshold can also be provided for neighbour cells.

Observation 2: It is beneficial for the power saving purposes if the UE can measure for NTN or TN coverage only when relevant.

Proposal 5: RAN2 is asked to re-consider the method of detecting the transmission energy or SIB presence to find NTN UEs at the border of TN coverage.

Proposal 6: location information can be included in the system information to indicate UEs when start searching for TN/NTN cells and the UE does not have to search for TN/NTN when not in the indicated area.

Observation 3: Each NTN RAT Type e.g. NR(LEO), NR(GEO) etc. must be configured with non-overlapping Tracking Area (TA) or the CN Registration Area or simply Mobility Registration Area (MRA).

Observation 4: For the NTN and TN overlapping scenarios, the TA or MRA of NTN and TN must be clearly defined without an overlap.

Observation 5: It may be desirable to allow a UE in RRC_INACTIVE state to re-select from NTN cell to TN cell or vice-versa without a RRC state transition i.e., either to RRC_CONNECTED or to RRC_IDLE.

Observation 6: When an RRC_INACTIVE state UE re-selects from NTN cell to TN cell or vice-versa, UE must trigger Mobility Registration Area Update procedure to inform the AMF that it has entered a new Tracking Area (TA) or Mobility Registration Area (MRA).

Proposal 3: RAN2 to discuss how to enable an RRC_INACTIVE state UE to re-select from NTN cell to TN cell or vice-versa without an RRC state transition to RRC_CONNECTED or to RRC_IDLE, while preserving the PDU sessions and configured DRBs.

# References

- TS 23.501 stage 2 TS System Architecture for 5G system

- RP-221819 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements 3GPP TSG RAN Meeting #96 Budapest, Hungary, June 6-9, 2022

- R2-2210801 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #119bis electronic