3GPP TSG-RAN2 Meeting #120-e	R2-2212271

Toulouse, France, 14-18 November 2022

Agenda Item:	6.1.3 CP corrections

Source: 	Ericsson, Qualcomm Incorporated

Title:  	RedCap CFR for MBS broadcast

Document for:	Discussion and Decision

# Introduction

In this contribution it is proposed to introduce a separate RedCap CFR for MBS broadcast.

# Background

History: LTE and eMBMS

In LTE the CN can indicate, via an operator defined QCI value, if the session is intended for UEs with limited capabilities (e.g. NB1/NB2, M1/M2 including CE level), see 23.246 section 6.3.2:

Table 6.3.2-2: QCI association with UE Categories and coverage level

| UE Categories supported | Operator defined QCI (not standardised) |
| --- | --- |
| IoT UE type M2, Coverage level: Medium | a |
| IoT UE type M2, Coverage level: Normal | b |
| IoT UE type M2 Coverage level: High | c |
| Both UE Category M1 and M2 UEs | d |
| IoT UE type NB2 | e |
| Both UE Category NB1 and NB2 | f |

The RAN can use this information for proper configuration (e.g. maximum TBS size and bandwidth).

MBS broadcast in NR: SA2 LS and RAN plenary

SA2 asked RAN1 if similar signalling would be beneficial for RedCap UEs and MBS broadcast [1]. But RAN1 indicated not having time to evaluate this [2], while RAN plenary agreed that any UE can support MBS broadcast [3].

RAN2#119

Possible enhancements to support RedCap UEs in MBS were discussed e.g. signalling the bandwidth of the MBS service in the service announcement, which was not agreed [4]. Some companies suggested that when a RedCap UE indicated to be interested in an MBS broadcast service, the gNB could decide to lower the configured bandwidth in SIB20, if needed (for all UEs and all services) [5].

# Discussion

The locationAndBandwidthBroadcast in SIB20 indicates the configured bandwidth for both MCCH and MTCH, i.e. it indicates the bandwidth used for all MBS broadcast sessions in the cell. If a RedCap UE does not support the configured bandwidth in SIB20 then the RedCap UE cannot receive MBS broadcast.

Observation 1: A RedCap UE cannot receive MBS broadcast when the bandwidth of the configured CFR in SIB20 exceeds the UE’s capability.

But a RedCap UE could indicate in MII message the frequencies/services it is interested to receive. And the gNB could use the MII signalling of a RedCap UE to lower the bandwidth of the default CFR in SIB20, if needed. But this approach has the following drawbacks/limitations/problems:

- If the PCell includes SIB20 and the configured bandwidth of the CFR exceeds the RedCap capabilities, then the Redcap UE would not be able to acquire the MCCH and the UE can only indicate interest in services based on SIB21 and USD info.

- A RedCap UE only sends MII signalling in connected mode when interested to receive MBS broadcast, i.e. the gNB does not know anything about RedCap UEs in RRC_IDLE or RRC_INACTIVE.

- It is not obvious that when a single RedCap UE in RRC_CONNECTED indicates to be interested in an MBS broadcast service, that the gNB would lower the configured bandwidth for all UEs and all MBS broadcast services. Furthermore a change of the CFR configuration interrupts the reception of MBS broadcast for all UEs/services, and it may also violate the QoS (e.g. required bitrate) of those services.

The gNB does not have an accurate view of the RedCap UEs interested in MBS services, and it may not be desirable to lower the bandwidth for all UEs and all MBS broadcast services to support (some) RedCap UEs:

Observation 2: The gNB does not have an accurate view of the interest of RedCap UEs in MBS services, and it may not be desirable to lower the bandwidth for all UEs and all MBS broadcast services to support some RedCap UEs.

To support group communications when some UEs have limited capabilities is complicated, and it is difficult to make it perfect for everybody. But we think it would be beneficial if the NW has the option to configure a separate RedCap CFR with limited bandwidth for RedCap UEs and not be required to lower the bandwidth for all UEs and services:

Proposal 1: Introduce a separate RedCap CFR which can be used when the configured bandwidth for the default CFR in SIB20 exceeds the RedCap capability.

The introduction of a RedCap CFR does not have impact on other UEs supporting MBS broadcast.

To assist the gNB when to configure a RedCap CFR it is beneficial when the CN indicates when the session is intended for RedCap UEs:

Proposal 2: Send LS to SA2 to ask SA2 to define solution for CN to indicate when a session is intended to be received by RedCap UEs.

The gNB may ignore the indication that a session is intended for RedCap UEs and not configure a RedCap CFR (e.g. when the configured bandwidth of the default CFR enables RedCap UEs to receive the service).

A RedCap UE uses the RedCap CFR when configured, otherwise it uses the default CFR provided that the UE supports the required bandwidth.

The gNB does not configure a RedCap CFR when the default CFR is also supported by RedCap UEs or when the CN does not indicate that the session is intended for RedCap UEs, i.e. low bitrate/bandwidth sessions are not mapped onto the RedCap CFR when the session is not intended for RedCap UEs.

For reliability and coverage purposes the gNB may transmit the MCCH/MTCH with maximum power and bandwidth (similar as for Paging/System Information). Thus when a separate RedCap CFR is configured, then also a separate mcch-ConfigRedCap is required, because the MCCH/MTCH transmissions for a RedCap cannot occur in the same slot(s) as for “normal” UEs.

# Summary

RAN2 is kindly asked to discuss MBS broadcast and RedCap:

Observation 1: A RedCap UE cannot receive MBS broadcast when the bandwidth of the configured CFR in SIB20 exceeds the UE’s capability.

Observation 2: The gNB does not have an accurate view of the interest of RedCap UEs in MBS services, and it may not be desirable to lower the bandwidth for all UEs and all MBS broadcast services to support some RedCap UEs.

Proposal 1: Introduce a separate RedCap CFR which can be used when the configured bandwidth for the default CFR in SIB20 exceeds the RedCap capability.

Proposal 2: Send LS to SA2 to ask SA2 to define solution for CN to indicate when a session is intended to be received by RedCap UEs.

# References

- S2-2203020, UE capabilities for MBS, SA2#150-e, LS out, To: RAN1, Cc: RAN, RAN2, RAN3

- R1-2205461, Reply LS on UE capabilities of MBS, RAN1#109-e, LS out, To: SA2, Cc: RAN

- RP-221861, Reply LS on UE capabilities for MBS, LS out, To: SA2, Cc: RAN1, RAN2, RAN3, RAN#96

- R2-2208885, Reply LS on the MBS broadcast service continuity and MBS session identification, LS out, To:SA4 Cc:RAN3, SA2, RAN2#119-e

- R2-2208884, [Offline 605] discussion report: Reply LS to SA4, Qualcomm, Report, RAN2#119-e

- R2-2208882, Summary of [AT119-e][602] Rel-17 MBS CP Other CP corrections, MediaTek, Report, RAN2#119-e

# Appendix: TP 38.331

#### –	SIB20

SIB20 contains the information required to acquire the MCCH configuration for MBS broadcast.

SIB20 information element

-- ASN1START

-- TAG-SIB20-START

SIB20-r17 ::=	SEQUENCE {

mcch-Config-r17                MCCH-Config-r17,

cfr-ConfigMCCH-MTCH-r17        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...,

[[

cfr-ConfigMCCH-MTCH-RedCap-r17 CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

mcch-ConfigRedCap-r17          MCCH-Config-r17         OPTIONAL   -- Cond CFR-RedCap

]]

}

MCCH-Config-r17 ::= SEQUENCE {

mcch-RepetitionPeriodAndOffset-r17   MCCH-RepetitionPeriodAndOffset-r17,

mcch-WindowStartSlot-r17             INTEGER (0..79),

mcch-WindowDuration-r17              ENUMERATED {sl2, sl4, sl8, sl10, sl20, sl40,sl80, sl160}     OPTIONAL, -- Need S

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, r2048, rf4096, rf8192, rf16384, rf32768, rf65536}

}

MCCH-RepetitionPeriodAndOffset-r17 ::= CHOICE {

rf1-r17                                INTEGER(0),

rf2-r17                                INTEGER(0..1),

rf4-r17                                INTEGER(0..3),

rf8-r17                                INTEGER(0..7),

rf16-r17                               INTEGER(0..15),

rf32-r17                               INTEGER(0..31),

rf64-r17                               INTEGER(0..63),

rf128-r17                              INTEGER(0..127),

rf256-r17                              INTEGER(0..255)

}

-- TAG-SIB20-STOP

-- ASN1STOP

SIB20 field descriptions

cfr-ConfigMCCH-MTCH Common frequency resource used for MCCH and MTCH reception. If the field is absent, the CFR for broadcast has the same location and size as CORESET0 and PDSCH configuration of MCCH is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1.

cfr-ConfigMCCH-MTCH-RedCap Common frequency resource used for MCCH and MTCH reception for RedCap UEs when the configured bandwidth in cfr-ConfigMCCH-MTCH exceeds the RedCap UE capability. If the field is absent, the RedCap UE can use cfr-ConfigMCCH-MTCH if the UE supports the configured bandwidth.

mcch-WindowDuration Indicates, starting from the slot indicated by mcch-WindowStartSlot, the duration in slots during which MCCH may be scheduled. Absence of this field means that MCCH is only scheduled in the slot indicated by mcch-WindowStartSlot. The network always configures mcch-WindowDuration to be shorter or equal to the length of MCCH repetition period.

mcch-ModificationPeriod Defines periodically appearing boundaries, i.e. radio frames for which SFN mod mcch-ModificationPeriod = 0. The contents of different transmissions of MCCH information can only be different if there is at least one such boundary in-between them. Value rf2 corresponds to two radio frames, value rf4 corresponds to four radio frames and so on.

mcch-RepetitionPeriodAndOffset Defines the length and the offset of the MCCH repetition period. rf1 corresponds to a repetition period length of one radio frame, rf2 corresponds to a repetition period length of two radio frames and so on. The corresponding integer value indicates the offset of the repetition period in the number of radio frames. MCCH is scheduled in the MCCH transmission window starting from each radio frame for which: SFN mod repetition period length = offset of the repetition period.

mcch-WindowStartSlot Indicates the slot in which MCCH transmission window starts.