3GPP TSG-RAN WG2 Meeting#120 		       					   											   R2-2212277

Toulouse, France, 14th – 18h November 2022		  	 	                                 is re-submission of R2-2210663

Source: 			ZTE corporation, Sanechips

Title: 	Further consideration on NTN neighbor cell list in SIB19

Agenda item:		6.10.3

Document for: 	Discussion and Decision

- Introduction

At RAN2#119e and the post email discussion [101], there has been discussion on the structure of the NTN neighbor cell list to allow configuration of more than one neighbor NTN cell associated with the same ntn-Config-r17. We share some further consideration on the structure of the NTN neighbor cell list to allow more flexibility and signaling reduction in configuration without impact the ASN.1.

- Discussion

-----------------------------------------------------------------------------ASN.1 structure agreed in R2-2209247--------------------------------------------------------------------------------

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigList-v1720. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies.

-----------------------------------------------------------------------------ASN.1 structure agreed in R2-2209247--------------------------------------------------------------------------------

Figure 1: Understanding of the latest NTN neighbor cell list structure

With the latest structure and field description agreed in R2-2209247 [1], which is also copied above, at most two cells can be associated with the same ntn-Config-r17.

We understand such configuration is still restrictive and would like to propose the following configuration and interpretation so that we can list up to 8 cells per satellite without repeat the ntn-Config-r17:

SIB19 field descriptions

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigListExt-v1720. If ntn-Config is absent for an entry in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt applies.

Figure 2: Understanding of the proposed change

Proposal 1: The following interpretation of the ntn-NeighCellConfigList and ntn-NeighCellConfigListExt in SIB19 should be agreed to allow more flexible configuration.

SIB19 field descriptions

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigListExt-v1720. If ntn-Config is absent for an entry in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt applies.

Proposal 2: Agree the corresponding 331 CR [2].

- Conclusion and proposals

Based on the analysis in previous sections, the following proposal is given:

Proposal 1: The following interpretation of the ntn-NeighCellConfigList and ntn-NeighCellConfigListExt in SIB19 should be agreed to allow more flexible configuration.

SIB19 field descriptions

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList (without suffix) and all elements of ntn-NeighCellConfigListExt-v1720. If ntn-Config is absent for an entry in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList or ntn-NeighCellConfigListExt applies.

Proposal 2: Agree the corresponding 331 CR [2].

- References

[1] R2-2209247 Corrections for Release-17 NTN, Ericsson

[2] R2-2210664 Clarification on the NTN neighbor cell list in SIB19, ZTE corporation, Sanechips