3GPP TSG RAN WG2 #120	                                    			R2-2212297

Toulouse, France, 14th – 18th November, 2022

Agenda Item:	8.6.3.2

Source:	InterDigital, Inc.

Title:	Other IoT-NTN mobility enhancements.

Document for:	Discussion

# Introduction

The WI on IoT NTN enhancements [1] includes the following objectives for mobility enhancement:

- Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

In RAN2#119e the following agreements were made:

Agreements:

- IoT NTN can use the mechanism for neighbour cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g. triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g. the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

In RAN2-119bis-e the following agreements were made:

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events

For both NR and IoT NTN, mobility enhancements so far have mainly introduced additional triggers for cell reselection, measurement reporting, and CHO. This is due to the predictability of satellite movement, and recognition that RSRP based triggers are less effective in NTN cells due to a more uniform RSRP measurement across the cell (as compared to terrestrial networks).

In this contribution we elaborate on the potential impact for NB-IoT and eMTC NTN mobility enhancement, other than measurements before RLF, in Rel-18.

# eMTC RRC_CONNECTED enhancements

## 2.1	Background

Rel-17 enhancements to RRC_CONNECTED focus primarily on adapting mobility and measurement procedures to non-terrestrial environments. Key modifications to mobility include additional execution conditions for conditional handover such as the A4 event, and time/location-based conditions:

- The location-based event is defined by the condEventD1, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold.

- The time-based event is defined by the condEventT1, where the event will be satisfied if conditional handover execution occurs between T1 and T2, where T2 = T1 + a duration.

Both time and location-based trigger conditions must be simultaneously configured with a measurement condition (e.g. A4) in Rel-17 NTN.

Other modifications apply to measurements, and include the following::

- location-based measurement reporting (based on eventD1, with similar execution conditions as condEventD1).

- Multiple SMTCs can be configured per carrier for a given set of cells using propagation delay difference, feeder-link delay, and serving/neighbour cell statellie ephemeris

- Measurement gaps can be configured using the same propagation delay differences as computed for SMTC.

## 2.2	 Discussion

For eMTC, it should be possible to re-use the connected mode enhancements to CHO and measurement triggering. As the WID suggests, minimum necessary changes should be introduced to adapt them to eMTC. In our understanding, changes/deviations from NR are primarily a stage 3 matter, for example the RRC specification style and arrangement is slightly different for how conditional handover is defined, however the stage 2 agreement could potentially already be made.

It was agreed to continue working on a new time-based trigger. We think we can simply re-use what was agreed for NR, and elaborate further on the previous agreements as follows:

Proposal 1: For eMTC NTN, introduce a time-based conditional reconfiguration trigger based on condEventT1 in NR, where the event will be satisfied if conditional handover execution occurs between T1 and T2, where T2 = T1 + a duration.

In the previous meeting, it was further agreed to support a location based trigger, therefore we can elaborate further based on NR as follows:

Proposal 2: For eMTC NTN, introduce an optional location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold.

In addition, we can elaborate more on the additional RSRP based trigger which has been introduced in NR in Rel-17 as follows:

Proposal 3: For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR).

For NR, both time and location-based trigger conditions must be simultaneously configured with a measurement condition (e.g. A4) in Rel-17 NTN. For eMTC, since power saving is one of the main considerations, we may need to allow only time based triggers to be enabled by the network for some devices or scenarios such that the UE does not necessarily need to fulfil e.g. RSRP threshold if configured with location and/or time based triggers only.

Proposal 4: Time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition) for eMTC NTN.

# IDLE/INACTIVE enhancements

## 3.1	Background

NR Rel-17 enhancements to IDLE/INACTIVE cell reselection focus primarily on new measurement rules. Two key enhancements have been specified based on UE distance from a cell reference point, and on the time a quasi-Earth cell will stop serving the current area, indicated by t-Service. The cell reference point, t-Service and distanceThresh (the parameter used to evaluate the distance condition) are optionally broadcast in SIB19, a new system information block which carriers NTN-specific information.

The location-based enhancement enables measurement relaxation when a UE is located within a threshold distanceThresh from a cell reference point (usually the cell center). A UE may not perform intra-frequency measurements, measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority if all following conditions are satisfied:

- The serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ;

- The UE has valid UE location information (it’s up to UE implementation to have available UE location information); and

- the distance between the UE and the serving cel reference location is shorter than distanceThresh.

The time-based enhancement requires a UE to perform cell re-selection measurements in a quasi-earth fixed cell at some time prior to t-Service (the exact timing is up to UE implementation). The UE shall perform intra-frequency, inter-frequency, or inter-RAT measurements before t-Service regardless of the distance between the UE and serving cell measurement or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

These new distance and time-based measurement rules do not affect measurement of higher-priority NR inter-frequency or inter-RAT frequencies. The UE shall perform these measurements according to [2] regardless of remaining service time or distance from the cell reference point.

## 3.2	 Discussion

There was no location-based cell reselection criteria introduced in R17 for IoT-NTN, for similar reasons as discussed in the previous section. It was also noted in the previous meeting that it is not a requirement for eMTC UEs to support simultaneous GNSS and LTE operation, and power consumption for IoT devices needs to be considered, however this does not mean it is not possible for some devices.

For UEs capable of simultaneous GNSS and LTE it should also be possible to also support location based CHO. Not all devices have the same battery requirements, and this could be considered as an optional feature in R18.

Proposal 5: Introduce optional distance-based relaxation for RRC_IDLE based on distanceThresh in NR.

# Conclusion

In this contribution we discuss the potential mobility enhancements for R18 IoT-NTN and have the following proposals:

For conditional reconfiguration triggers for eMTC:

Proposal 1: For eMTC NTN, introduce a time-based conditional reconfiguration trigger based on condEventT1 in NR, where the event will be satisfied if conditional handover execution occurs between T1 and T2, where T2 = T1 + a duration (similar to condEventT1 in NR)

Proposal 2: For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR)

Proposal 3: For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR).

Proposal 4: Time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

For Idle/Inactive:

Proposal 5: Introduce optional distance-based relaxation for RRC_IDLE based on distanceThresh in NR.

# References

- RP-221806,  “Revised WID on IoT NTN enhancements”