3GPP TSG-RAN WG2 Meeting #120                                                                        R2-2212333

Toulouse, France, November, 2022

Agenda Item	:	8.5.4.2

Title	:	Scheduling Enhancement for XR

Source	:	Google Inc.

Work Item	:	FS_NR_XR_enh

Document for	:	Discussion and Decision

# Introduction

A new Rel-18 Study Item for XR enhancements in NR has been approved in RAN#94. Three main topics are to be studied in RAN1 and RAN2: XR-awareness, XR-specific power saving, and XR-specific capacity improvements.

As clarified in [1] ,the study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166).

The objectives of the Study Item includes:

- XR-specific capacity improvements (RAN1, RAN2):

  - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

    - SPS and CG enhancements;

In RAN2#119bis-e, the following agreements have been reached:

Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting.

RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

In this Tdoc, we discuss XR-specific scheduling enhancements to improve XR system capacity.

# UL CG Enhancement

Configured Grant (CG) is very useful to reduce the latency of the UL transmission compared to DG. The UE can transmit UL data without the need for sending a scheduling request (SR) or the need for receiving an UL grant from the network.

The UL pose/control information arrives periodically (e.g. 4 ms period) with no jitter. The packet size for this traffic is fixed (e.g. 100 bytes packets). CG scheduling is a good option to support the UL pose/control information. Legacy CG type-1 or CG type-2 could be used with no enhancement for this type of traffic.

- CG scheduling to be used for UL pose/control information.

UL AR video traffic can also benefit from the use of CG to reduce the latency since the UL traffic is quasi-periodic with very small or no jitter compared to the DL traffic. CG can also be used to report potential assistance information and gNB will have the flexibility to continue using CG or switch to DG scheduling.

- CG scheduling to be used for UL AR traffic jointly with DG.

However, configuring CG resources to cater for the largest packet size is very resource inefficient and can penalize the system capacity. Also, the jitter, although small, can impact the performance of CG. Therefore, configuring multiple PUSCH occasions in a CG period is useful to address the variability in the UL packets sizes.

The UE has awareness about the UL data and can request increase or decrease of the CG resources. UE can send a request via UCI piggy-backed on PUSCH or via MAC CE for adjustment of the UL allocated PUSCH occasions. The UE can send explicit information about the size of data in the buffer via BSR or can be more explicit by signalling the number and the sizes of the PDU sets waiting to be transmitted together with information about their priority level or their remaining delay budget. gNB can use this information to adjust CG resources and to adjust the CG configuration. E.g. for high priority PDU set with small remaining delay budget, the gNB can advance the CG PUSCH occasions, it can also allocate larger resources and reduce MCS to ensure reliability. gNB can also decide to cancel some PUSCH occasions. e.g. for smaller PDU sets or some low priority PDU sets with limited remaining delay budget. The UE can be given the ability to request extra PUSCH occasions or to cancel some occasions.

- Multiple PUSCH occasions in a CG period is useful for UL AR traffic.

Multiple active CGs is also useful for UL AR to address the jitter and the variable packet sizes. Currently, joint release of multiple CGs is supported in NR, but only one CG can be activated at a time. Hence, joint activation of multiple CGs can be useful.

- Support joint activation of multiple CG configurations.

Also, current UL AR traffic periodicities are not supported by CG. Hence, current CG periodicities should be enhanced to align with the non-integer XR traffic periodicities.

- CG periodicities require enhancement to align with UL AR traffic periodicities

# Adaptive number of HARQ retransmissions

gNB and UE can have awareness about delay budget and priority of PDUs or PDU sets. gNB can also have awareness about RAN congestion. Therefore, gNB and UE should be able to decide to drop a PDU or a PDU set due to limited delay budget (e.g. PDB close to expiry) or due to congestion (e.g. dropping low priority PDUs or PDU sets).

gNB and UE should be able to decide to limit the number of HARQ retransmissions to meet the delay budget and limit the congestion. In DL, gNB can signal to the UE that the current transmission will have no retransmissions mainly for UL pose/control information which have very limited delay budget.  Then, the UE doesn’t need to send HARQ-ACK feedback and can flush the buffer after the decoding regardless of the decoding result. This will reduce the DL retransmissions and the HARQ-ACK feedback overhead and improves the DL and UL capacity.

In UL, if it consists of the first transmission, UE can indicate to the gNB that the current transmission will have no retransmissions.  If it consists of a retransmission UE can also indicate that this is the last retransmission. In these cases, gNB doesn’t need to schedule a retransmission. UE can be more explicit and indicate the remaining delay budget for a specific PDU or PDU set and signal this information to the gNB via UCI piggy-backed on PUSCH or via MAC-CE.

Also, the network can configure a retransmission-less CG configuration for example for the UL pose/control information. gNB doesn’t need to schedule retransmissions and the UE doesn’t need to monitor PDCCH for retransmissions. One alternative to configure a retransmission-less CG is to configure a zero value for the configuredGrantTimer.

- Retransmission-less CG configuration could be studied for better system capacity and better UE power saving.

Also, in RAN1#11bis-e, the following conclusion has been reached:

Conclusion

“Retransmission-less CG for UL pose transmission (Item 3.3-5)” is a RAN2 issue, leave the discussion to RAN2, RAN1 does not further investigate the issue.

- Note: how to capture evaluation results and findings will be separately discussed

This topic has been discussed in RAN1 and left to RAN2 to discuss it and decide about it.

# Conclusion

In this contribution, we have presented several potential enhancements for the XR service that could be studied and explored further to enable XR service capacity enhancement.

We made the following proposals:

- CG scheduling to be used for UL pose/control information.

- CG scheduling to be used for UL AR traffic jointly with DG.

- Multiple PUSCH occasions in a CG period is useful for UL AR traffic.

- Support joint activation of multiple CG configurations.

- CG periodicities require enhancement to align with UL AR traffic periodicities

- Retransmission-less CG configuration could be studied for better system capacity and better UE power saving.

# References

- RP-213587, “New SID: Study on XR Enhancements for NR” Nokia (moderator), 3GPP TSG RAN#94e, Dec. 2021

- 3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”

- RAN1#109e chairman notes.