3GPP RAN WG2 Meeting #120	R2-2212336

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	8.7.2

Source:	InterDigital

Title:	Blind Msg3 retransmission in Rel-18 NTN

Document for:	Discussion, Decision

# Introduction

An objective for Rel-18 NR NTN is to study the applicability of general NR coverage enhancement solutions to non-terrestrial networks [1]. This includes identifying potential issues and enhancements to coverage considering NTN-specific characteristics such as large propagation delay and satellite movement. The primary use cases are voice and low-rate data services using commercial smartphones. Work is currently underway with a 6-month study phase to derive clear and limited scope.

The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above:

- NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels

- NTN-specific techniques for improved diversity and/or reduced polarization loss

- Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR

  - NOTE: Intent is not to introduce a new codec.

Several Msg3 coverage enhancement techniques are currently supported for in Rel-17 NTN. For example, Type A PUSCH repetition is supported by starting the ra-ContentionResolutionTimer in the first symbol after the end of all repetitions of the Msg3 transmission plus the UE-gNB RTT. Blind Msg3 retransmission is supported by ignoring ra-ContentionResolutionTimer expiry if a PDCCH addressed to TC-RNTI indicating an UL grant for a Msg3 retransmission is received after the start of the ra-ContentionResolutionTimer.

The following additional Rel-18 NTN coverage enhancements have been agreed in RAN2#119bis-e [4]:

- RAN2 thinks a UE may use application layer frame aggregation by implementation (no RAN2 spec impacts). (RAN2 can further discuss whether RAN needs to know whether UE is using frame aggregation in the voice packet)

- RAN2 understands that it is up to network implementation to decide whether to configure SDAP header and integrity protection for a VoNR DRB to reduce the protocol overhead (no RAN2 spec impacts)

One issue identified and so far not addressed is the large delay after initial Msg3 transmission before blind Msg3 retransmission grant reception is possible. This document continues the discussion on blind Msg3 retransmission grant reception in NTN, summarizing the issue and previously proposed solutions. An evaluated of each solution is then presented and possible improvements described.

# Discussion

## PDCCH monitoring during a 4-step Random Access procedure

Upon Random Access Preamble transmission, the MAC entity will start the ra-ResponseWindow at a PDCCH occasion specified in [2]. While the ra-ResponseWindow is running, the UE will monitor PDCCH. Upon successful RAR reception containing Random Access Preamble identifiers that match the transmitted PREAMBLE_INDEX, the MAC entity may stop ra-ResponseWindow and hence monitoring for Random Access Response(s).

Upon Msg3 (re)transmission, the MAC entity will start the ra-ContentionResolutionTimer in the first symbol after the end of Msg3 (re)transmission. While the ra-ContentionResolutionTimer is running, the UE will monitor PDCCH. The MAC entity will stop the ra-ContentionResolutionTimer if notification of a reception of a PDCCH transmission is received from lower layers and the PDCCH transmission is addressed to C-RNTI or TEMPORARY_C-RNTI.

This procedure is modified for non-terrestrial networks by offsetting the start of ra-ResponseWindow and ra-ContentionResolutionTimer by the UE-gNB RTT. This ensures that the ra-ResponseWindow and ra-ContentionResolutionTimer do not prematurely expire due to the long propagation delays characteristic of non-terrestrial networks.

## Blind Msg3 retransmission in non-terrestrial networks

In terrestrial networks the gap between a MAC entity stopping the ra-ResponseWindow and starting the ra-ContentionResolutionTimer is usually quite brief, allowing near-continuous monitoring of PDCCH. By offsetting the start of the ra-ContentionResolutionTimer in NTN this gap is significantly increased, limiting the ability to quickly receive a blind Msg3 retransmission grant after the ra-ResponseWindow is stopped (Figure 1).

Figure 1: (A) Msg3 blind retransmission grant reception in a terrestrial network; (B) Msg3 blind retransmission grant reception in a non-terrestrial network;

Proposed solutions for fast (i.e. less than UE-gNB RTT) blind Msg3 retranmission grant reception after initial Msg3 transmission can be summarized as follows:

Solution 1) Do not stop the ra-ResponseWindow after initial RAR reception: In this case, the UE will continue to monitor PDCCH for a blind Msg3 retransmission grant after successfully receiving RAR.

Although this will have less impact to UE power consumption as compared to Solution 2, the differential delay due to very large cell sizes in NTN may mean a UE on cell edge (a key beneficiary of coverage enhancements) will only receive the RAR close to ra-ResponseWindow expiry anyways. In these scenarios, the additional benefit of continued monitoring may be limited unless the ra-ResponseWindow is extended (causing even greater power consumption).

Solution 2) Starting the ra-ContentionResolutionTimer immediately after Initial Msg3 transmission: In this case, a UE operating in a non-terrestrial network will start the ra-ContentionResolutionTimer in the first symbol after the end of initial Msg3 transmission to monitor for a blind Msg3 retransmission grant.

Unless an additional configuration is introduced to disabled this feature, the UE would have to start the ra-ContentionResolutionTimer after every initial Msg3 transmission in a non-terrestrial network regardless of whether the network intends to send a blind Msg3 retransmission grant or not. Furthermore, the UE may have to start the ra-ContentionResolutionTimer a second time after the UE-gNB RTT due to the same initial Msg3 transmission, which is undesirable from a UE power consumption perspective.

Observation 1:	Previous solutions for fast (i.e. < UE-gNB RTT) blind Msg3 retransmission grant reception in NTN mandate additional monitoring for all UEs regardless of coverage conditions, causing unecessary power consumption.

A second issue is that under certain NTN deployment scenarios (e.g., GEO) the propagation delay is larger than the ra-ContentionResolutionTimer. In this case, if the UE does not receive a blind Msg3 retransmission grant during the first monitoring duration (started immediately after initial Msg3 transmission), the ra-ContentionResolutionTimer may expire before the second monitoring duration (started in the first symbol after initial Msg3 transmission plus the UE-gNB RTT). Under current specification the UE will then consider Contention Resolution unsuccessful.

Observation 2:	If the ra-ContentionResolutionTimer is started immediately after initial Msg3 transmission and a blind Msg3 retransmission grant is not received before timer expiry, the UE will declare premature Contention Resolution failure if the timer duration is less than UE-gNB RTT.

Considering this issue arises due to offsetting the start of the ra-ContentionResolutionTimer by the UE-gNB RTT (which is only done in NTN), any resulting enhancements would be NTN-specific and therefore included as part of this work item objective. Furthermore, this feature is a minor enhancement over solutions already specified in Release 17 and does not have any RAN1 impact (since it relates primarily to RA timer handling, which is specified in MAC).

RAN2 may therefore study solutions related to enable fast (i.e. < UE-gNB RTT after initial Msg3 transmission) blind Msg3 retransmission grant reception in non-terrestrial networks without waiting for the conclusion of the RAN1 evaluation. However, it is noted that outcome of RAN1 studies may be useful when determining whether/when to enable this feature.

Proposal 1:	Study enhancements to enable fast (i.e. < UE-gNB RTT after initial Msg3 transmission) blind Msg3 retransmission grant reception in Rel-18 NTN.

To balance additional UE power consumption with the latency and coverage benefits of fast blind Msg3 retransmission grant reception, RAN2 may consider additional solutions. For example, whether the UE will additionally monitor for a blind Msg3 retransmission grant may be configurable by the network, or indicated in RAR. In this way, the UE shall only perform additional monitoring when necessary e.g. when the UE is coverage limited.

Proposal 2:	Fast blind Msg3 retransmission grant reception is optional and enabled based on configuration and/or indication (e.g. via RAR).

# Conclusion

In this contribution the following observation and proposals were made concerning fast blind Msg3 retransmission grant reception in non-terrestrial networks:

Observation 1:	Previous solutions for < UE-gNB RTT blind Msg3 retransmission grant reception mandate additional monitoring for all UEs regardless of coverage conditions, increasing power consumption uncessecarily.

Observation 2:	If the ra-ContentionResolutionTimer is started immediately after initial Msg3 transmission and a blind Msg3 retransmission grant is not received prior to timer expiry, UE may declare premature Contention Resolution failure in some NTN deployment scenarios (e.g. GEO).

Proposal 1:	Study enhancements to enable fast (i.e. < UE-gNB RTT after initial Msg3 transmission) blind Msg3 retransmission grant reception in Rel-18 NTN.

Proposal 2:	Fast blind Msg3 retransmission grant reception is optional and enabled based on configuration and/or indication (e.g. via RAR).

# References

RP-221819 – 3GPP Work Item Description: NR NTN enhancements (Rel-18)

3GPP TS 38.213: Physical layer procedures for control v17.2.0

3GPP TS 38.321: Medium Access Control (MAC) protocol specification v17.1.0

Draft_R2_119bis-e_meeting_report_v1