3GPP TSG-RAN WG2 #120	R2-2212383

Toulouse, France,  14th - 18th Nov 2022

Source:		NEC

Agenda item:		8.3.6	Others

Title:		Discussion on Wake Up Signalling and paging-less NES cells

Document for:		Discussion and decision

# 1. Introduction

During email discussion [1], some remaining issues were highlighted:

List of remaining issues on NES cell without SIB: The following two scenarios will be studied: - The anchor cell transmit SIs for NES cells, and NES cells transmit neither SSBs nor SIs; - The anchor cell transmit SIs for NES cells, and NES cells transmit SSBs but not SIs. Aspects to be addressed: - the detailed solution and potential specification impacts for each direction; - the benefits for energy saving and constraints for each direction (baseline is the anchor cell + SIB-less cell); - impact on the UE behaviour, e.g. whether the UE always camp on the anchor cell, or can also camp on the NES cells (this is rather dependent on specific directions), how the UE will determine which cell to perform RACH; the applicable RRC state, e.g. whether it only applies to idle mode, or also applies to connected mode; - applicability of existing solutions, e.g. how much we can reuse from NB-IoT solution and what needs to be enhanced compared with NB-IoT - how to handle paging: - detailed solution description, benefits and potential specification impact (baseline is the anchor cell + SIB-less cell); - impact on UE behaviour on cell camping; - applicability of existing solutions whether a common solution can be applied to both SSB-less and SIB-less solutions

In RAN1, WUS solutions were discussed in more detail in [2]:

- Technique #A-3: wake up signal (WUS) for gNB - Support of wake up of gNB that is in a dormant power state/energy saving state (e.g., SSB-less/SIB1-less/SSB relaxed state), support of wake up signal (WUS) transmitted by the UE/neighboring gNB including UEs to the gNB (e.g. the gNB/cell in dormant state or the anchor gNB/cell). - Whether UE detection of a dormant power state/energy saving state is required before WUS transmission should be identified. - Resource reserved for WUS and the assumption of the gNB receiver should be identified - This may include support of assistance information from the UEs intended to aid wake up operations by the gNBs. - This is mainly for connected mode UEs

WUS has been extensively discussed in RAN1 with multiple solutions proposed. In this contribution, we discuss RAN2 impacts as well as discuss paging issues associated with dormant or paging-less cells.

# 2. Discussion

First, we need a clarification about NES cell. In our opinion, an NES cell is a cell that can implement an NES configuration (sometimes referred to as “being in an NES state”). This cell could be in legacy operation and not implement any NES configuration, typically if its load is medium or high. However, depending on traffic condition, the NES cell could at any moment decide to change its state and implement an NES configuration.

Hence, there is still a need for UEs to be aware that the cell is an NES cell, even when it is in legacy operation mode, as signalling regarding potential future/upcoming NES states may be needed.

Proposal 1: RAN2 to clarify the definition of an NES cell.

Proposal 2: An NES cell is a cell susceptible to change its NES state and implement an NES configuration but can also be operating in legacy mode.

Another issue is the case of Wake Up Signalling. Solutions to adapt DTX/DRX cycles according to connected UEs have been proposed to implicitly provide WUS opportunities at NES cell. However, a cell may be completely asleep or not provide SSB, SIB, RACH, or indeed WUS resources. In this case, WUS can only be performed at the anchor cell.

Observation 1: DTX/DRX adaptation is not applicable to dormant NES cells. A mechanism to achieve WUS at anchor cell is necessary in this case.

Proposal 3: RAN2 needs to specify a WUS procedure that is performed at the anchor cell only.

Furthermore, it is unclear whether there will be one or multiple anchor cells. A typical scenario would be that the NES cell is a capacity cell located within the coverage of one umbrella cell. However, an NES cell may also be at an umbrella cell’s edge and be within reach of multiple umbrella cells or have other small cells neighbour within the coverage of the umbrella cell, some of which could be NES cells themselves. The case of a centralised or distributed WUS procedure will impact NW decision and overall WUS solution benefits for NES.

Observation 2: Depending on whether there is one dedicated anchor cell in charge of waking up the NES cell or multiple anchor cells susceptible to do so, this would have an impact on the WUS procedure.

Proposal 4: RAN2 to discuss the scenarios of one or multiple anchor cells and its impact on WUS procedure.

Lastly, we need to discuss the issue of UEs camping on an NES cell and their connection state regarding the anchor cell, especially related to paging.

In NES cells, paging may be optional. In this simple case, a UE would need to rely on its anchor cell for paging. There is also the case that paging periodicity is particularly long in an NES cell, which could negatively impact UEs camping on NES cells.

Proposal 5: RAN2 to discuss paging enhancements at anchor cell for UEs camping on an NES cell.

# 3. Conclusion

In this paper, we discuss NES issues for NES and anchor cells and propose the following:

Proposal 1: RAN2 to clarify the definition of an NES cell.

Proposal 2: An NES cell is a cell susceptible to change its NES state and implement an NES configuration but can also be operating in legacy mode.

Proposal 3: RAN2 needs to specify a WUS procedure that is performed at the anchor cell only.

Proposal 4: RAN2 to discuss the scenarios of one or multiple anchor cells and its impact on WUS procedure.

Proposal 5: RAN2 to discuss paging enhancements at anchor cell for UEs camping on an NES cell.

# Reference

[1] [POST119bis][304][NES] TP on cell selection/reselection and SSB/SIB-less  (Huawei)

[2] R1-2210744, Discussion Summary #5 for energy saving techniques of NW energy saving SI (Intel)