3GPP TSG-RAN WG2 #120	Tdoc R2-2212391

Toulouse, November 14 –18, 2022

Agenda Item:	8.19

Source:	Ericsson

Title:	On RAN2 aspects for UL TX switching Rel-18

Document for:	Discussion, Decision

# 1	Introduction

RAN2 received LSs with further RAN1/RAN4 agreements with signaling impact (R1-2210724/ R4-2217741). This contribution discusses a general aspect for UL TX switching and the points raised in the LSs.

# 2	Discussion

## 2.1	General aspect

For Rel-17 UL Tx switching, it was concluded that the UE must support at least one common switching option (Option 1:Switched UL or Option 2:Dual UL) between 1Tx-2Tx switching and 2Tx-2Tx switching:

uplinkTxSwitching-OptionSupport2T2T-r17

Indicates which option is supported for dynamic UL 2Tx-2Tx switching for inter-band UL CA. switchedUL represents option 1 as specified in TS 38.214 [12], dualUL represents option 2 as specified in TS 38.214 [12], both represents both option 1 and option2 as specified in TS 38.214 [12]. The field is mandatory for inter-band UL CA cases where UE supports dynamic UL 2Tx-2Tx switching. The UE indicating support of this feature shall indicate support of at least one common switching option between uplinkTxSwitching-OptionSupport2T2T-r17 and uplinkTxSwitching-OptionSupport-r16.

As discussed during RAN2#119-e in R2-2207333, a possible way forward would be to not specify any dependency between Rel-17 and Rel-16 UL Tx switching options. But given the above conclusion, it would be good to follow the same principle for Rel-18 UL Tx Switching for the sake of consistency and avoid repeating the discussion already held for the conclusion in Rel-17. This may be further discussed depending on RAN1 outcome, but it would be good to consider it as a baseline. At this stage, it could not be captured in the same level of detail as for Rel-17, i.e. it is not clear how the case of 3 and 4 bands would be supported. But we should expect at least some dependency between the Rel-18 support of the feature and the Rel-17 support.

As a baseline, RAN2 understands that a UE supporting Rel-18 UL Tx switching would also support at least one switching option (Option 1:Switched UL or Option 2:Dual UL).

## 2.2	RAN1 progress

On the RAN1 progress, several points were discussed in R1-2210724. This contribution highlights the ones that should have an immediate RAN2 impact.

The following was concluded in RAN1#110bis-e:

- Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

    - Note：If there is no report on the supported band pair(s) for concurrent transmission while the UE reports “dualUL” or “both” for the band combination, gNB may assume that the UE supports concurrent transmission on all the band pairs within the band combination

  - Alt.3: report {dualUL} for each band pair in the band combination

    - Note: Within the band combination, the UE shall be capable of being operated in switched UL mode for all band pairs

For the UE capability reporting, since the UE mechanism to report UL Tx switching in Rel-16 and Rel-17 is based on the report of UL Tx switching option per band combination, it would be simpler to adopt the same for Rel-18, i.e. Alt.2. The other options would not only imply higher overhead and more gNB complexity e.g. one would have to understand the interaction between the newly defined per-band framework with the existing per-BC framework.

The UE capability report of Tx switching option (switchedUL, dualUL, both) is done per BC.

RAN1 continues:

- Ask RAN2 to consider following alternatives and specify gNB configuration

  - Alt.1: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination)

  - Alt.2: configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination)

  - Alt.3: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination), and configure combination(s) of serving cells (i.e., as supported serving cell pair(s) for each band pair in the band combination) for concurrent transmission

For the UE configuration, since the UL Tx switching configuration in Rel-16 and Rel-17 is performed for the applicable serving cells, it would be simpler to adopt the same for Rel-18, i.e. Alt.2. While Alt.1 may seem simpler, it deviates from the current way of configuring UL Tx switching and thus would end up being more complex to configure the feature in multiple levels. Similarly, Alt.3 would introduce a new logic of combination of serving cells, which may not be compatible with the current Rel-16 and Rel-17 configuration.

The configuration of Tx switching option for combination(s) of serving cells (i.e., for each band pair in the band combination).

## 2.3	RAN4 progress

On the RAN4 progress, several points were discussed in R4-2217741. This contribution highlights the ones that should have an immediate RAN2 impact.

For DL interruption, the following was agreed in RAN4:

In general, RAN4 agreed to reuse the Rel-16/17 agreement, and reached the following agreements:

•	For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

•	For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

−	UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

As stated by RAN4, this is essentially the same behaviour as for Rel-16 and Rel-17. It makes sense to simply confirm in RAN2 the signaling for DL interruption.

For the cases where DL interruption is applicable, the UE capability report of DL interruption is defined as a bitmap per UL Tx Switching pair per BC.

On UL MIMO coherence, the following was agreed:

•	For Rel-18 Tx switching across 3 or 4 bands, apply per band per BC capability on UL-MIMO coherence for the 2Tx-capable UL band(s). If the per band per BC capability on UL-MIMO coherence is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL band(s) for Tx switching.

•	It is up to RAN2 discussion on whether the Rel-17 per band per BC signalling on UL-MIMO coherence capability for 2Tx-2Tx switching can be reused for Rel-18 Tx switching.

Note it was left to RAN2 to decide on whether uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 can be reused for Rel-18 Tx switching. If our first proposal is agreeable, i.e. that a UE supporting Rel-18 UL Tx switching would also support at least one switching option (Option 1:Switched UL or Option 2:Dual UL) as defined for the Rel-17 capability of UL Tx switching, then it should be possible to reuse the Rel-17 field for UL-MIMO coherence, since the UE should anyway indicate support for the Rel-17 UL Tx switching when indicating support for the Rel-18 UL Tx switching. If the UE supports UL Tx coherence e.g. for Rel-17 but not for Rel-18, it can always include a BC where it indicates support of Rel-18 UL Tx switching but no UL Tx coherence, and a fallback BC where it indicates support of Rel-17 UL Tx switching and UL Tx coherence but no support of Rel-18 UL Tx switching.

The field uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 can also indicate support of UL Tx coherence for Rel-18 UL Tx Switching.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	As a baseline, RAN2 understands that a UE supporting Rel-18 UL Tx switching would also support at least one switching option (Option 1:Switched UL or Option 2:Dual UL).

Proposal 2	The UE capability report of Tx switching option (switchedUL, dualUL, both) is done per BC.

Proposal 3	The configuration of Tx switching option for combination(s) of serving cells (i.e., for each band pair in the band combination).

Proposal 4	For the cases where DL interruption is applicable, the UE capability report of DL interruption is defined as a bitmap per UL Tx Switching pair per BC.

Proposal 5	The field uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 can also indicate support of UL Tx coherence for Rel-18 UL Tx Switching.