3GPP TSG-RAN WG2 Meeting #120	R2-2212399

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On the mapping of the resource pool index in DCI format 3_0

WID/SID:	NR_SL_Relay-Core

Document for:	Discussion and Decision

# 1	Introduction

In the last RAN1#110bis meeting, RAN1 identified a problem in the CR R1-2210524 related to the mapping of the resource pool index in DCI format 3_0. In the corresponding LS R1-2210585, RAN1 has asked RAN2 to resolve the identified issue in RAN2#120 meeting.

In this contribution we will discuss the problem and provide a way forward on how to resolve the issue.

# 2	Discussion

## 2.1	Non-unique resource pool index in DCI format 3_0

In Rel-17 SL-Relay, RAN2 has introduced an additional resource pool type dedicated for SL relay discovery only, namely sl-DiscTxPoolScheduling. In principle both resource pool types, the shared pool (sl-TxPoolScheduling) and the dedicated discovery pool (sl-DiscTxPoolScheduling) can be configured simultaneously. For NR sidelink in resource allocation mode 1, RAN1 has identified the problem that it is not clear to which resource pool (either sl-TxPoolScheduling or sl-DiscTxPoolScheduling) the resource pool index in the DCI format 3_0 (defined in TS38.212 section 7.3.1.4) is referring to, in case both sl-TxPoolScheduling and sl-DiscTxPoolScheduling are configured.

Observation 1: If both resource pool types, shared resource pool (sl-TxPoolScheduling) and dedicated discovery resource pool (sl-DiscTxPoolScheduling) are configured, it is not clear to which resource pool the resource pool index in DCI format 3_0 is referring to.

Since the DCI format 3_0 has been introduced in Rel-16 for NR sidelink without Rel-17 SL relay support in mind at the time, the introduction of a new dedicated resource pool type for discovery in NR SL relay Rel-17, has led to this mapping problem of the resource pool index in DCI format 3_0.

Without RAN2 resolving this issue, the variable (IE-information element) resource pool index in DCI format 3_0 in the current form is no longer unique in Rel-17.

Observation 2: The mapping of the IE resource pool index in DCI format 3_0 (specified in Rel-16) is not unique in Rel-17 if both sl-TxPoolScheduling and sl-DiscTxPoolScheduling are configured.

Although the specification of the DCI falls under RAN1 responsibility, RAN1 has asked RAN2 in the LS R1-2210585 “to decide whether/how to specify the indexing of the configured Tx resource pools”.

In case RAN2 decides not to do anything at RAN2#120 meeting, the general concept of having a dedicated resource pool for discovery is led ad absurdum and potentially SL-relay Rel-17 is not implementable (at least not with a dedicated resource pool for discovery). Hence the question is not “whether RAN2 specifies the indexing of the configured TX resource pools” but rather “how to specify the indexing of the configured TX resource pools”.

Observation 3: RAN2 not specifying anything (related to R1-2210585) in RAN2#120 is not an option.

Proposal 1: RAN2 to discuss potential solutions in RAN2#120 related to the problem raised by RAN1 in R1-2210585.

## 2.2	Compatibility Issue between Rel-16 and Rel-17 with respect to DCI format 3_0

On a way forward i.e. “how to resolve the mapping issue of the resource pool index in DCI format 3_0” RAN2 should keep in mind that the targeted solution should maintain compatibility of the DCI format 3_0 between Rel-16 and Rel-17 (plus future releases as well). Since the ASN.1 for Rel-17 is frozen, it is also noted that RAN2 should not resolve the issue in a way that would necessitate ASN.1 changes (regardless of whether those changes would be BC or NBC) for Rel-17 specification.

Proposal 2: RAN2 shall not create any incompatibilities between Rel-16 and Rel-17 for DCI format 3_0 to resolve the mapping issue of the resource pool index in DCI format 3_0.

Concerning the total number of resource pools, the maximum number of transmit resource pools is fixed to 8 by

# 3	Conclusion

In this Tdoc we have made the following observations and proposals:

Observation 1: If both resource pool types, shared resource pool (sl-TxPoolScheduling) and dedicated discovery resource pool (sl-DiscTxPoolScheduling) are configured, it is not clear to which resource pool the resource pool index in DCI format 3_0 is referring to.

Observation 2: The mapping of the IE resource pool index in DCI format 3_0 (specified in Rel-16) is not unique in Rel-17 if both sl-TxPoolScheduling and sl-DiscTxPoolScheduling are configured.

Observation 3: RAN2 not specifying anything (related to R1-2210585) in RAN2#120 is not an option.

Proposal 1: RAN2 to discuss potential solutions in RAN2#120 related to the problem raised by RAN1 in R1-2210585.

Proposal 2: RAN2 shall not create any incompatibilities between Rel-16 and Rel-17 for DCI format 3_0 to resolve the mapping issue of the resource pool index in DCI format 3_0.