3GPP TSG-RAN WG2 Meeting #120	R2-2212400

Toulouse, France, 14 – 18 November 2022

Agenda item:	6.15.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On resource exclusion for random resource selection

WID/SID:	NR_SL_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#118, RAN2 sent an LS to RAN1 asking whether the following was a valid scenario:

In RAN2#118-e meeting, RAN2 discussed the inter-UE coordination scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g. UE B performs mode 2 random resource selection, etc. For this case, RAN2 has the following questions:

As well as

If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not?

This Tdoc discusses the reply of RAN1 and propose a solution.

# 2	Discussion

As per the response of RAN1, the reply to the first question was somewhat ambiguous, stating that the scenario is a “possible” scenario. Our understanding is that the term ‘possible’ covers the intended scope of the RAN2 enquiry on the validity of the problem.

Observation 1: RAN2 should interpret the wording in RAN1’s reply to Q1 in LS R2-2206314 as the scenario being valid.

As for the second question on whether the exclusion needs to be performed by UE B for the non-preferred resource set, ut is not very clear, as the reply states that RAN1 did not reach consensus. However, RAN1 confirms that the issue exists and so we believe it should be up to RAN2 to reach consensus on how to handle the discussed case. One can also argue that this implementation-related issue arises mainly in MAC specification, where it has yet to be discussed how to obtain the IUC information before random selection, and RAN2 have assumed the information is available when needed.

Observation 2: As RAN1 agrees on the issue being valid but could not reach consensus on a solution, and random resource selection is specified in MAC, it should be decided in RAN2 how to handle this issue.

Proposal 1: RAN2 to discuss how to resolve the issue consisting of that the UE B receives the IUC Scheme 1 non-preferred resource set from UE A but the UE B does not perform sensing in the resource pool associated with the non-preferred resource set (e.g. UE B performs mode 2 random resource selection).

Below we present three possible solutions in the order of improving performance but higher complexity:

- Handle the case simply as if no IUC information was received

  - Random resource selection may have some performance impact for the sidelink operation in general, and thus RAN2 should strive to ensure as efficient behaviour as possible. Although handling the case as if no IUC information was present provides a simple implementation in specification, it will still require specification change, and and better solution options are at the same time available.

- Specify in MAC that the non-preferred set should also be taken into account during random resource selection

  - Although suboptimal, allowing the random resource selection procedure to at least take into account the non-preferred set will have the benefit of possibly utilizing part or the entire IUC information received.

- Specify in MAC a procedure where IUC information is requested from PHY even before random reselection

  - In this case, it is certain that the newest and most relevant IUC information is available to the MAC, thus the best performance can be achieved. Although this procedure may provide the best result, the specification impact will be larger than any other change, and there will also be cross-WG impact.

Proposal 2: RAN2 to discuss and decide between potential solutions to the issue that the non-preferred set is not taken into account within random resource selection procedure:

Option 1: Handle the case simply as if no IUC information was received.

Option 2: Specify in MAC that the non-preferred set should be taken into account also during random resource selection.

Option 3: Specify in MAC a procedure in which the IUC information is requested from PHY even before random reselection.

# 3	Conclusion

Observation 1: RAN2 should interpret the wording in RAN1’s reply to Q1 in LS R2-2206314 as the scenario being valid.

Observation 2: As RAN1 agrees on the issue being valid but could not reach consensus on how to solve it, how to handle this should be decided in RAN2 as random resource selection is specified in MAC.

Proposal 1: RAN2 to discuss how to resolve the issue where UE-B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g. UE B performs mode 2 random resource selection, etc.

Proposal 2: RAN2 to discuss and decide between potential solutions to the issue that the non-preferred set is not taken into account within random resource selection procedure:

Option 1: Handle the case simply as if no IUC information is received.

Option 2: Specify in MAC that the non-preferred set should be taken into account also during random resource selection.

Option 3: Specify in MAC a procedure where IUC information is requested from PHY even before random reselection.