3GPP TSG-RAN WG2 Meeting #120	R2-2212404

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on U2U relay (re)selection

WID/SID:	NR_SL_relay_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#119bis-e U2U relay was discussed, leaving some open issues on especially the relay reselection. This Tdoc discusses these open issues among others and proposes new topics that RAN2 should discuss during the RAN2#120.

# 2	Discussion

## 2.1	U2U relay (re)selection

RAN2#119bis agreed that:

Agreements:

Proposal 1.1 (modified):		In UE-to-UE relay, the remote/relay UE in RRC_IDLE/RRC_INACTIVE or OOC can acquire discovery configuration as in Rel17 (i.e., cell-specific configuration/preconfiguration).  FFS if any restrictions specific to UE-to-UE relay are introduced for in-coverage UE in RRC_CONNECTED.

Proposal 2.1: 	Protocol stack for U2N Relay discovery is re-used for U2U Relay Discovery

Proposal 2.2: 	U2U Relay re-uses SL-SRB4 (with associated PDCP, RLC procedures and configuration) to carry discovery messages

Proposal 4.1:	Both shared and dedicated resource pool can be used for U2U discovery transmission and Rel-17 pool selection principle is re-used.

Proposal 5.1:	SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria.  FFS when each of the two quantities are used and whether to re-use the criteria in Rel17.

Proposal 7.1a:	Relay selection triggers include at least 1) Upper layer trigger; 2) PC5 signal strength conditions.  RAN2 further discuss details for trigger 2).

Proposal 7.1b (modified):	Relay reselection triggers include at least 1) Upper layer trigger; 2) PC5-RLF detection at the remote UE; 3) PC5-RLF indication received from the relay; 4) PC5 signal strength conditions; 5) PC5 link release message from relay to remote.  RAN2 further discuss details for trigger 4), potentially including T400 expiry.  FFS if some of the conditions could be indicated to upper layer instead of directly causing reselection.

In SL U2U relays, U2U Relay UE may need to discover a destination UE proactively or upon a U2U relay discovery solicitation from a source UE. The discovery of the destination UE by the U2U Relay UE may be based on either Model A or Model B. Thus, if the U2U Relay UE knows which of Model A or Model B the destination UE is using, the Relay UE may be able to perform the discovery of the destination UE in a fast and efficient way by applying the same discovery model the destination UE is using. Otherwise, the U2U Relay UE may need to perform the discovery in both Model A and Model B for minimizing latency of the discovery. The prior knowledge of the discovery model the destination UE may be provided to the U2U Relay UE by the source UE at least in case the source UE and the destination UE in need of U2U relay discovery and (re)selection are coordinated beforehand over an existing direct or indirect SL connection, corresponding to a direct-to-indirect path switch or an indirect-to-indirect path switch. For examples, the source UE may request the destination UE to use Model A or Model B or both or the destination UE may indicate to the source UE which of Model A or Model B it is using before the U2U relay discovery is initiated for the needed U2U relay (re)selection. Then the source UE may indicate the discovery model of the destination UE to U2U Relay UE candidates along with its U2U relay discovery solicitation for example.

Proposal 1: The source UE may indicate to U2U Relay UE candidates the discovery model (Model A or Model B) the destination UE is using along with its U2U relay discovery solicitation.

For U2U relay selection for establishment of an indirect path between the source UE and the destination UE, either the source UE or the destination UE may be allowed to make U2U relay selection decision. For an example, in case the source UE initiates the U2U relay selection using an E2E DCR integrated with a U2U discovery solicitation, as concluded in TR 23.700-33, the destination UE is allowed to make the U2U relay selection decision.  In another example, in case the U2U relay selection is for a direct-to-indirect path switch, the source UE may provide a list of U2U relay UE candidates to the destination UE over the existing direct path and let the destination UE to make the U2U relay selection decision. It is noted that in a bidirectional SL communication the roles of source and destination are interchangeable. The source UE may be allowed to make the U2U relay selection decision in general.

Proposal 2: Either the source UE or the destination UE may be allowed to make the U2U relay selection decision.

Proposal 3: One of the source UE or the destination UE may provide the other UE with a list of U2U Relay UE candidates for the other UE, either the source UE or the destination UE, to make the U2U relay selection decision for a direct-to-indirect path switch.

For U2U relay reselection for reestablishment of an indirect path between the source UE and the destination UE, either the source UE, the destination UE or the current U2U Relay UE may trigger a U2U relay reselection for an indirect-to-indirect path switch, as agreed in RAN2. However, for minimizing service interruption in case the current U2U Relay UE is not able to continue providing relay services to the source UE and the destination UE for some reason other than PC5 RLF, it is desirable that the current U2U Relay UE continues serving the source UE and the destination UE for a certain time period for the source UE and the destination UE to perform the U2U relay reselection. For this the PC5 release from the current U2U Relay UE to either one of the source UE or the destination UE, as agreed in RAN2, may indicate a time instant the current U2U Relay UE stops serving the source UE and the destination UE in term of PC5 slot or system frame number for examples.

Proposal 4: The trigger from the current U2U Relay UE for U2U relay reselection may indicate a condition on when the current U2U Relay UE stops serving the source UE and the destination UE.

For U2U relay reselection, either the source UE or the destination UE may be allowed to make the U2U relay selection decision, as the role of source and destination is interchangeable in a bidirectional communication between the source UE and the destination UE. The source UE and the destination UE may also coordinate for performing U2U relay discovery and reselection for an indirect-to-indirect path switch, similarly to that described above for U2U relay discovery and selection for a direct-to-indirect path switch. Hence, the following proposals are made, similar to Proposals 2 and 3 above.

Proposal 5: Either the source UE or the destination UE may be allowed to make the U2U relay reselection decision.

Proposal 6: One of the source UE or the destination UE may provide the other UE with a list of U2U Relay UE candidates for the other UE, either the source UE or the destination UE, to make the U2U relay reselection decision for an indirect-to-indirect path switch.

## 2.2	U2U relay path switches for service continuity

In addition to support for SL U2U relay reselection which can be seen as an indirect-to-indirect path switch, direct-to-indirect and indirect-to-direct path switches should also be considered for a service continuity support between the source UE and the destination UE, as for the remote UE in SL U2N relay.

For path switch from direct path to indirect path, U2U relay discovery and selection need to be triggered by either the source UE or destination UE while communicating over the direct path. The triggers at the source UE or at the destination UE may be based on monitoring and measurement of SCI or HARQ feedback over the direct SL connection between the source UE and the destination UE for examples. However, it is important to keep the latency due to U2U relay discovery and selection as low as possible for the path switch, especially when data traffic between the source UE and the destination UE is delay sensitive and having high priority.

For path switch from indirect path to direct path, triggers for the path switch need to be facilitated or, that is, the source UE and the destination UE need to be enabled to probe one another over SL for direct reachability while communicating over the indirect path. One option with minimized standardization impact is to configure the source UE and the destination UE to transmit some SL message such as SL discovery announcement to one another periodically while communicating over the indirect path.

Proposal 7: RAN2 should study support of direct-to-indirect, indirect-to-direct, and indirect-to-indirect path switches for service continuity between the source UE and the destination UE.

## 2.3	gNB involvement in U2U relay

Agreements:

RAN2 will strive to simplify the gNB involvement in U2U-relay-specific operation as compared to the U2N case.  Details are FFS, including whether some gNB control is needed for the in-coverage scenario and how/whether the gNB involvement can be simplified compared to U2N.

Rel17 SI assumptions on RRC state and coverage scenarios can be re-used.

Both L2 and L3 U2U relays may be considered as relay extensions for SL discovery and communications. In our view there may be no need for gNB involvement further than functions and services the gNB provides for SL discovery and communication.

Proposal 8: No further gNB involvement is needed for U2U relays compared to that provided for SL discovery and communication.

# 3	Conclusion

In this document we have provided the following observations and proposals:

Proposal 1: The source UE may indicate to U2U Relay UE candidates the discovery model (Model A or Model B) the destination UE is using along with its U2U relay discovery solicitation.

Proposal 2: Either the source UE or the destination UE may be allowed to make the U2U relay selection decision.

Proposal 3: One of the source UE or the destination UE may provide the other UE with a list of U2U Relay UE candidates for the other UE, either the source UE or the destination UE, to make the U2U relay selection decision for a direct-to-indirect path switch.

Proposal 4: The trigger from the current U2U Relay UE for U2U relay reselection may indicate a condition on when the current U2U Relay UE stops serving the source UE and the destination UE.

Proposal 5: Either the source UE or the destination UE may be allowed to make the U2U relay reselection decision.

Proposal 6: One of the source UE or the destination UE may provide the other UE with a list of U2U Relay UE candidates for the other UE, either the source UE or the destination UE, to make the U2U relay reselection decision for an indirect-to-indirect path switch.

Proposal 7: RAN2 should study support of direct-to-indirect, indirect-to-direct, and indirect-to-indirect path switches for service continuity between the source UE and the destination UE.

Proposal 8: No further gNB involvement is needed for U2U relays compared to that provided for SL discovery and communication.