3GPP TSG-RAN WG2 #120	R2-2212436

Toulouse, France, 14 – 18 November 2022

Agenda Item:	8.4.2.3

Source:	Ericsson

Title:	Discussion on the execution of LTM cell switch

Document for:	Discussion, Decision

# 1	Introduction

RAN2#119bis-e made these agreements:

RAN2 to use “LTM” as term for the L1/L2-triggered mobility. Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed. For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: - MAC/RLC reset (when configured) - RF retuning (e.g. needed for inter-frequency), baseband retuning

This contribution discusses further the LTM cell switch, and in particular, the following:

- Content of the LTM cell switch MAC CE

- Alternative signalling approaches for the execution of the switch

- Need of an uplink signal from the UE to acknowledge the execution

- Triggering of L2 reset (MAC reset, RLC reset and PDCP) during the switch

- Handling of SCells during the switch

# 2	Discussion

## 2.1	Content of the LTM cell switch MAC CE

RAN2#119bis-e agreed:

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index.

One important part of this LTM cell switch MAC CE is to understand what information it may contain. While it  is still a bit premature to have a full list of the info to be included, something that is likely to be needed are at least:

- Candidate configuration index (already agreed, a pointer to the RRC configuration for the candidate target cell);

- L2 reset flag(s) (e.g. for MAC reset, discussed in a separate section of this contribution);

- A beam indication, containing e.g.

  - RS identifier used for QCL indication (or indication of a TCI state) in order to point the UE to the right beams to be used;

  - A TA value in order to allow the UE to perform a RACH-less LTM cell switch;

  - BWP ID for UL and/or DL

In the RAN2 agreement above, it is still FFS whether DCI is used for the actual triggering.

Since a MAC CE anyway conveys the LTM cell switch trigger information, adding a DCI trigger would simply delay the execution. Looking at the contents of the LTM cell switch MAC CE above, we don’t see the commonalities with the ICBM approach, where a set of targets are preconfigured using MAC CE, and DCI selects one of them, is not viable since at least the TA is not known in advance. DCI also has a size limitation. The preconfigured information in case of LTM is

Hence, we propose

The LTM cell switch MAC CE is used for the actual triggering of switch.

## 2.2	Alternative signalling approaches

There are two main approaches for the procedure for LTM cell switch. Main differences are the relative timing between Uu and F1AP signalling.

Figure 1. Approach 1 for LTM cell switch procedure

In approach 1, (illustrated by Figure 1) the serving DU decides about the execution, transmits the LTM cell switch MAC CE to the UE and only thereafter informs candidate/target DU, via the CU, of the execution.

In this approach, the target cell and beam are determined by the serving DU. Also, the candidate/target DU becomes aware that the UE is coming and in which UL direction (beam) based on information from the serving DU.

In approach 1, the delay for triggering is very short. On the other hand, the decision about triggering is made entirely by the source DU and also the content of the LTM cell switch MAC CE is prepared by the source DU. This implies that the serving DU may only use information that is available at the time for configuration of the candidate target cells also during execution. Due to uncertainty network delays between the serving DU, the CU and the candidate/target DU (especially for the inter-DU case), it may be hard for the candidate/target DU to know exactly when UE can be scheduled. This is solved by an indication from the UE, illustrated by the “UL signal” in the signalling flow, that may be conveyed in L1 or MAC (FFS).

Figure 2. Approach 2 for LTM cell switch procedure

In approach 2, the serving DU first requests the candidate/target DU, via the CU, that execution of L1/L2-triggered mobility is required, and the candidate/target DU makes a final decision about target cell/beam including the TCI state, SSB index and/or any other information needed to quickly access the target cell (such as RACH resource for CFRA). Only thereafter the serving DU creates and transmits the LTM cell switch MAC CE to the UE.

In approach 2, there is inter-node signalling before the source DU transmits the LTM cell switch MAC CE, which adds some delay in the order of tens of milliseconds. On the other hand, at least parts of the content of the LTM cell switch MAC CE can be prepared by the target DU and target DU resources can be reserved late. Also, target DU and the CU may have possibility to reject the execution. The candidate/target DU may not know when the UE performs the switch since it is up to the serving DU when to transmit the LTM cell switch MAC CE to the UE. Also in this approach, the UL signal from the UE would be an indication for the candidate/target DU to start scheduling the UE.

We think that both these approaches have pros and cons. We also think that which approach to use could depend on network implementation. Actually, the specification may support both approaches, as long as the interfaces supports the information to be transferred for approach 2 during the configuration and execution phases. Whether we support approach 2 is mainly up to RAN3. In other words, if approach 2 for execution is supported, also approach 1 can be supported.

The interfaces could quite easily support multiple approaches for execution of the LTM cell switch. Which approach to use may be up to network implementation.

For the execution of LTM cell switch, the following approaches are supported:

  - Approach 1: Serving DU decides execution of LTM and send the LTM cell switch MAC CE to the UE.

  - Approach 2: Serving DU decides execution of LTM, but it contacts first the Candidate DU (via the CU) before sending the LTM cell switch MAC CE to the UE.

## 2.3	Need of an uplink signal from the UE to acknowledge the execution

On both the approaches for execution of L1/L2-triggered mobility cell switch above, we have indicated an uplink signal transmitted from the UE after having switched to the target cell configuration. The main purpose with this signal is to confirm that the UE has completed the procedure for execution of LTM cell switch and has started using the target cell configuration. It also enables the network to start the scheduling of uplink and downlink data as soon as possible (considering also somewhat unpredictable network delays) and serves as a trigger for the target DU to transmit e.g., a DL data delivery status indication to the CU.

In legacy L3 handover, the MSG1+MSG3 and RRCReconfigurationComplete serve the same purpose. It is not clear we can reuse the same solution for L1/L2-triggered mobility, because as discussed below, execution of L1/L2-triggered mobility without random access should also be supported and as the execution of L1/L2 inter-cell mobility is a lower layer signalling procedure, it would be awkward to have a L3 response. When the LTM cell switch is triggered by reception of a MAC CE, it is natural for the response to also be transmitted in MAC.

Even if the clearest solution would be to have an explicit indication by the UE that the execution of LTM cell switch had been successfully performed, yet another alternative is to also consider an implicit indication by the UE. However, as we have agreed to support RACH-less LTM cell switch, we can’t use MSG1 or MSG3 as implicit indication. Therefore, an implicit indication may be either the transmission of an SR request, or even the sending of UL data. But this relies on that there is some UL data that is sent immediately after the switch.

We have not agreed yet on the RRC model for the LTM candidate target cell configuration. If the RRC model is an RRCReconfiguration message, it argues for transmitting an RRCReconfigurationComplete in the target.

The need to specify an explicit UL signal (e.g. a MAC CE) to the target to confirm the execution of LTM cell switch depends mainly on the RRC model.

RAN2 to discuss what the UL signal to the target cell would be to indicate the successful execution of the LTM cell switch:

  - An existing or new MAC CE

  - An RRCReconfigurationComplete message

  - An implicit indication, such as a scheduling request

## 2.4	Triggering of L2 reset

RAN2 has agreed:

R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

Today, an RLC re-establishment in the UE is triggered via the RRC layer by including the reestablishRLC flag in the IE RLC-BearerConfig, part of the IE CellGroupConfig:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

. . .

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

. . .

Currently the field recoverPDCP, within the IE RadioBearerConfig, is used to trigger the UE to perform PDCP data recovery.

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),

sdap-Config                             SDAP-Config

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...,

The current RRC signalling to control RLC re-establishment and PDCP data recovery allows it to be controlled independently and on radio bearer / RLC bearer basis. This gives freedom for alternative network implementations when to trigger L2 reset and to avoid L2 reset when not needed.

However, by using RRC signalling to control L2 reset, it is not completely clear how to avoid RRC reconfiguration during subsequent LTM cell switch(es). In the example illustrated in Figure 3, when the UE is configured with LTM candidate target cell configuration(s) (e.g. cell A, cell B and Cell C) while connected to a first cell (e.g. Cell X), e.g., with these cells associated to different DU(s) as in the figure below. Then, the UE performs LTM cell switch to a candidate cell (e.g. to Cell A, in the same DU as Cell X), but still considers the previous LTM candidate target cells (e.g. Cell B and Cell C) as possible candidate cells. Assuming Cell X and Cell A are in the same DU, such a cell switch would not require a L2 reset.

Figure 3. Example of a scenario where L2 reset may be needed or not.

Now, let us assume that the UE in cell A moves to cell B, a L2 reset would be required (though it would also be required if that would have been from cell X, so the UE can still apply the LTM candidate target cell configuration it received in cell X). However, if the UE receives the LTM cell switch command (e.g. MAC CE) indicating a change to cell C (from cell B) and L2 reset, in principle, would not be required (since cell B and C are served by the same DU), though when the cell C was configured, a L2 reset would be required (as the UE was in cell X when cell C was configured as a candidate). This results in the UE performing an unnecessary MAC reset, which increases the LTM cell switch delay. Similarly, if the UE subsequently receives an LTM switch command indicating a switch back to cell A while being in cell C, the stored target cell configuration would not include the MAC reset (as it was not needed for the LTM switch from cell X to cell A), but in this case MAC reset would be needed since cell C and cell A are served by different DUs. This means that the LTM cell switch will fail. This leads to the current observation:

To fully support subsequent LTM cell change without RRC reconfiguration, network needs to select whether to perform L2 reset or not at the time for cell switch.

For selecting whether to performing L2 reset at the time of the cell switch there are three main alternatives:

- Alternative 1: Network indicates in the LTM cell switch command (e.g. MAC CE) whether to perform L2 reset or not. This gives network the freedom to control the L2 reset on the fly without RRC reconfiguration. However, this would require MAC to trigger RLC re-establishment and PDCP recovery. It will likely reduce the current flexibility to be able to trigger RLC re-establishment and PDCP recovery independently and per-bearer basis, unless this is indicated per RB basis also in MAC. The existing flexibility in RRC may be needed in certain network implementations. Without such flexibility, RLC re-establishment for a single RLC bearer would result in RLC-reestablishment for all bearers, for example.

- Alternative 2: The RRC configuration of the LTM candidate target cell includes the information about L2 reset (i.e. using existing or similar fields for at least RLC and PDCP and possibly a separate flag for MAC). After an LTM cell switch, if needed (e.g. after inter-DU cell switch), network reconfigures certain LTM candidate target cells to reflect the new situation. It may be acceptable to use an RRC reconfiguration after inter-DU cell switch if those cases are relatively rare and there is already a cost associated with such a switch due to a L2 reset.

- Alternative 3: As in alternative 2, the RRC configuration of the LTM candidate target cell includes the information about L2 reset. Network configures two versions of a configuration of the LTM candidate target cell, one with and one without L2 reset. During LTM cell switch, the network indicates the appropriate configuration to use. This means that UE would be required to store more candidate target configurations and also affects the value range for configuration index.

From specification point of view both alternatives 2 and 3 are very similar and they also reuse existing procedures to trigger RCL re-establishment and PDCP recovery. For the MAC reset, we note that reconfiguration With Sync triggers a MAC reset and this MAC reset is always a “full” reset. It is not clear whether reconfiguration With Sync can be combined with LTM cell switch even in those cases when MAC reset would be required. A full MAC reset implies that all ongoing MAC procedures are aborted and TA timers are stopped. As the LTM cell switch is triggered from MAC by reception of a MAC CE, the implication of triggering a MAC reset during the LTM cell switch seems contra-productive, for example if the MAC has established UL time alignment and started beam failure detection with the target cell before receiving the LTM cell switch command. Therefore we think that the MAC reset during LTM cell switch needs to be carefully specified together with the handling of the LTM cell switch MAC CE, i.e. within MAC. We think that therefore the triggering of (partial) MAC reset would be simpler to specify if it is indicated in the LTM cell switch MAC CE itself. This is somewhat similar to the partial MAC reset which is triggered from within MAC at SCG activation. We make the following observations:

The MAC reset triggered by Reconfiguration with sync is a “full MAC reset”.

There exists today “partial MAC reset” which is performed at SCG activation and this is triggered from within MAC.

If the LTM cell switch is triggered by a MAC CE, a “partial MAC reset” may be specified as part of the actions related to this MAC CE.

Returning to the alternatives for how to select whether to perform “L2 reset” discussed above, we suggest that:

- For selecting and triggering of RLC-reestablishment and PDCP recovery we use RRC (alternative 2 would be baseline, alternative 3 is enhancement).

- For selecting and triggering whether to perform MAC reset, we suggest using a flag in the MAC CE.

According to the suggestion above, we propose:

The UE performs MAC reset according to a MAC reset flag within the MAC CE with the LTM cell switch information.

During the LTM cell switch, UE performs RLC re-establishment according to the reestablishRLC flag (within RLC-BearerConfig IE) in the candidate target cell configuration.

During the LTM cell switch, UE performs PDCP data recovery according to an indication (similar to the recoverPDCP field in DRB-ToAddMod IE) within the LTM candidate target cell configuration.

## 2.5	Handling of SCells

SCells are configured by RRC in the LTM candidate target cell configuration to which the UE switches. At last RAN2#119-bis-e meeting, the following FFS was identified:

FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined).

L1/L2-triggered mobility targets reduction of mobility interruption time. However, some services (such as XR) which typically may rely on CA to achieve required throughput may become seriously affected if not CA can be maintained during mobility. This means that it would be desired to have SCells in the target candidate configuration activated already when the UE performs the cell switch.

The current specifications allow MAC CE triggered SCell activation (with and without TRS) and deactivation as well as RRC configuration of SCell state as activate/deactivated.

The source may be aware of which SCell(s) to activate in the target before the LTM cell switch, based on L1 or L3 measurement reports, but it requires measurements are configured for those frequencies. It would also require the network to update the candidate target configuration to reflect the desired states of the SCells, which increases signalling and due to the latency, the state may not be up to date when the LTM switch is performed.

We see the following main options for activation/deactivation of the SCells part of the LTM candidate target cell configuration during the LTM cell switch:

- UE applies the SCell activation state according to candidate target cell configuration in RRC during the switch. Then, the target may activate/deactivate the SCells using legacy mechanisms (including MAC CEs).  This is a baseline approach that will be supported in any case.

- An enhancement would be that the source indicates the SCell activation state when sending the LTM cell switch command to the UE. This avoids the delay by reconfiguration by RRC and allows the network to select more precisely which SCells to activate at any given time, in the same way it has when not during mobility.

For the enhanced solution, we see the following alternatives for how the source transmits the SCell activation state during the LTM cell switch:

- Alternative 1: Using the existing MAC CEs for SCell activation. As these would refer to the SCells after the switch, UE would need to process such a MAC CE after performing the switch. Typically, such a MAC CE would need to be concatenated in the same MAC PDU and after the MAC CE with the LTM cell switch information and the MAC specification need to ensure that the UE process the MAC CEs in the right order. The advantage with this alternative is that this allows that the features for LTM and SCell activation can be evolved independently. However, the requirements to support alternative 1 in MAC and RRC (including inter-layer modelling) needs to be carefully looked at during stage 3

- Alternative 2: A second approach is to include SCell activation information inside the MAC CE with the LTM cell switch information. The advantage with this alternative is that the SCell activation state would be more predictable especially if MAC would override the configuration of SCell state in the RRC candidate target cell configuration.

Even if we have a slight preference for alternative 1, it is worth noticing that in general the state of an SCell as a baseline shall be included first within an LTM candidate cell configuration. In this case, the MAC CE for SCell activation/deactivation can just be used to override what is already configured by RRC.

The initial state of an SCell for an LTM cell switch is part of the LTM candidate cell configuration.

Existing MAC CEs for SCell activation/deactivation are supported to change the state of an SCell upon the execution of an LTM cell switch (the MAC CE for SCell activation/deactivation is sent after the LTM cell switch MAC CE).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The interfaces could quite easily support multiple approaches for execution of the LTM cell switch. Which approach to use may be up to network implementation.

Observation 2	The need to specify an explicit UL signal (e.g. a MAC CE) to the target to confirm the execution of LTM cell switch depends mainly on the RRC model.

Observation 3	The current RRC signalling to control RLC re-establishment and PDCP data recovery allows it to be controlled independently and on radio bearer / RLC bearer basis. This gives freedom for alternative network implementations when to trigger L2 reset and to avoid L2 reset when not needed.

Observation 4	To fully support subsequent LTM cell change without RRC reconfiguration, network needs to select whether to perform L2 reset or not at the time for cell switch.

Observation 5	The MAC reset triggered by Reconfiguration with sync is a “full MAC reset”.

Observation 6	There exists today “partial MAC reset” which is performed at SCG activation and this is triggered from within MAC.

Observation 7	If the LTM cell switch is triggered by a MAC CE, a “partial MAC reset” may be specified as part of the actions related to this MAC CE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The LTM cell switch MAC CE is used for the actual triggering of switch.

Proposal 2	For the execution of LTM cell switch, the following approaches are supported:

a.	Approach 1: Serving DU decides execution of LTM and send the LTM cell switch MAC CE to the UE.

b.	Approach 2: Serving DU decides execution of LTM, but it contacts first the Candidate DU (via the CU) before sending the LTM cell switch MAC CE to the UE.

Proposal 3	RAN2 to discuss what the UL signal to the target cell would be to indicate the successful execution of the LTM cell switch:

a.	An existing or new MAC CE

b.	An RRCReconfigurationComplete message

c.	An implicit indication, such as a scheduling request

Proposal 4	The UE performs MAC reset according to a MAC reset flag within the MAC CE with the LTM cell switch information.

Proposal 5	During the LTM cell switch, UE performs RLC re-establishment according to the reestablishRLC flag (within RLC-BearerConfig IE) in the candidate target cell configuration.

Proposal 6	During the LTM cell switch, UE performs PDCP data recovery according to an indication (similar to the recoverPDCP field in DRB-ToAddMod IE) within the LTM candidate target cell configuration.

Proposal 7	The initial state of an SCell for an LTM cell switch is part of the LTM candidate cell configuration.

Proposal 8	Existing MAC CEs for SCell activation/deactivation are supported to change the state of an SCell upon the execution of an LTM cell switch (the MAC CE for SCell activation/deactivation is sent after the LTM cell switch MAC CE).

# 4	References