3GPP TSG-RAN WG2 #120	R2-2212438

Toulouse, France, 14 – 18 November 2022

Agenda Item:	8.4.2.2

Source:	Ericsson

Title:	Qualitative analysis on what to include in the RRC model for LTM

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2#119-bis-e meeting, it was discussed what RRC model is needed to characterize an LTM candidate cell configuration and the following agreements were reached:

A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered.

RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details.

- a.	Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration

- b.	Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

According to this, in this contribution a qualitative analysis on what may be needed in a LTM candidate configuration is provided, together with a possible RRC model example for LTM.

# 2	Discussion

In the last RAN2 meeting, the topic on what RRC model should be chosen for LTM took place and the current situation is that RAN2 basically will focus its efforts on two main RRC models:

- Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration.

- Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration.

Strictly speaking, before expressing a preference on whether Model 1 and Model 2 should be chosen for LTM, one needs first to understand what configuration are needed to cope with the requirements, use cases, and scenarios for LTM.

In this sense, let’s have a look on what is currently included in the RRCReconfiguration message (please note that only main configurations/IEs are considered in this analysis):

RRCReconfiguration message

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

< unnecessary fields skipped >

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

< unnecessary fields skipped >

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

< unnecessary fields skipped >

According to this, we will try to give an analysis on the main fields/configurations included in the RRCReconfiguration message and understand on whether a certain field/configuration may be needed or not.

### 2.1	Need for CellGroupConfig IE in LTM candidate cell configuration

As already discussed in RAN2, the CellGroupConfig IE is one of the fundamental configuration needs for LTM as it contains all the lower layer (but also cell) configurations generated by DU. Within discussing further on whether this IE is needed or not, we can safely state that this IE is a must for LTM:

RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration.

### 2.2	Need for RadioBearerConfig IE in LTM candidate cell configuration

About the RadioBearerConfig IE, first of all is good to clarify that this IE is generated by the gNB-CU entity, based also on information received by the gNB-DU. In general, this IE is used to setup radio bearer at the UE for both the standalone and non-standalone case.

In the context of LTM, the only scenario supported at the moment is an intra-CU scenario and thus the CU does have the general understanding on what is needed at the UE during LTM. In this sense, the basic case that should be supported at the UE is where only SRB1/2 is setup during the LTM cell switch procedure. This is a scenario that works both for CA and DC and is good enough to allow the UE to perform the LTM cell switch procedure without any particular problems.

The RadioBearerConfig IE is generated by the CU and in an LTM intra-CU scenario this IE does not need necessarily to be changed.

Of course, there are possible use cases on whether the RadioBearerConfig IE could be needed in the LTM candidate cell configuration. Some of them can be summarized as follows:

- An LTM candidate SCG wants to setup an SRB3

- An LTM candidate MCG may want to setup a split SRB

- An SN-terminated MCG bearer is changed to an MN-terminated MCG bearer (or vice versa)

Nevertheless, the support of these use cases is not critical for the correct execution of the LTM cell switch procedure by the UE if at least SRB1/2 is kept. The same can also be valid for the number of DRBs as this number does not need to be necessary be changed upon the execution of LTM.

In case the UE is configured with at least SRB1/2 in all the subsequent LTM cell switches this means that the RadioBearerConfig IE may not need to be changed.

It is not critical to change the/ number of DRBs at each LTM cell switch procedure.

Therefore, according to this analysis, one can conclude that, as baseline, the RadioBearerConfig IE does not need to be changed during the LTM cell switch procedure and thus there is no need to have it in the LTM candidate cell configuration. However, depending on how the RAN2 agrees to handle the SCG during the configuration and execution of LTM, one may want for the LTM candidate cell configuration to optionally include this field.

The RadioBearerConfig IE is not required in the LTM candidate cell configuration.

RAN2 to discuss on whether, for the case of DC, the RadioBearerConfig IE can be optionally supported in an LTM candidate configuration

### 2.3	Need for MeasConfig IE in LTM candidate cell configuration

Regarding the MeasConfig IE, also this configuration is entirely generated by the gNB-CU entity according to some information provided by one or multiple gNB-DU(s).

In the context of LTM, only the intra-CU scenario is supported and thus is only one gNB-CU that needs to generate, configure, and maintain the measurement configuration at the UE. In general, the gNB-CU has the overall understanding on what frequencies all the LTM candidate cell configurations have been configured and may also have an understanding on how the frequencies are distribution in the overall area in which LTM is going to operate. Therefore, the gNB-CU when generates the MeasConfig IE for the UE can already consider all the frequencies configured within an LTM candidate cell configuration. This basically means that the MeasConfig IE does not need necessarily to be changed upon the execution of the LTM cell switch procedure.

Since the gNB-CU does not change during the LTM cell switch, the MeasConfig IE may not need to be changed either.

The gNB-CU when generates the MeasConfig IE for the UE can already consider all the frequencies on which the LTM candidate cell configurations may operate.

Assuming that the MeasConfig IE does not change during the LTM cell switch procedure, one aspect to be considered is what are the UE actions after the execution of LTM. In this case, one straightforward action that the UE should perform is to consider the LTM target cell configuration indicated in the MAC CE for LTM as the new serving cell. At the same time, the “old” serving cell in which the UE was operating should now become a new LTM candidate cell configuration. However, this is an internal UE process that does not require a considerable standardization effort.

In case the MeasConfig IE does not change during the LTM cell switch procedure, the UE should consider the LTM target cell configuration indicated in the MAC CE for LTM as the new serving cell and the “old” serving cell in which the UE was operating as new LTM candidate cell configuration.

Therefore, based on this analysis, is clear that including a MeasConfig IE should not be required for the correct operation of an LTM cell switch. The gNB-CU has the capability to keep the MeasConfig IE always updated based on the latest LTM candidate cell configurations provided at the UE. Therefore, we propose:

The MeasConfig IE is not required to be part of the LTM candidate cell configuration.

RAN2 to discuss on whether, the MeasConfig IE can be optionally supported in an LTM candidate configuration.

### 2.3	Need for MRDC-SecondaryCellGroupConfig IE in LTM candidate cell configuration

In general, the MRDC-SecondaryCellGroupConfig IE is used to configure DC at the UE. So far, in the context of LTM current agreements related to DC just mention that the LTM cell switch procedure should happen independently at the MN and SN. Therefore, there could no need to have this IE, unless we want to enable the case on where, during an LTM cell switch procedure at the MCG, the network configures the UE to add also an SCG. However, this requires further discussion in RAN2.

As LTM cell switch is done independently on the MN and SN, the MRDC-SecondaryCellGroupConfig IE is not required to be part of the LTM candidate cell configuration.

RAN2 to further discuss on whether to allow any interaction between MN and SN for LTM.

### 2.4	Need for OtherConfig IE in LTM candidate cell configuration

The OtherConfig IE is used to configures at the UE other miscellaneous configurations that are not explicitly configured by a top level structure of the RRCReconfiguration message. However, these configurations are generally not critical for a system to work and thus there is no need to have this IE within an LTM candidate cell configuration. Thus we propose:

The OtherConfig IE is not required to be part of the LTM candidate cell configuration.

### 2.3	A possible ASN.1 structure for an LTM candidate cell configuration

In light on what has been described in the previous section, hereafter we try to formulate a possible ASN.1 structure for the RRC model to be used for an LTM candidate cell configuration. In particular, first we can decide that an LTM candidate cell configuration should be modelled at the UE as an ToAddMod/ToRelease structure, since an LTM configuration can be configured, released, but also modified. Having the LTM candidate cell configuration designed not as a full message brings also benefits in terms of message size but also UE processing. In fact, since the UE does not receive multiple RRCReconfiguration messages (e.g., like in CHO), this means that the ASN.1 decoding, and validity/compliance check should be done only once when the RRCReconfiguration message is received.

Having the LTM candidate cell configuration designed as a structure rather than a full message brings also benefits in terms of message size but also UE processing.

if the LTM candidate cell configuration does not comprise the all RRCReconfiguration message, this means that the ASN.1 decoding and validity/compliance check should be done only once when the RRCReconfiguration message is received

The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure.

For what concerns the actual content of the LTM candidate cell configuration, in RAN2 we already agreed that at least a configuration ID should be signalled in the MAC CE used for the execution of the LTM cell switch procedure. Therefore, each LTM candidate cell configuration should be identified by a unique identifier.

Further, based on the discussion we provided in section 2.1, 2.2, and 2.3, we can argue that the CellGroupConfig IE is mandatory needed, since each DU has to provide the lower layer configuration for the candidate cell in which LTM is going to be configured.

When it comes to the presence of the RadioBearerConfig IE and MeasConfig IE, as analysed this should not constitute a requirement for configuring an LTM candidate cell configuration and thus they not be needed. Therefore, we propose the following:

The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID.

Finally, we want to provide an example on what a possible ASN.1 structure for an LTM candidate cell configuration could be. In doing this, we want to clarify that this may not represent the final RRC model, but it would be used as baseline to be considered by RAN2.

RRCReconfiguration message

RRCReconfiguration-Ies ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                                 OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-Ies                                           OPTIONAL

}

RRCReconfiguration-vXXXX-Ies ::=              SEQUENCE {

ltm-CandidatesConfig-r18                SetupRelease {LTM-CandidatesConfig-r18}                                OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                            OPTIONAL

}

LTM-CandidatesConfig information element

LTM-CandidatesConfig-r18 ::=   SEQUENCE {

ltm-CandidateToRemoveList-r18         LTM-CandidateToRemoveList-r18                                            OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         LTM-CandidateToAddModList-r18                                            OPTIONAL,   -- Need N

...

}

LTM-CandidateToReleaseList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-CandidateId-r18                 OPTIONAL, -- Need N

LTM-CandidateToAddModList-r18 ::= SEQUENCE (SIZE (1..maxNrofCellsLTM-r18)) OF LTM-CandidateToAddMod-r18

LTM-CandidateToAddMod-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                   LTM-CandidateId-r18,

ltm-CellGroupConfig-r18               OCTET STRING (CONTAINING CellGroupConfig),

FFS on whether any interaction between MN and SN is sopporterd in LTM

ltm-SecondaryCellGroup-r18            LTM-SecondaryCellGroup-r18

FFS on whether RadioBearerConfig IE is part of an LTM Candidate cell configuration

ltm-RadioBearerConfig-r18             RadioBearerConfig                                                        OPTIONAL,

FFS on whether MeasConfig IE is part of an LTM Candidate cell configuration

ltm-MeasConfig-r18                    MeasConfig                                                               OPTIONAL,

...

}

LTM-SecondaryCellGroup-r18 ::=    SEQUENCE {

ltm-CellGroupConfigSCG-r18            OCTET STRING (CONTAINING CellGroupConfig)                                OPTIONAL,

ltm-RadioBearerConfigSCG-r18          OCTET STRING (RadioBearerConfig)                                         OPTIONAL,

ltm-MeasConfigSCG-r18                 OCTET STRING (MeasConfig)                                                OPTIONAL,

...

}

Figure 1. ASN.1 structure to be used as baseline for the configuration of an LTM candidate cell configuration.

According to the ASN.1 structure provided in Figure 1, we propose the following:

The ASN.1 structure in Figure 1 the baseline for an LTM candidate cell configuration.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The RadioBearerConfig IE is generated by the CU and in an LTM intra-CU scenario this IE does not need necessarily to be changed.

Observation 2	In case the UE is configured with at least SRB1/2 in all the subsequent LTM cell switches this means that the RadioBearerConfig IE may not need to be changed.

Observation 3	It is not critical to change the/ number of DRBs at each LTM cell switch procedure.

Observation 4	Since the gNB-CU does not change during the LTM cell switch, the MeasConfig IE may not need to be changed either.

Observation 5	The gNB-CU when generates the MeasConfig IE for the UE can already consider all the frequencies on which the LTM candidate cell configurations may operate.

Observation 6	In case the MeasConfig IE does not change during the LTM cell switch procedure, the UE should consider the LTM target cell configuration indicated in the MAC CE for LTM as the new serving cell and the “old” serving cell in which the UE was operating as new LTM candidate cell configuration.

Observation 7	Having the LTM candidate cell configuration designed as a structure rather than a full message brings also benefits in terms of message size but also UE processing.

Observation 8	if the LTM candidate cell configuration does not comprise the all RRCReconfiguration message, this means that the ASN.1 decoding and validity/compliance check should be done only once when the RRCReconfiguration message is received

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration.

Proposal 2	The RadioBearerConfig IE is not required in the LTM candidate cell configuration.

Proposal 3	RAN2 to discuss on whether, for the case of DC, the RadioBearerConfig IE can be optionally supported in an LTM candidate configuration

Proposal 4	The MeasConfig IE is not required to be part of the LTM candidate cell configuration.

Proposal 5	RAN2 to discuss on whether, the MeasConfig IE can be optionally supported in an LTM candidate configuration.

Proposal 6	As LTM cell switch is done independently on the MN and SN, the MRDC-SecondaryCellGroupConfig IE is not required to be part of the LTM candidate cell configuration.

Proposal 7	RAN2 to further discuss on whether to allow any interaction between MN and SN for LTM.

Proposal 8	The OtherConfig IE is not required to be part of the LTM candidate cell configuration.

Proposal 9	The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure.

Proposal 10	The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID.

Proposal 11	The ASN.1 structure in Figure 1 the baseline for an LTM candidate cell configuration.

# References

3GPP Chairman’s notes, RAN2#119-bis-e meeting, October 2022.