3GPP TSG-RAN WG2 Meeting #120	R2-2212440

Toulouse, France, November 14 - 18, 2022

Agenda item:	6.15.3

Source:	Samsung

Title:	Default CBR parameters

Document for:	Discussion and Decision

# 1	Introduction

Changes related to default CBR parameters in MAC and RRC were discussed at RAN2#119bis-e but not fully concluded. In this contribution, we would like to show our view on the issue.

# 2	Discussion

Changes related to default CBR parameters in MAC and RRC were discussed. It was not fully concluded because companies had different understanding for some case, i.e. case 3. Discussion is copied below.

[Session chair]: Check companies’ understanding (assuming R17 default CBR is configured)

- Case 1: partial sensing, R17 normal pool, R17 default CBR – partial

- Case 2a: random selection, R17 normal pool, R17 default CBR – random

- Case 2b: random selection, R16/17 exceptional pool, R16 default CBR

- Case 3: full sensing, R16/17 normal pool, R16 default CBR or invalid case?

- Case 1, 2a, 2b are confirmed. Case 3 will be revisited next meeting.

[OPPO]: For case 3, when full sensing result is available, CBR result is always available. When full sensing result is not available, anyway exceptional pool is used and R16 default CBR will be applied. [Ericsson, Qualcomm]: Sensing and CBR measurement are independent, so CBR measurement may not be available while sensing result is available. [Session chair]: What’s Rel-16 situation? Is default CBR applicable for normal resource pool also? [OPPO]: Due to common signalling design, default CBR can be also configured for normal pool, but we don’t have any corresponding UE procedure. [Vivo]: The UE requires CBR measurement in normal pool in the current specification, and it can be interpreted as the UE has always CBR measurement result. [Session chair]: Do need to send LS to RAN1? [OPPO, Ericsson, Intel, Vivo, Nokia]: It is enough each company to check with own RAN1 delegate. No LS is needed.

In our understanding (after internally checking with RAN1), with the consideration full sensing is continuous operation during the sensing window that is longer than the one for CBR measurement, we believe CBR measured result would be always available when full sensing result is available. Note if full sensing result is not available, the UE uses exceptional resource pool with random resource selection. Thus it is proposed default CBR parameters are not applied for R16/17 normal pool with full sensing.

[Proposal 1]: RAN2 is asked to confirm default CBR parameters are not applied for R16/17 normal pool with full sensing.

For case 1, 2a and 2b, next question is where to capture them. With the consideration MAC already specified when default CBR parameters are used in R16 (copied below), it would be natural to capture each case (i.e. when and which default CBR parameters are used) in MAC.

3>	select the number of HARQ retransmissions from the allowed numbers, if configured by RRC, in sl-MaxTxTransNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped in sl-MaxTxTransNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;

3>	select an amount of frequency resources within the range, if configured by RRC, between sl-MinSubChannelNumPSSCH and sl-MaxSubchannelNumPSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped between MinSubChannelNumPSSCH and MaxSubchannelNumPSSCH indicated in sl-CBR-PriorityTxConfigList for the highest priority of the logical channel(s) allowed on the carrier and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;

[Proposal 2]: RAN2 is asked to capture case 1, case 2a and case 2b into MAC.

For RRC, with the current field description we think R17 default CBR parameters for partial sensing case is clear enough, but R17 default CBR parameters for random selection is not clear since it is associated with random resource selection that is used for both normal pool and exceptional pool. We think it would be good to have clarification that R17 default CBR parameters for random selection is applied to normal pool and/or R16 default CBR parameters is applied to exceptional pool.

sl-DefaultCBR-PartialSensing

Indicates default value of SL CBR measurement for a UE that is configured to perform partial sensing by its higher layer (including when SL DRX is configured) if the number of SL RSSI measurement slots over CBR measurement window is below sl-MinNumRssiMeasurementSlots, (see TS 38.214 [19], clause 8.1.6). Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on.

sl-DefaultCBR-RandomSelection

Indicates default value of CBR measurement for a UE that performs random resource selection if no SL CBR measurement result over SL CBR measurement window, (see TS 38.214 [19], clause 8.1.6). Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on.

sl-DefaultTxConfigIndex

Indicates the PSSCH transmission parameters to be used by the UEs which do not have available CBR measurement results, by means of an index to the corresponding entry in sl-Tx-ConfigIndexList. Value 0 indicates the first entry in sl-Tx-ConfigIndexList. The field is ignored if the UE has available CBR measurement results.

[Proposal 3]: RAN2 is asked to clarify sl-DefaultCBR-RandomSelection is applied to normal pool with random resource selection and sl-DefaultTxConfigIndex is applied to exceptional pool in RRC.

# 3	Conclusion

We have seen the issue of default CBR parameters and made the following proposals.

[Proposal 1]: RAN2 is asked to confirm default CBR parameters are not applied for R16/17 normal pool with full sensing.

[Proposal 2]: RAN2 is asked to capture case 1, case 2a and case 2b into MAC.

[Proposal 3]: RAN2 is asked to clarify sl-DefaultCBR-RandomSelection is applied to normal pool with random resource selection and sl-DefaultTxConfigIndex is applied to exceptional pool in RRC.