3GPP TSG-RAN WG2 #120	Tdoc R2-2212460

Toulouse, France, 2022-11-14 - 2022-11-18

Agenda Item:	6.2.2

Source:	Ericsson

Title:	Measurements for conditional reconfigurations

Document for:	Discussion, Decision

# 1	Introduction

At the RAN2#119bis-e meeting there were some discussions about the measurements that a UE is required to do for conditional reconfigurations, which are not used by any conditional reconfiguration.

For some parts the discussions were left open to the RAN2#120 meeting:

4: Can continue to discuss in the November meeting how to solve the situation that, unlike Rel-17 UEs, Rel-16 UEs are required to perform conditional measurements regardless whether there is an associated conditional reconfiguration, and the Rel-17 network is not aware of this. Should provide inter-operability analysis and reasons why something could go wrong. RAN2 can consider minuting the expected behaviour in chair notes even if no CR is agreed.

In this paper we address these questions related to measurement configurations for conditional reconfigurations.

# 2	Discussion

In Rel-17, in case of SN-initiated inter-SN CPC, there may be cases where a UE is configured (at least for some time) with conditional measurements without any associated conditional configuration. This is due to that the target candidate SN does not accept all the CPC configurations that are proposed by the source SN and that the MN sends the CPC configurations to the UE before or without informing the source SN (to allow for an update of the configurations from the source SN). The UE may then end up with having a measurement configuration where some parts are not used. RAN2 has then agreed, and captured in 38.331, that the UE is not required to perform conditional measurements for which there is no associated conditional reconfiguration. This was captured as that the UE is only required to perform measurements for conditional measurement configurations with an associated conditional reconfiguration, in line with existing specification text.

At the RAN2#119bis-e meeting there were some discussions on how the requirement should be best captured, with some agreed changes. However, a question was raised whether something could go wrong due to that Rel-16 UEs (unlike Rel-17 UEs) would be required to perform conditional measurements in case of such a configuration. This question was left open until the RAN2#120 meeting:

4: Can continue to discuss in the November meeting how to solve the situation that, unlike Rel-17 UEs, Rel-16 UEs are required to perform conditional measurements regardless whether there is an associated conditional reconfiguration, and the Rel-17 network is not aware of this. Should provide inter-operability analysis and reasons why something could go wrong. RAN2 can consider minuting the expected behaviour in chair notes even if no CR is agreed.

Since it was only captured in rel-17 that a UE is not required to perform conditional measurements for which there is no associated conditional reconfiguration, a rel-16 UE would still consider itself to be required to perform those measurements if they would be configured. For a rel-16 UE the network should however be able to avoid such configurations, i.e. conditional measurement configurations without any associated conditional configuration. A rel-16 UE does e.g. not support the SN initiated inter-SN CPC and there is only one node involved in generating the conditional configuration in rel-16 (the SN in case of intra-SN CPC and the source MN in case of CHO).

In case a rel-16 UE would be configured with conditional measurements without any associated conditional reconfiguration, there would also not be any functional errors, but it would only result in that the UE can be considered to perform those measurements “in vain”.

There would not be any functional errors in case a Rel-16 UE would be configured with conditional measurements without any associated conditional reconfiguration. The only issue would be that the UE would perform those measurements in vain.

The network can avoid configuring a rel-16 UE with conditional measurements with no associated conditional reconfiguration.

RAN2 to capture in chair notes that a rel-17 the network can avoid configuring UEs supporting only CHO and/or rel-16 CPC with measurements not referred to by any execution condition.

RAN2 to capture in chair notes that no functional errors are foreseen in case a rel-16 UE would be configured with conditional measurements without any associated conditional reconfiguration.

# Conclusion

In the previous sections we made the following observations:

Observation 1	There would not be any functional errors in case a Rel-16 UE would be configured with conditional measurements without any associated conditional reconfiguration. The only issue would be that the UE would perform those measurements in vain.

Observation 2	The network can avoid configuring a rel-16 UE with conditional measurements with no associated conditional reconfiguration.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to capture in chair notes that a rel-17 the network can avoid configuring UEs supporting only CHO and/or rel-16 CPC with measurements not referred to by any execution condition.

Proposal 2	RAN2 to capture in chair notes that no functional errors are foreseen in case a rel-16 UE would be configured with conditional measurements without any associated conditional reconfiguration.