3GPP TSG-RAN WG2 #120	R2-2212465

Toulouse, France, 14-18 November 2022

Agenda Item:	8.14.2 QoE measurements in RRC_IDLE INACTIVE

Source:	Ericsson

Title:	QoE configuration and reporting for RRC_INACTIVE and RRC_IDLE states

Document for:	Discussion

# 1	Introduction

Multicast and Broadcast Service (MBS) was specified in LTE and NR, in [1], while Quality of Experience (QoE) was specified for Virtual Reality (VR) and Dynamic Adaptive Streaming over HTTP (DASH) in [2]. The WID [3] for Release 18, entitled “Enhancement on NR QoE management and optimizations for diverse services”, aims at introducing enhanced QoE support for diverse services, such as MBS. This contribution addresses the following objective defined in [3]: “Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]” and further discusses the issues raised in the RAN2#119 meeting.

# 2	Discussion

## 2.1	QoE configuration and measurement reporting in RRC_INACTIVE and RRC_IDLE states

In the RAN2#119 meeting it was agreed that gNB can send the broadcast MBS associated QoE configuration to a UE by an RRC message in RRC_CONNECTED state. It was left FFS on how a UE in RRC_INACTIVE or RRC_IDLE will receive QoE configurations for MBS in the respective RRC states. We think that it would be more efficient for a UE in RRC CONNECTED to receive a QoE configuration containing MBS-related indications pertaining the non-RRC CONNECTED states to enable application layer measurement collection in those states.

Consider including indications that application layer measurements are configured for the RRC_INACTIVE/IDLE states in a QoE configuration that is delivered to a UE while it is in an RRC CONNECTED state.

Another aspect to consider is the QoE measurement reporting when a UE is in the RRC_INACTIVE or RRC_IDLE states. Currently a UE needs to enter RRC_CONNECTED state to send a QoE measurement report to a RAN node (via RRC message via SRB4). However, transferring a UE to an RRC_CONNECTED state only to transfer the measurement reports is not optimal. Therefore, if a UE is in other than RRC_CONNECTED state, it should not be allowed to enter the RRC_CONNECTED state only to transmit QoE measurement reports.

The UE should not request to be transferred to RRC_CONNECTED just for the reason to transmit a QoE report.

For QoE measurements in RRC_CONNECTED, the area scope is not sent to the UE. However, for measurements in RRC_IDLE and RRC_INACTIVE it is necessary to send the area scope to the UE as the network may not be aware of the mobility of the UE in these states. The area scope can be sent to the UE as part of the RRCReconfiguration message. A question is whether the area scope should be forwarded to the application layer or not. As it is the UE AS layer that knows in which cell the UE in, it is most natural that the UE AS layer that takes care of that the measurements are performed in the right cells. The UE AS layer doesn’t know when the sessions in the application layer are ongoing, but the session start/stop indication can be used to inform the UE AS layer when there are ongoing sessions. That means that also the SA4 requirement to not interrupt ongoing measurements can be fulfilled without any additional impact. Therefore, it is proposed that the UE AS layer handles the area scope when the UE is in RRC_IDLE and RRC_INACTIVE.

The area scope is sent to the UE in RRCReconfiguration for QoE easurements in RRC_IDLE and RRC_INACTIVE.

UE application layer may indicate session start/stop to UE AS while in RRC_INACTIVE or RRC_IDLE state.

The UE AS layer handles the area scope when the UE is in RRC_IDLE and RRC_INACTIVE.

To enable application layer measurement reporting after the UE returns from RRC_IDLE state to the correct MCE, the MCE IP address should be retained and stored after the UE enters either of non-RRC CONNECTED states, and should be provided to a target gNB. The UE itself does not need to know the MCE IP address. Another option is to store the MCE IP address in the CN, similarly to how the UE Context is stored there and can be retrieved and verified by the RAN.

Consider storing the MCE IP address in the core network, similarly as the UE Context can be stored in the core network, so that a UE returning from an RRC_IDLE, could send the application layer measurements to the right MCE.

The network needs to know the UE configuration for QoE, such as the QoE reference, measurement configuration ID, whether the UE was configured to send session start/stop indication. The QoE measurement report contains the identity of the application layer measurement in an IE measConfigAppLayerId.

Store UE configuration for QoE in the core network to enable a specific UE to continue the application layer measurements according to the saved QoE

# Conclusion

Based on the previous discussions and observations, we propose to consider the following proposals:

Proposal 1	Consider including indications that application layer measurements are configured for the RRC_INACTIVE/IDLE states in a QoE configuration that is delivered to a UE while it is in an RRC CONNECTED state.

Proposal 2	The UE should not request to be transferred to RRC_CONNECTED just for the reason to transmit a QoE report.

Proposal 3	The area scope is sent to the UE in RRCReconfiguration for QoE easurements in RRC_IDLE and RRC_INACTIVE.

Proposal 4	UE application layer may indicate session start/stop to UE AS while in RRC_INACTIVE or RRC_IDLE state.

Proposal 5	The UE AS layer handles the area scope when the UE is in RRC_IDLE and RRC_INACTIVE.

Proposal 6	Consider storing the MCE IP address in the core network, similarly as the UE Context can be stored in the core network, so that a UE returning from an RRC_IDLE, could send the application layer measurements to the right MCE.

Proposal 7	Store UE configuration for QoE in the core network to enable a specific UE to continue the application layer measurements according to the saved QoE

# References

3GPP TS 23.247, Architectural enhancements for 5G multicast-broadcast services, v17.3.0, 2022-06

3GPP TS 38.300, NR and NG-RAN Overall Description, v17.0.0, 2022-03

3GPP WID RP-221803, Enhancement on NR QoE management and optimizations for diverse services, 2022-06

3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v17.0.0, 2022-03