3GPP TSG-RAN WG2 #120	R2-2212494

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.16.2

Source:	Ericsson

Title:	General aspects for AI/ML for NR air interface

Document for:	Discussion, Decision

# 1	Introduction

This is the second meeting on which RAN2 discusses the “AI/ML for NR Air Interface” SI (see the approved SID in RP-213599, and the revised version in RP-221348).

During RAN2#119-bis-e, the following assumption was made concerning RAN2’s involvement:

Some initial Assumptions on the work: - Assume that RAN2’s work can be somewhat split: A) use-case-centric configuration, signalling and control procedures, B) management of data and AI/ML models (where part of discussion may overlap between use cases). - Assume that e.g. for the management of data and AI/ML models, RAN2 could start by focusing on data collection, model transfer, model update, model monitoring and model selection/(de)activation/switching/fallback (to the extent needed), whether UE capabilities has a role in this. - Chair assumes that we will input on various aspects when the time is right, and e.g. postpone things that obviously need R1 decisions, but there could be some rare exception.

In this paper we focus on “B) management of data and AI/ML models”, while also providing additional aspects that should be covered by this Agenda Item (AI 8.16.2). 
Additionally, we develop the main aspects that RAN2 agreed to start working on, i.e.:

- model transfer

  - NW-UE collaboration levels

  - model update

- data collection

- model monitoring

- model selection/(de)activation/switching/fallback.

# 2	Discussion

## 2.1	Functionality mapping and degrees of commonality between uses cases

During RAN1#109-e the following was captured in the Session Notes (see R1-2205695):

Observation Where AI/ML functionality resides depends on specific use cases and sub-use cases. Conclusion - RAN1 discussion should focus on network-UE interaction. - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Therefore, we believe that RAN2 should start by agreeing what RAN1 has indicated above. On this, since AIML-based models are to be considered for both NW and UE sides, we note a necessity to also consider the UE as a NW entity.

RAN2 to study the AI/ML functionality mapping within network entities (UE included).

Furthermore, and regarding RAN1’s observation and conclusion, we understand that use cases could lead to distinct evaluation mechanisms, procedures, etc. And thus, to distinct functionality mapping within the network for each of those.

However, RAN2 should also be aware that the development of the SI could lead to finding degrees of commonality between the use cases.

In this regard, last meeting, some companies showed concerns about trying to define a common framework for all use cases. On this, we would like to stress that our thinking is not to start from that basis. But that the exercise of identifying similarities between use cases may lead us to find common sets of functionalities within network entities.
Such approach could simplify the outcome of this SI and allow for more straightforward WI-discussions.

Having common sets of functionalities within the NW (for the different use cases) could simplify the outcome of the SI and facilitate the WI phase.

Thus, we propose to use this AI as an “umbrella” item gathering use-cases input. This, for the sake of studying degrees of commonality between them and to (whenever possible) develop common sets of functionalities allocated to the different network entities.

Under this Agenda Item (AI 8.16.2), RAN2 should strive to find a degree of commonality between the use cases, to further analyse the feasibility of having common sets of functionalities allocated to network entities.

## 2.2	Model transfer (/delivery/update)

Before continuing with ‘model transfer’ or ‘NW-UE collaboration’ details, we would like to touch upon three points.

First, that seemingly RAN2 is indistinguishably using the terms ‘model transfer’ and ‘model delivery’ for the same purpose. On this, and even when RAN1 has defined these two terms separately, we observe that until now no ambiguity has arisen from RAN2 discussion. Hence, there seem to be no clear motivation (for now) to stress on the difference between such terms.

Unless real ambiguity is created, we see no issue with RAN2 interchangeably using the terms ‘model transfer’ and ‘model delivery’.

Secondly, we would like to come back to the agreements reached during RAN2#119-bis-e:

- R2 assumes that for the existing (under discussion) AI/ML use cases, proprietary models may be supported and/or open format may be supported (and maybe RAN2 doesn’t have to further elaborate on this assumption). - R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. - R2 assumes that a model is identified by a model ID. Its usage is FFS. - General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

Specifically, we would like to address the highlighted text (above). On this, RAN2 should spend some time discussing on whether there is really a need to capture anything on proprietary model support.

We believe that RAN2 should focus on those scenarios for which there is specification impact, i.e., there is no need to capture anything on proprietary solutions. Therefore, all implementation-based solutions should simply be left out of the scope of this SI (not only in RAN2, but in all concerning WGs).

RAN2 to focus on aspects that have specification impact, i.e., we should avoid capturing aspects concerning proprietary solutions/models. Implementation specific approaches are outside the scope of this SI.

Thirdly, it is our understanding that models are to be updated, e.g., according to Life Cycle Management (LCM) procedures or, after model monitoring related triggers are activated. However, as it seems premature to get into such discussions, and since we understand that a model is updated by (re)transferring a new “version” of it. For the time being, we propose to treat ‘model update’ as part of ‘model transfer’.

For the time being, treat ‘model update’ within ‘model transfer’.

### 2.2.1	NW-UE collaboration

During RAN1#109-e it was agreed to have the following 3 levels of network-UE collaboration:

Take the following network-UE collaboration levels as one aspect for defining collaboration levels 1. Level x: No collaboration 2. Level y: Signaling-based collaboration without model transfer 3. Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

RAN2 spent some time discussing this during the previous WG meeting (i.e., RAN2#119-bis-e). However, only the following Observation was captured in the Chair’s notes:

Observation: the collaboration levels definitions doesn’t really clarify what is required, more work is needed

RAN1 further discuss this during their last WG meeting (i.e., RAN1#110bis-e) and the following was captured in the session notes (see R1-2210690):

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery.

It should then be possible to describe Level ‘y’ as the one on which NW and UE are (somehow) aware that AIML-based procedures/models are available or being used for NR Air Interface related purposes. For this case, and depending on the scenario, the AIML model could be implemented in the NW, UE, or at both. Hence, signalling is used to e.g., indicate such information, to proceed with UE configuration, etc.

We though observe that Level ‘y’ does not exclude the case on which a model is being transferred over-the-top between NW and UE.

Level ‘y’ describes the scenario on which NW and UE are aware that AIML is being used for NR Air Interface purposes.

For Level ‘y’, signalling is used to allow for NW and UE to collaboration, e.g., to indicate capabilities/support of features, for UE configuration purposes, etc.

Level ‘y’ does not exclude the scenario on which a model is transferred. However, if this is the case, the model is transferred over-the-top.

For Level ‘y’, it is within RAN2 scope to study how the signalling-collaboration is achieved.

Furthermore, and as opposed to Level ‘y’, Level ‘z’ includes the case on which the model being transferred is carried by 3GPP-based signalling.

For Level ‘z’ the model is transferred using 3GPP signalling.

Additionally, during the previous RAN1 meeting (i.e., RAN1#110bis-e) the following was agreed:

Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Thus, NW-UE collaboration Level ‘x’ should not have any specification impact.

Based on the above, we propose the following.

RAN2 to focus on analysing impact for when there is NW-UE signalling-based collaboration, with and without AI/ML model transfer.

RAN2 to study the feasibility and necessary aspects (and mechanisms) to support the transfer of AI/ML models.

Regarding this last proposal, and as already depicted above, the following FFS was captured in the meeting notes:

General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

On this, RAN2 could initially focus on analysing potential AI/ML model formats, the container in which the AI/ML model is packaged and transferred, time duration expected to perform model transfers, UE indications of supported/stored models, etc.

However, further RAN1 input for each use case is needed for RAN2 to work on these aspects.

For model transfer, RAN2 could focus on studying the container in which the AI/ML model is transferred, time duration expected to perform model transfers, UE indications of supported/stored models.

Further RAN1 input is needed (e.g., requirements) for RAN2 to start working on model transfer aspects.

## 2.3	Data collection framework

The network is responsible for the overall operation of mobile communications. For this reason, we believe that it is of paramount importance to have AI/ML models at the network side. As such, the network should be in control of collecting data, as it is done already for existing procedures. This allows the network to efficiently configure UEs and offer optimal link performance.

As concluded by RAN1 in RAN1#110bis-e:

Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

RAN2-related aspects could be linked to configuration of the data collection processes, e.g., measurement and UE reporting configurations, triggers for a UE to start performing measurements and data logging, triggers for a UE to report collected/logged data to the NW, data collection duration, UE capability signalling aspects, etc.

RAN2 could potentially start by gathering and analysing existing procedures, while trying to reuse or mimic (as much as possible) those that could fit the purposes of this SI. In this sense, RAN2 could try to build upon a general framework capable of addressing the different use cases (e.g., at this point this seems especially to be the case for CSI and beam management).

Further ‘data collection’ details are discussed in our use-case-specific contribution in Agenda Item 8.16.3.

RAN2 should investigate the feasibility of building a general data collection framework capable of covering as many use cases as possible, to support UE performing data logging/collection and reporting the collected data to NW.

Consider RAN1 inputs to analyse data collection requirements for different Life Cycle Management (LCM) stages of the different use cases.  From this, recommend a data collection solution for NW-sided model training/monitoring.

## 2.4	Model monitoring aspects

Model monitoring can be used to analyse different aspects of the AI/ML-based systems. In this regard, RAN1 has agreed during RAN1#110bis the following:

Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Hence, we think this should be used as a guideline for RAN2.

RAN2 should study AI/ML model monitoring for at least the following purposes: model (de)activation, selection, switching, fallback, and update.

Aspects concerning RAN2 could firstly be link to the configuration of the monitoring approach (e.g., measurements configuration/reporting). However, we also observe that RAN2 needs to take into consideration the data that is needed to allow for an effective monitoring of the models, etc. On this matter, model monitoring and data collection aspects overlap between each other.

Model monitoring and data collection are tightly related.

As proposed above for data collection, we also believe that RAN2 should start by trying to consider existing procedures that could fit towards the goal of monitoring an AI/ML model.

Analyse existing RAN2-based mechanisms that could serve towards monitoring the performance of an AI/ML model (e.g., configuration of measurements and reports). Investigate to what extent it is possible to draw inspiration from these to create a framework for monitoring AI/ML models.

## 2.5	Model selection/(de)activation/switching/fallback

As early depicted, during RAN2#119-bis-e the following was agreed:

R2 assumes that a model is identified by a model ID. Its usage is FFS.

This represents an early foundation for model selection/(de)activation/switching. However, the aspects have a strong relationship to the use case discussion progress.
In this sense, RAN2 needs further progress in RAN1 to proceed with detailed discussions. On this matter, we observe that during RAN1#110bis-e, RAN1 also agreed to further study the following aspects:

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

So, for the time being it seems reasonable to postpone discussion on this matter, since RAN1 has already agree to further study mechanisms to address model selection/(de)activation/switching/fallback.

Early discussion on model selection, (de)activation, switching and fallback could eventually be covered in Agenda Item 8.16.3.

Wait for RAN1 inputs on the requirements and mechanism for model selection, (de)activation, switching, and fallback.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Having common sets of functionalities within the NW (for the different use cases) could simplify the outcome of the SI and facilitate the WI phase.

Observation 2	Unless real ambiguity is created, we see no issue with RAN2 interchangeably using the terms ‘model transfer’ and ‘model delivery’.

Observation 3	Level ‘y’ describes the scenario on which NW and UE are aware that AIML is being used for NR Air Interface purposes.

Observation 4	For Level ‘y’, signalling is used to allow for NW and UE to collaboration, e.g., to indicate capabilities/support of features, for UE configuration purposes, etc.

Observation 5	Level ‘y’ does not exclude the scenario on which a model is transferred. However, if this is the case, the model is transferred over-the-top.

Observation 6	For Level ‘y’, it is within RAN2 scope to study how the signalling-collaboration is achieved.

Observation 7	For Level ‘z’ the model is transferred using 3GPP signalling.

Observation 8	For model transfer, RAN2 could focus on studying the container in which the AI/ML model is transferred, time duration expected to perform model transfers, UE indications of supported/stored models.

Observation 9	Further RAN1 input is needed (e.g., requirements) for RAN2 to start working on model transfer aspects.

Observation 10	RAN2 should study AI/ML model monitoring for at least the following purposes: model (de)activation, selection, switching, fallback, and update.

Observation 11	Model monitoring and data collection are tightly related.

Observation 12	Early discussion on model selection, (de)activation, switching and fallback could eventually be covered in Agenda Item 8.16.3.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to study the AI/ML functionality mapping within network entities (UE included).

Proposal 2	Under this Agenda Item (AI 8.16.2), RAN2 should strive to find a degree of commonality between the use cases, to further analyse the feasibility of having common sets of functionalities allocated to network entities.

Proposal 3	RAN2 to focus on aspects that have specification impact, i.e., we should avoid capturing aspects concerning proprietary solutions/models. Implementation specific approaches are outside the scope of this SI.

Proposal 4	For the time being, treat ‘model update’ within ‘model transfer’.

Proposal 5	RAN2 to focus on analysing impact for when there is NW-UE signalling-based collaboration, with and without AI/ML model transfer.

Proposal 6	RAN2 to study the feasibility and necessary aspects (and mechanisms) to support the transfer of AI/ML models.

Proposal 7	RAN2 should investigate the feasibility of building a general data collection framework capable of covering as many use cases as possible, to support UE performing data logging/collection and reporting the collected data to NW.

Proposal 8	Consider RAN1 inputs to analyse data collection requirements for different Life Cycle Management (LCM) stages of the different use cases.  From this, recommend a data collection solution for NW-sided model training/monitoring.

Proposal 9	Analyse existing RAN2-based mechanisms that could serve towards monitoring the performance of an AI/ML model (e.g., configuration of measurements and reports). Investigate to what extent it is possible to draw inspiration from these to create a framework for monitoring AI/ML models.

Proposal 10	Wait for RAN1 inputs on the requirements and mechanism for model selection, (de)activation, switching, and fallback.

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

R1-2205695, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)	“, Ad-hoc Chair (CMCC), RAN1#109-e, May 2022

R1-2208145, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)”, Ad-hoc Chair (CMCC), RAN1#110, Toulouse, August 2022

R2-2210677, “RAN2 Work Plan for Rel-18 SI on AI/ML for NR air interface”, Ericsson, Qualcomm Inc., RAN2#119-e, Oct. 2022

R1-2210690, “Session notes for 9.2 (Study on AI/ ML for NR air interface)”, CMCC, RAN1#110bis-e, Oct. 2022