3GPP TSG-RAN WG2 #120 	R2-2212497

Toulouse France, 14th –18th November 2022

Agenda Item:	8.1.3

Source:	NEC

Title:	Down-selection of NCR management solutions

Document for:	Discussion, Decision

Introduction

The Rel-18 study item on NR network-controlled repeaters (NCR) has been finished and the report was summarized in TR38.867[1]. Consequently, a WI was agreed in RP-222673 at RAN#97e meeting. It includes following objective:

Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability.

This contribution provides our views on down-selection of solutions in section 8 of TR 38.867 taking SA3 feedback in [2] into account.

Discussion

4 solutions on repeater management are captured in section 8 of TR 38.867. Since there are some concerns on Solution 1 optional NCR validation procedure and Solution 2, RAN3 sent an LS [3] to SA3 and SA5 (cc to RAN2 and SA2) while SA3 summarized their reply as followings [2],

Q1a: Is there any security issue for solution 2 which does not provide Uu security, non-protected NCR indication info and the OAM container in Step 5? Reply: Yes. Due to the lack of Uu security, this solution exposes the OAM indirectly to attacks over the air interface. Q1b: Does SA3 believe that the NCR needs to be securely validated? Any security issue for configuring locally stored information in the gNB in Solution 1? Reply: SA3 cannot provide answers before the security validation related steps in solution1 are clarified. For example, if this refers to an additional authentication procedure, typically RAN does not perform such procedure. If this is not the authentication procedure, it is unclear the purpose and the feasibility to have such additional steps as well as what kind of information is stored in RAN.

SA3 clearly stated that there are security issues for solution 2.

SA3 could not provide answer for solution 1 since SA3 thinks clarification of the validation steps in solution 1 is needed.

Taking the above SA3 answers into account, as well as already discussed that the solution 2 does not compliance with WID that requires multi-vendor interoperability, our first proposal is

RAN2 to agree that Solution 2 is down-selected firstly.

Regarding the validation related steps in solution 1, our understanding is that it is for further check the validity of NCR, i.e., it refers to an additional authentication procedure. SA3 LS reply clearly stated that typically RAN does not perform authentication procedure. Considering the validation steps are optionally performed based on the operator's requirement (actually whether such operator requirements exist or not is not clear), our proposal is

RAN2 not to work further on validation steps in solution 1 during normative phase.

- The simplified call flows of solution 1 (without validation steps), solution 3 and solution 4 are shown in Fig.1.

- Fig.1 The simplified call flows of solution 1, 3 and 4

- From RAN2 perspective, it is feasible to design a common AS procedure for solution 1 and 3.

From RAN2 perspective, it is feasible to design a common AS procedure for solution 1 and 3.

RAN2 to agree to support both solution 1 and 3 by designing a common AS procedure.

- Another observation from Fig.1 is that solution 4 does not have AS impact, therefore our proposal is

Whether down-select solution 4 or not is up to RAN3 decision.

Summary

This contribution provides our views on down-selection of solutions in section 8 of TR 38.867 taking SA3 feedback in [2] into account.

- Our observations are

- Observation 1 SA3 clearly stated that there are security issues for solution 2.

- Observation 2 SA3 could not provide answer for solution 1 since SA3 thinks clarification of the validation steps in solution 1 is needed.

- Observation 3 From RAN2 perspective, it is feasible to design a common AS procedure for solution 1 and 3.

- Our proposals are

- Proposal 1 RAN2 to agree that Solution 2 is down-selected firstly.

- Proposal 2 RAN2 not to work further on validation steps in solution 1 during normative phase.

- Proposal 3 RAN2 to agree to support both solution 1 and 3 by designing a common AS procedure.

- Proposal 4 Whether down-select solution 4 or not is up to RAN3 decision.

- References

[1] TR38.867

[2] S3-222519 Reply LS on NCR Solution

[3] R3-225253 LS on NCR Solutions