3GPP TSG-RAN WG2 Meeting #120		R2-2212502

Toulouse, France, November, 2022

Agenda Item:	8.4.3

Source: 	LG Electronics

Title:  	Selective Cell Group Activation

Document for:	Discussion

Introduction

For Rel-18 Further NR mobility enhancements, RAN2 made the following agreements in RAN2#119bis-e [1]:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs): - a. Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. - b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells. - c. Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell. Confirm that “CPA” selective activation of cell groups will be supported for this WI objective Confirm that we aim to support delta configuration, i.e. that there need to be a known reference. RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In this contribution, we discuss some issue points of the selective activation of the cell group based on the agreements and FFS.

Discussion

Candidates Management

At the RAN2#119bis-e meeting, RAN2 remained FFS if the UE should keep all CPA/CPC configurations for subsequent mobility after mobility or if those are indicated by the network to keep.

We think that the UE should be able to have the most flexible choice for the subsequent mobility at least as much as the legacy mobility scenario. Considering the real cell deployment, if subsequent CPA/CPC are only allowed only when all CPA/CPC configuration shall be maintained, the scenarios in which the UE can use the configuration for the subsequent CPA/CPC are very limited. To use the configuration for the subsequent CPA/CPC, all candidate cells are connected to the same MN, and intra-SN mobility scenario is only allowed considering that mobility execution is possible before the measurement update. If subsequent mobility can be available only in this limited scenario, there will be little gain compared to the legacy conditional mobility.

Therefore, subsequent CPA/CPC information regardless of performing mobility is proposed to be maintained unless the network explicitly indicates to remove it.

Proposal 1.	The UE maintains the list of candidates for the subsequent CPA/CPC after mobility is complete unless the network indicates to remove.

However, whenever performing mobility, the UE must know which candidate cell for subsequent mobility is available in a new target cell.

From the network point of view, if some candidate cells can be used frequently in the subsequent CPA/CPC scenarios, the candidate cells would be better to keep even though those are not available in the current SCG. Otherwise, because the network is likely to re-initiate CPA/CPC preparation for those cells after leaving the current SCG, signalling overhead and configuration latency may occur.

Thus, we think it is beneficial that those candidate cells which are not available in the current SCG can be kept for the next subsequent CPA/CPC. For this, the network may also indicate to deactivation of those candidate cells while staying at the current SCG. Then, if the candidate cells are indicated as deactivated by the network, the UE doesn’t evaluate the mobility execution condition for the candidate cells.

Proposal 2.	The UE maintains the list of candidates for the subsequent CPA/CPC as deactivated if the network indicates the list is inapplicable in a new target cell, i.e. the UE doesn’t evaluate the execution condition for the list while staying in the new target cell.

One of the other FFS is how much subsequent conditional mobilities are targeted for the subsequent CPA/CPC.

We think that the UE does not need to know how much the subsequent conditional mobility is to perform. This is because the network only knows resource validation through inter-node signalling whether the UE is maintaining information for the subsequent CPA/CPC, it is easy that the network can release the configuration for the subsequent conditional mobility when any candidate is decided to remove.

Proposal 3.	The network doesn’t provide information for the UE to indicate how many subsequent mobilities can be targeted, i.e. the network release the conditional reconfiguration for the subsequent CPA/CPC if not valid or not needed as the legacy.

Coexistence b/w R18 CPAC and R16/R17 CPAC

We consider the co-existence cases between the legacy conditional reconfiguration and the subsequent conditional mobility. Considering the legacy conditional reconfiguration may be provided together, indicating to distinguish whether the mobility information is to be maintained or not is another point to be discussed.

In our view, it is beneficial for the UE that the network provides candidates as many as possible to support flexibility of selection. If the scenario for R18 CPA / CPC is limited, e.g. within the same MN or same SN only, the legacy CPA / CPC must be configured together for the UE to have enhanced mobility performance.

Moreover, looking at the agreement for the baseline procedure made at the RAN2#119bis-e meeting, RAN2 already allows candidate cells for subsequent CPC to be provided to the UE before PSCell addition, but just there is no agreement for the support of the coexistence b/w R18 CPAC and R16 /R17.

Thus, we propose to provide the list of candidate CGs for subsequent CPA/CPC and the legacy conditional reconfiguration together.

Proposal 4.	The network provides the list of candidates for the subsequent CPA/CPC and the legacy conditional reconfiguration together.

Delta Configuration

In RAN2#119bis-e meeting, RAN2 has agreed to support delta configuration but how to design is still FFS.

Since RAN2 has discussed for a short while whether the nested configuration (i.e. recursive configuration such as a conditional reconfiguration in another conditional reconfiguration) is supported for the delta configuration, we consider if there are other approaches can be one of the options and what is pros and cons of each options for the delta configuration handling.

The progress of the LTM (L1/L2 mobility) discussion in [1] can be considered together even though the harmonization solution may not be possible. The following options can be considered:

- CellGroupConfig IE for a candidate cell group is provided in conditional reconfiguration for the subsequent CPA/CPC

- CellGroupConfig IE for a candidate cell group is provided out of conditional reconfiguration for the subsequent CPA/CPC

- RRCReconfiguration message for a candidate cell group is provided in conditional reconfiguration for the subsequent CPA/CPC

- RRCReconfiguration message for a candidate cell group is provided out of conditional reconfiguration for the subsequent CPA/CPC

Proposal 5.	RAN2 discusses supporting delta configuration based on the below options:

- CellGroupConfig IE for a candidate cell group is provided in conditional reconfiguration for the subsequent CPA/CPC

- CellGroupConfig IE for a candidate cell group is provided out of conditional reconfiguration for the subsequent CPA/CPC

- RRCReconfiguration message for a candidate cell group is provided in conditional reconfiguration for the subsequent CPA/CPC

- RRCReconfiguration message for a candidate cell group is provided out of conditional reconfiguration for the subsequent CPA/CPC

We first analyze the common points included in each option based on the current RRC Reconfiguration structure. We consider the parameters that may affect the subsequent CPA/CPC based on the RRC reconfiguration structure attached to the Appendix. The common points of each option can be described as the following:

For option 1) and option 3), these options mean the provision way of nested configuration. For the nested configuration, CPA/CPC configuration for subsequent mobility should be included in the conditional reconfiguration. The network has to configure available candidates in each candidate repeatedly so that the UE can keep performing the subsequent mobility no matter which target cell is selected.

For option 2) and option 4), these options mean the provision way of non-nested configuration. For the non-nested configuration, CPA/CPC configuration for subsequent mobility is not included in the conditional reconfiguration. RAN2 needs further discussion on how to provide the configuration to the UE for the non-nested configuration. We are almost certain that the non-nested configuration should be provided to the UE separately from the conditional reconfiguration. Also, the UE can be provided the non-nested configuration earlier than the conditional reconfiguration.

For option 1) and option 2), these options provide the candidate cell group configuration as CellGroupConfig IE instead of RRCReconfiguration message. The CellGroupConfig IE instead of RRCReconfiguration message is applicable only to secondary cell group configuration.

For option 3) and option 4), these options provide the candidate cell group configuration as RRC Reconfiguration message. As the legacy principle, the UE can update any RRC configuration for both MN and SN through this RRC Reconfiguration message.

A detailed analysis of each option based on the common points above can be described as follows:

For option 1), there is no reason to use the nested configuration because the CellGroupConfig IE is used for a candidate. This option just eliminates the gain in signaling efficiency that can be obtained by not using the RRC Reconfiguration message. We exclude this option because there are no pros compared to other options.

For option 2), since all candidates are provided to the UE without additional containing RRC Reconfiguration message in a single RRC Reconfiguration message, even after PSCell change, the UE can maintain candidates through delta configuration as the legacy principle. Also, since the configuration for each candidate is much smaller than that of RRC Reconfiguration, a mismatch issue from reference configuration does not occur if all configurations are provided as full configuration.

However, option 2 has far more cons than the above pros. To support option 2, we have too many scenario constraints. In option 2, without an additional update from the network, the UE cannot update the measurement configuration nor master cell group configuration. In addition, since the SCG configuration is affected in most cases when the MN is changed, candidates cannot be likely to be maintained when PCell handover. Above all, the current SN should be able to interpret all CellGroupConfig IEs provided as candidates, otherwise, encapsulation should be required. Moreover, this option is applicable only in NR-DC scenario and not in MR-DC scenario. Considering future feature extensions, these numerous scenario constraints do not seem to be beneficial, so we exclude this option as well.

For option 3), since this nested configuration is an RRC Reconfiguration-based approach, this resolves most of the cons of option 2. However, the UE must suffer inefficient signaling because a large amount of RRC Reconfiguration message for candidates must be provided, and even each RRC Reconfiguration messages are likely to be duplicated for each target cell. Also, as RAN2 has already recognized, a mismatch issue of the reference configuration occurs due to subsequent mobility.

For option 4), this option includes all pros of option 3) and it is possible to resolve the signaling overhead and signaling redundancy issue caused by the nested configuration of option 3). In addition, since candidates are provided separately from the information of the conditional reconfiguration, each reference configuration for the candidate can also be provided separately even if the UE continues to perform subsequent mobility, i.e. a mismatch issue of reference configuration can be prevented. For the reference configuration and configuration for the candidate, some new principle may be required for delta configuration handling. The detailed operation of reference configuration is described in section 2.4.

Based on the above analysis, we can summarize briefly in the table below:

Table 1 Analysis for the delta configuration

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Nested Config | O | X | O | X |
| Pros | 1. message size reduction due to CellGroupConfig IE 2. no reference mismatch | 1. message size reduction due to CellGroupConfig IE 2. no reference mismatch 3. easy delta configuration handling | 1. measurement update available 2. MCG update available 3. MN change available 4. MR-DC available for future extension 5. current SCG not require interpretation for candidates | 1. measurement update available 2. MCG update available 3. MN change available 4. MR-DC available for future extension 5. current SCG not require interpretation for candidates 6. no reference mismatch 7. easy delta configuration handling |
| Cons | 1. measurement update not available w/o additional signaling 2. MCG update not available w/o additional signaling 3. same MN only 4. NR-DC only 5. current SCG requires interpretation for candidates 6. signaling overhead due to duplication of candidates | 1. measurement update not available w/o additional signaling 2. MCG update not available w/o additional signaling 3. same MN only 4. NR-DC only 5. current SCG requires interpretation for candidates 6. new principle required | 1. signaling overhead due to duplication of candidates 2. reference mismatch | 1. new principle required |
| New Impacts |  | 1. pre-configuration required at least for reference cell |  | 1. pre-configuration required at least for reference cell |

Proposal 6.	RAN2 considers table 1 to analyze each option for the delta configuration of the subsequent CPA/CPC.

Then since we prefer option 4), we propose also the following:

Proposal 7.	The UE receives RRCReconfiguration message for a candidate cell group for the subsequent CPA/CPC as the non-nested configuration of conditional reconfiguration which can be provided earlier than the conditional reconfiguration.

Reference Configuration

The big issue of delta configuration for the subsequent conditional mobility seems the mismatch of the reference cell. According to the legacy principle, the reference information should be source PSCell configuration and this can cause a configuration mismatch between the network and the UE. This is because the source PSCell can be changed while continuously performing the subsequent CPA/CPC but the configuration for each candidate may not be updated for the new source PSCell.

In our view, this issue can be simply resolved if option 4) is selected for the delta configuration handling. For option 4), the reference cell configuration can be provided to the UE as separated cell information instead of the source PSCell. Also, the reference cell configuration can be provided earlier than the conditional reconfiguration same as like candidate configurations for option 4), i.e. the network provides information for this separated reference cell as pre-configuration before performing subsequent CPA/CPC.

There may be various ways in which the configuration for the reference cell is provided. Since the cell configuration provided for PSCell addition is the full type of the configuration, the reference cell of the candidate for subsequent CPC can be the cell configuration provided for CPA. Otherwise, a specific cell configuration that is commonly applied to all candidates can be provided as a reference cell configuration. We think a detailed approach can be discussed later. The most important point at this moment is that the reference cell is separately provided to the UE, and the source PSCell should not be used as a reference cell to perform the subsequent CPA/CPC.

Thus we propose the following for the reference cell configuration:

Proposal 8.	The network provides a pre-configuration for reference cell configuration which is separately designated for the subsequent CPA/CPC (i.e. not source PSCell as the legacy).

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1.	The UE maintains the list of candidates for the subsequent CPA/CPC after mobility is complete unless the network indicates to remove.

Proposal 2.	The UE maintains the list of candidates for the subsequent CPA/CPC as deactivated if the network indicates the list is inapplicable in a new target cell, i.e. the UE doesn’t evaluate the execution condition for the list while staying in the new target cell.

Proposal 3.	The network doesn’t provide information for the UE to indicate how many subsequent mobilities can be targeted, i.e. the network release the conditional reconfiguration for the subsequent CPA/CPC if not valid or not needed as the legacy.

Proposal 4.	The network provides the list of candidates for the subsequent CPA/CPC and the legacy conditional reconfiguration together.

Proposal 5.	RAN2 discusses supporting delta configuration based on the below options:

- CellGroupConfig IE for a candidate cell group is provided in conditional reconfiguration for the subsequent CPA/CPC

- CellGroupConfig IE for a candidate cell group is provided out of conditional reconfiguration for the subsequent CPA/CPC

- RRCReconfiguration message for a candidate cell group is provided in conditional reconfiguration for the subsequent CPA/CPC

- RRCReconfiguration message for a candidate cell group is provided out of conditional reconfiguration for the subsequent CPA/CPC

Proposal 6.	RAN2 considers table 1 to analyze each option for the delta configuration of the subsequent CPA/CPC.

Proposal 7.	The UE receives RRCReconfiguration message for a candidate cell group for the subsequent CPA/CPC as the non-nested configuration of conditional reconfiguration which can be provided earlier than the conditional reconfiguration.

Proposal 8.	The network provides a pre-configuration for reference cell configuration which is separately designated for the subsequent CPA/CPC (i.e. not source PSCell as the legacy).

Reference

- R2-2209301, “RAN2 Chairman Notes,” MediaTek, Aug 2022

Appendix

This structure is based on TS 38.331-h10. Some not related parameters are omitted for readability.