3GPP TSG-RAN WG2 Meeting#120		       						R2-2212553

Toulouse, Nov 14th – 18th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Further Considerations on Expectation of Enhancement for LTM

Agenda item:		8.4.2.1.1

Document for: 	Discussion and Decision

- Introduction

Unitl RAN2#119bis emeeting, almost all possible enhancements identified at this stage for reducing the lantency of L1/L2 mobility have been coverred, and the agreements achieved in RAN2 have been sent to RAN1/RAN4 for their further study. However, for some enhancements as shown below:

- RAN2 assumes that RAN1 will drive discussions on L1 measurement enhancements, if any. If RAN1 identifies the need for e.g. event reporting, filtering etc, RAN2 can then be involved if needed.

- Inter-freq L1L2 mobility: R2 Confirms that For L1L2 mobility inter-freq scenarios in general should be supported (including mobility to inter-frequency cell that is not a current serving cell), including the support of inter-frequency L1 measurements, if feasible by R4 and R1.

we just inform them of the enhancements but not tell them why they need to be enhanced, it may result that the enhancements discussion in RAN1/RAN4 maybe inadequate or even worse, the outcome of the discussion may not reach our expectation.

In addition, we also left a FFS from the previous meeting about the pre-configuration of candidcate cells:

- FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified.

so in this paper, we would like to make a further discussion on above enhancements for facilitating the RAN1/RAN4 discussion on them, and share our views on the FFS.

Discussion

For measurement part in LTM, RAN2 have assumed that the L1 measurement enhancements is needed which shall be discussed in RAN1. However, we do not tell them what's the target of inter-cell L1 measurement enhancements which may cause a vague perspective for inter-cell L1 measurement enhancement.

In our understanding, there is one main target for inter-cell L1 measurement enhancements, that is the minimization of ping-pong effection from inter-cell L1 measurement. Even though the ping-pong cell switch in not included in the delay model of LTM, ping-pong cell switch may cause multiple useless cell switch which is more serious than any components in the delay model of LTM.

The inter-cell L1 measurement can provide more prompt reaction to the wireless environment change, it will result in the ping-pong cell switch as a price to pay due to the sensitivity to the sharp channel state change. In the intra-DU LTM case, the ping-pong effection maybe not obvious at this stage since it is mostly like a frequenet cell switch between the target cell and source cell and RAN2 assumed that no L2 re-establishments were needed for intra-DU LTM. Howeve, for inter-DU LTM case, the ping-pong cell switch may cause the severe UP data interruption for the inter-DU HO due to the L2 re-establishment (e.g. RLC entity re-establishment, PDCP recovery, F1 path switch..).

Proposal 1: The inter-cell L1 measurement mechanism enhancement shall focus on how to minimize the ping-pong cell switch, especially for the case of inter-DU LTM. And send LS to RAN1 for their further work on L1/L2 mobility.

In the case of inter-freq LTM, RAN2 have assumed the inter-cell LTM is generally supported, and would like RAN1/RAN4 to assess the feasibility. However, it is hard to understand the meaning of the feasiblity, in our understanding, we need to provide more explicit guidence for RAN4/RAN1 to continue their work on inter-cell freq LTM.

In our opinion, the most challenge part of the inter-freq LTM is measurement gap, the measurement gap shall be configured to UE for switching the RF chain to perform the measurement on the frequency other than the camped frequency, for example, the UE does not support the capability of the inter-frequency measurement without gap or there is no activated BWP which is including the frequency point to be measured. So we suggest, instead of the feasibility, and provide more clear statement for RAN4/RAN1 further work on inter-freq LTM.

Proposal 2: For the inter-frequency inter-cell L1 measurement, RAN2 assumes that at least the measurement gap is not needed if UE have a capability of the inter-freq measurement without gap or the frequency points of the (Candidate) target cells included in the current active BWP.

Proposal 3: For the inter-frequency inter-cell L1 measurement, measurement gap is optionally required based on the UE capability and the frequency of target cell. One LS can be sent to RAN1/RAN4 to check whether the existing measurement gap can be reused in inter-frequency inter-cell L1 measurement for LTM.

In the previous bis meeting, companies have discussed the issue of whether to performASN.1 decoding and validity/compliance check upon receiving the RRC signaling of canndidate cell configuration, and no conclusion is reached. From UE persepctive, this behavior is totally regarding the UE interior processing which has few impact on LTM, it is reasonable to leave such issue to UE implementation which is as similar as CHO.

Proposal 4: As same as CHO, whether to perform ASN.1 decoding and validity/compliance check upon receiving the RRC signaling is up to UE implementation.

- Conclusion and proposals

In this contribution, we discussed L1/L2 inter-cell mobility with the following observations and proposals:

Proposal 1: The inter-cell L1 measurement mechanism enhancement shall focus on how to minimize the ping-pong cell switch, especially for the case of inter-DU LTM. And send LS to RAN1 for their further work on L1/L2 mobility.

Proposal 2: For the inter-frequency inter-cell L1 measurement, RAN2 assumes that at least the measurement gap is not needed if UE have a capability of the inter-freq measurement without gap or the frequency points of the (Candidate) target cells included in the current active BWP.

Proposal 3: For the inter-frequency inter-cell L1 measurement, measurement gap is optionally required based on the UE capability and the frequency of target cell. One LS can be sent to RAN1/RAN4 to check whether the existing measurement gap can be reused in inter-frequency inter-cell L1 measurement for LTM.

Proposal 4: As same as CHO, whether to perform ASN.1 decoding and validity/compliance check upon receiving the RRC signaling is up to UE implementation.

- References

[1] RAN2#119bis emeeting Chairman Notes EOM