3GPP TSG-RAN WG2 Meeting #120          	          R2-2212574

Toulouse, France, November 14 – 18, 2022

Agenda Item:	6.21.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on per-FR-gaps

Document for:	Discussion and decision

- Introduction

Per-FR-gaps has been discussed in RAN2#119bis-e with the following conclusions [1].

Exclude Alt 1.1 for now. On the table: Alt 1.3, Alt 1.3 per BC, Alt 2 (add info, based on current config as today, FFS excl/incl DC)

In this contribution, we further analyse the potential solutions.

- Discussion

To support finer granularity indication for per-FR-gaps, the motivation is that with more complex NR CA combinations (UE operating in a high order CA combination will not have additional hardware available to perform per FR gap), the per-UE level per-FR-gap capability becomes less effective since if there is one BC that cannot support per-FR-gap, UE can only report per-UE level per-FR-gap capability as “not supported”. In this case, the NW will not configure per-FR-gap to some BCs even if per-FR-gap can be supported. This downgrades the system performance.

Observation: The per-UE level per-FR-gap capability becomes less effective if there exist complex BC(s) that cannot support per-FR-gap.

The following potential solutions are on the table for supporting finer granularity indication for per-FR-gaps.

Solution ① (Original Alt 1.1) Introducing per-FR-gap capability in per-BC level. This solution is simple and backward compatible, and both the NR SA and MR-DC can be supported, but the signalling overhead increases.

Solution ② (Original Alt 1.3) Introducing a threshold N indicating the maximum number of CCs in per UE/per FR level. Due to RF implementation in UE side, the threshold N may be different for different BCs, so a per UE/per FR threshold N may not be effective.

Solution ③ (Alt 1.3 per BC) Introducing a threshold N indicating the maximum number of CCs in per BC level. This updated solution can overcome the drawback of Solution ②, however, the reduction of  signalling overhead needs to be further justified. One example is give in Figure 1. (a), more fallback BCs still needs to be reported since the value “N” is different. Besides, we doubt whether the “N indicating the maximum number of CCs” really works well, see the example in Figure 1. (b), only the number of CCs without the band information may not be enough. Thus, in our view, the benefits of signalling reduction for this solution is not clear, instead, it leads to more uncertainty and complexity for UE reporting.

Figure 1. A threshold N indicating the maximum number of CCs reported in per BC level

Solution ④ (Original of Alt 2) Using similar framework as for “NeedForGap”. Based on the conclusion in last meeting, the new signalling for per-FR-gap capability is supposed to be based on current configuration as today, it means the gNB still use the current configuration to indicate the target bands that the UE is requested to report the gap requirement information. If the gap is needed, additionally the UE report whether the per-FR-gap can be supported. As analyzed in [2], only the current UE configuration processing affects the capability for per-FR-gap, not the measured frequencies. To a certain extent, we agree that differentiating each target bands is unnecessary, but considering that not only the RF channel will impact the per-FR-gap capability, but also the baseband resources, the per-FR-gap capability may be different to FR1 target band and FR2 target band. For example, although two independent RF channels are supported, if the target band is FR1 band, the baseband resources is not enough, the FR2 gap cannot be supported; if the target band is FR2 band, there is available baseband resources, the FR1 gap can be supported. Thus, per-FR-gap field can to be added to indicate whether per-FR-gap is supported or not with differentiating the target bands is FR1 or FR2. In addition, the solution combined with Alt 1.3 and Alt 2 was proposed, but as we analyze in Solution ③, so we think combining the number of CCs would make the solution more complicated but with unclear benefits.

Proposal 1: Add new per-FR-gap field in “NeedForGap” signalling to indicate whether per-FR-gap is supported or not with differentiating the target bands is FR1 or FR2.

In addition to Rel-16 measurement gap, the NCSG is introduced, the interruption at the beginning and the end of the measurement time will be caused. UE can indicate whether measurement gap or NCSG is required in RRCReconfigurationComplete/RRCResumeComplete message. For per-FR-gap capability, in addition to Rel-16 per-FR-gap capability, per-FR NCSG capability is defined. Thus, if finer granularity per-FR-gap capability is introduced, it should be applied to measurement gap and NCSG separately. The new per-FR-gap field in “NeedForGap” signalling can be introduced for measurement gap and NCSG separately.

| independentGapConfig This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in clause 9.1.2 of TS 38.133 [5]. The field also indicates whether the UE supports the FR2 inter-RAT measurement without gaps when (NG)EN-DC is not configured. | UE | No | No | No |
| --- | --- | --- | --- | --- |

| ncsg-MeasGapPerFR-r17 Indicates whether the UE supports per-FR NCSG. UEs supporting this shall indicate support of nr-NeedForGapNCSG-reporting-r17. | UE | No | No | No |
| --- | --- | --- | --- | --- |

We further discuss the relation between existing UE capability and new per-FR-gap field in “NeedForGap” signaling. If the UE report “supported” for existing independentGapConfig, the network considers that per-FR-gap is supported for all BCs. Otherwise, if the UE support nr-NeedForGap-Reporting-r16 and per-FR-gap reporting in “NeedForGap” signaling, the gNB can further request UE to report whether the per-FR-gap can be supported for current configuration. The similar operation can be applied to NCSG.

Proposal 2: The new per-FR-gap field in “NeedForGap” signalling is introduced for measurement gap and NCSG separately.

If “NeedForGap” signalling is adopted, the next question is whether the DC can be supported. The “NeedForGap” signalling for DC was discussed in Rel-16 but was not agreed to support DC at that time due to the complicated coordination between MN and SN. For example, gap capability including the per-FR-gap capability will change due to the change of SCG configuration (i.e. addition/release of SCells), UE reports the new gap capability via RRCReconfigurationComplete message to the SN, then the SN needs to send this information to the MN since only the MN configures gap to the UE. Even we only allow the MN to request the gap capability, that means the UE reports the new gap capability via RRCReconfigurationComplete message to the MN directly without transfer between MN and SN, however, the SN needs to inform the change of SCG configuration so that the MN can request the UE. It also increase the interaction between MN and SN. Based on the above analyses, it is suggested not to support DC in this release.

Proposal 3: If “NeedForGap” signalling is adopted, the DC is not supported in this release.

Regarding which release to support finer granularity indication for per-FR-gaps, if RAN2 can determine the solution, during this Quarter, it can be introduced from Release 17. However, if the solution is complicated and it is difficult to reach consensus, Release 18 is suggested.

Proposal 4: It is suggested to support finer granularity per-FR-gaps capability from Release 17 if solution can be determined during this Quarter. Otherwise, Release 18 is suggested.

The draft CRs are provided in [3][4].

- Conclusion

In this contribution, we further analyse the potential solutions for per-FR-gaps and provide the following observation and proposals:

Observation: The per-UE level per-FR-gap capability becomes less effective if there exist complex BC(s) that cannot support per-FR-gap.

Proposal 1: Add new per-FR-gap field in “NeedForGap” signalling to indicate whether per-FR-gap is supported or not with differentiating the target bands is FR1 or FR2.

Proposal 2: The new per-FR-gap field in “NeedForGap” signalling is introduced for measurement gap and NCSG separately.

Proposal 3: If “NeedForGap” signalling is adopted, the DC is not supported in this release.

Proposal 4: It is suggested to support finer granularity per-FR-gaps capability from Release 17 if solution can be determined during this Quarter. Otherwise, Release 18 is suggested.

- References

- RAN2#119bis-2 Chair Notes.

- R2-2210237, Discussion on per-FR gaps, Nokia, Nokia Shanghai Bell.

- R2-2212575, CR on finer granularity indication for per-FR-gaps (38.331), Huawei, HiSilicon.

- R2-2212576, CR on finer granularity indication for per-FR-gaps (38.306), Huawei, HiSilicon.