3GPP TSG-RAN2#120	R2-2212579

Toulouse, 14th Nov – 18th Nov, 2022

Agenda item:		8.5.3.1 (FS_NR_XR_enh)

Source:	LG Electronics Inc.

Title: 	DRX enhancement for power saving in XR

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed DRX enhancements for XR and made an agreement as follows.

1: At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed.

In this document, we present our views for XR-specific power saving based on the last meeting agreements.

2.	Discussion

In the last meeting, RAN2 agreed to consider DRX configuration switching based on RRC pre-configuration. Considering terms "pre-configuration" and "switching" in the agreement, we can assume that RAN2 support multiple DRX configurations for XR power saving, although there is no explicit agreement.

Observation 1. Based on the agreement in the last meeting, it can be assumed that RAN2 support multiple DRX configurations for XR power saving.

However, how many multiple DRX configurations are allowed, whether to allow multiple DRX configurations to be active, and how to switch DRX configuration among configured multiple DRX configurations need to be discussed further.

According to Study on XR Evaluation for NR [1], it is beneficial for power saving to start onDuration aligning with each XR DL traffic arrival time or to use multiple DRX patterns. That is, there is power saving gain if DRX configuration is configured considering the traffic characteristics. The following table shows the result of power saving evaluation in [1].

###### 8.3.3.1.1.1 DL+UL joint evaluation Table 8.3.3.1.1.1-1: Summary of FR1, DL+UL power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PSG (%), Note 1,4; Source. Mean (%); Range (%). DU; VR; 30; eCDRX; 11.64; 4.51 ~ 23.49; Source 18, Source 7, Source 16. CG; 30; eCDRX; 6; Source 7. AR (UL 1/2 streams); 30; eCDRX; 11.06; 4.6 ~ 20.77; Source 18. InH; VR; 30; eCDRX; 17.63; 7.23 ~ 25.12; Source 20, Source 18. 45; eCDRX; 25.64; 25.63 ~ 25.65; Source 20. CG; 30; eCDRX; 18.25; 18.23 ~ 18.26; Source 20. AR (UL 1/2 streams); 30; eCDRX; 12.23; 4.82 ~ 23.61%; Source 18. Note 1 : PSG was computed for the cases only with marginal loss in % of DL+UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see clause 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme.. ###### 8.3.3.1.1.2 DL-only evaluation Table 8.3.3.1.1.2-1: Summary of FR1, DL-only power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PSG(%), Note 1,4; Source. Mean (%); Range (%). DU; VR; 30; eCDRX; 15.70; 5.76 ~ 34.95; Source 9, Source 18, Source 7, Source 16. 45; eCDRX; 18.14; 9.72 ~ 27.26; Source 18. InH; VR; 30; eCDRX; 20.812; 9.36 ~ 29.43; Source 20, Source 18. 45; eCDRX; 19.96; 9.42 ~ 29.1; Source 20, Source 18. CG; 30; eCDRX; 26.38; 26.38; Source 20. UMa; VR; 30; eCDRX; 18.88; 10.05 ~ 29.06; Source 18. 45; eCDRX; 18.22; 9.86 ~ 27.33; Source 18. Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see clause 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme.. ###### 8.3.3.1.1.3 UL-only evaluation Table 8.3.3.1.1.3-1: Summary of FR1, UL-only power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PS Gain (%), Note 1,4; source. Mean (%); Range (%). DU; AR UL 1 / 2 streams; 10.2; eCDRX; 25.56%; 19.89 ~ 32.02%; Source 18. InH; AR UL 1 / 2 streams; 10.2; eCDRX; 28.67%; 22.66 ~ 35.24%; Source 18. Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see clause 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme..

Observation 2. There is power saving gain if DRX configuration is configured considering the traffic characteristics.

According to XR SID in [2], XR service can have multiple flows which may be configured to handle different traffic characteristic and the periodicity of XR traffic. For example, Flow#1 is mapped to video frame and Flow#2 is mapped to audio frame. Then, the traffic periodicity of Flow#1 may be shorter than Flow#2. Thus, considering the Observation 2, in terms of power saving, it would be beneficial to apply different DRX configuration for each flow.

Observation 3. In terms of power saving, it would be beneficial to apply different DRX configuration for each flow.

Based on Observation 2 and 3, DRX configuration should be applied to fit to the multiple XR traffic flows. However, in the current specification, it is only allowed to have two DRX groups, i.e., only two DRX configurations are allowed. In order to apply different DRX configuration for each flow, the current two DRX configurations may not be sufficient, and more than two DRX configurations should be able to be configured and to be active to support multiple flows of XR service.

In our view, if different DRX configurations are applied to each flow to fit to their traffic characteristic, considerable power saving gain would be achieved. Thus, for power saving in XR, we think RAN2 should consider to configure more than two DRX configurations and to allow more than two DRX configurations to be active

Proposal 1. RAN2 should consider to allow more than two DRX configurations to be active.

Another point to be considered is how to switch DRX configuration amongst configured multiple DRX configurations. As per the justification of [2], characteristics of the data streams (i.e., video) may be changed during ongoing XR service. In this condition, traffic patterns may be changed dynamically. Thus, if DRX configuration can be changed dynamically according to the changed characteristic of XR traffic, it is much beneficial for power saving.

Currently, however, DRX configuration can be changed by RRC, and it may not be sufficient to adapt quickly to the changed XR traffic characteristic during ongoing XR service. Thus, from power saving point of view, dynamic switching of DRX configuration (e.g. DCI or MAC CE) is beneficial than semi-static switching (RRC signalling). We propose that dynamic switching of DRX configuration other than semi-static switching (RRC signalling) should be considered to apply DRX configuration immediately in response to change of XR traffic characteristic.

Proposal 2. RAN2 should consider to switch DRX configuration dynamically.

In RAN2#119-e, RAN2 agreed to focus on the DRX enhancement to address issues from jitter. According to Study on XR Evaluation for NR [1], XR service has the characteristic of jitter which makes arrival timing random. XR DL data may not arrive at the onDuration due to the jitter and this delayed DL data may be received in the next DRX cycle. In this case, the packet delay budget requirement of XR DL data may not be met and the XR service quality may be deteriorated.

Considering that the packet arrival time would be spread out due to jitter, e.g., ±4ms, longer value of onDuration time may be considered as a simple way to resolve the impact of jitter. However, just longer value of onDuration time may not be a sufficient solution. This is because even if the longer value of onDuration time is introduced, XR DL data still may not arrive during onDuration time with longer value due to jitter and the UE should monitor PDCCH unnecessarily during this long onDuration time. Thus, in addition to longer value of onDuration time, some enhancements to avoid unnecessary power consumption should be considered.

First, we think the unnecessary PDCCH monitoring performed for all serving cells should be reduced. Considering that many serving cells may be configured to support high DL data rate and multiple flows of XR services, unnecessary power consumption would be significant and serious since the UE should monitor PDCCH on all serving cells belonging to a DRX group unnecessarily during this long onDuration time. If XR DL traffic arrival is delayed, the UE should wait for DL burst arrival while performing unnecessary PDCCH monitoring, i.e., onDuration. The unnecessary PDCCH monitoring increases UE power consumption.

Thus, if the longer value of onDuration time is introduced to cover jitter, we think it would be desirable to monitor PDCCH on all serving cells only when XR DL traffic is expected, i.e., no need to monitor PDCCH on all serving cells during onDuration time if XR DL traffic is not expected to be received. For example, UE monitors PDCCH only on SpCell when there is no data, and then once the UE receives a data, it monitors PDCCH on all serving cells. We think the unnecessary power consumption can be avoided by reducing the number of serving cell the UE monitors PDCCH.

Proposal 3. RAN2 needs to study how to reduce unnecessary PDCCH monitoring on all serving cells during onDuration time.

Currently, UE performs PDCCH monitoring continuously as long as onDuration timer is running. Considering that the UE should monitor PDCCH unnecessarily during this long onDuration time as explained above, if longer value of onDuration timer is introduced, continuous PDCCH monitoring brings significant power consumption.

Thus, we think discontinuous PDCCH monitoring can be performed during onDuration. More specifically, short value of onDuration timer would be running multiple times with small waiting time instead of one long value of onDuration timer running. For example, when onDuration is 10ms, if one long value of onDuration timer is used, the UE needs to monitor continuously PDCCH for 10ms. However, if short value of onDuration timer is 1ms, the UE starts short value of onDuration timer and monitors PDCCH for 1ms then the UE starts again short value of onDuration timer after 1ms waiting. In the end, the UE can discontinuously monitor PDCCH and power consumption can be reduced while the UE can sufficiently monitors PDCCH during onDuration period to cover the delayed data due to jitter. We think that RAN2 should study how to efficiently monitor PDCCH during onDuration without increasing significant power consumption when the data is delayed by jitter.

Proposal 4. RAN2 should study how to efficiently monitor PDCCH during onDuration without increasing unnecessary power consumption when the data is delayed by jitter.

As addressed in the justification of XR SID [2], there is clear different traffic characteristics between UL and DL, i.e., XR services may have high data rate in DL (i.e., video steam) combined with the frequent UL (i.e., pose/control update). Considering that the optimized DRX configuration matched to the UL traffic characteristics brings more power saving gain as seen in the following table in [1], we think it would be good to apply DRX parameters differently for DL and UL.

Observation 4. Separate DRX parameters for DL and UL is beneficial for power saving.

###### 8.3.3.1.1.3 UL-only evaluation Table 8.3.3.1.1.3-1: Summary of FR1, UL-only power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PS Gain (%), Note 1,4; source. Mean (%); Range (%). DU; AR UL 1 / 2 streams; 10.2; eCDRX; 25.56%; 19.89 ~ 32.02%; Source 18. InH; AR UL 1 / 2 streams; 10.2; eCDRX; 28.67%; 22.66 ~ 35.24%; Source 18. ###### 8.3.3.1.2.2 UL-only evaluation Table 8.3.3.1.2.2-1: Summary of FR2, UL-only power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PSG(%), Note 1,4; source. Mean (%); Range (%). DU; AR UL 1 stream; 10; eCDRX; 32.35; 31.72 ~ 32.97; Source 18. InH; AR UL 1 stream; 10; eCDRX; 37.57%; 36.79 ~ 38.35; Source 18.

Currently, however, only drx-HARQ-RTT-Timer and drx-RetransmissionTimer are separately configured for DL and UL, and others not. In order to provide the optimized DRX configuration for DL traffic and UL traffic separately, we think that at least drx-onDurationTimer and drx-InactivityTimer should be also separately configured for DL and UL.

In addition to DL/UL DRX timer separation, we also think that separate DRX timer for Short DRX cycle and Long DRX cycle may be beneficial from power saving perspective. Short DRX cycle is used when the probability of the DL transmission is expected to be high and Long DRX cycle is used when the probability of the DL transmission is expected to be low. Thus, it is reasonable to use different DRX timers for Short DRX cycle and Long DRX cycle, e.g., drx-onDurationTimer for Short DRX cycle is longer than drx-onDurationTimer for Long DRX cycle. Therefore, we think separate DRX timer for DL/UL and Short DRX cycle/Long DRX cycle should be considered for power saving.

Proposal 5. RAN2 should consider separate DRX timers for DL/UL and Short DRX cycle/Long DRX cycle.

3.	Conclusion

In this document, we present our view on power saving in XR, and made observations and proposals as follows.

Observation 1. Based on the agreement in the last meeting, it can be assumed that RAN2 support multiple DRX configurations for XR power saving.

Observation 2. There is power saving gain if DRX configuration is configured considering the traffic characteristics.

Observation 3. In terms of power saving, it would be beneficial to apply different DRX configuration for each flow.

Observation 4. Separate DRX parameters for DL and UL is beneficial for power saving.

Proposal 1. RAN2 should consider to allow more than two DRX configurations to be active.

Proposal 2. RAN2 should consider to switch DRX configuration dynamically.

Proposal 3. RAN2 needs to study how to reduce unnecessary PDCCH monitoring on all serving cells during onDuration time.

Proposal 4. RAN2 should study how to efficiently monitor PDCCH during onDuration without increasing unnecessary power consumption when the data is delayed by jitter.

Proposal 5. RAN2 should consider separate DRX timers for DL/UL and Short DRX cycle/Long DRX cycle.

4.	Reference

- 3GPP TR 38.838	Study on XR (Extended Reality) Evaluations for NR

- RP-220285	Study on XR Enhancements for NR	Nokia