3GPP TSG-RAN WG2 Meeting #120	R2-2212600

Toulouse, France, 14th – 18th November, 2022

Agenda item:	8.4.2.3

Source:	Samsung

Title:	Considerations on the Cell Switch for LTM

Document for:	Discussion & Decision

# Introduction

In RAN2#119bis-e meeting, RAN2 made following agreements on the Dynamic Switch for LTM [1].

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with L1 L2 mobility trigger MAC CE (if so, FFS how this is determined). RAN2 assumes that both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported. RACH-less if the UE doesn’t need to acquire TA during the cell switch. RAN2 understands that the feasibility of RACH-less may depend on RAN1, and expect that RAN1 is working on this. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress. R2 assumes that at L1L2 cell switch: Whether the UE performs partial or full MAC reset (FFS what partial reset is, e.g. to avoid data loss), re-establish RLC, perform data recovery with PDCP is explicitly controlled by the network. R2 assumes that this can be configured by RRC. FFS if MAC CE indication(s) is/are needed.

In this contribution, we consider the dynamic switch mechanism using L1/L2 signalling and potential applicable scenario based on L1/L2 signalling. The detail MAC related issues will be treated in our other contribution.

# Discussion

## LTM configuring/triggering node

In this section, we want to clearly determine which node decides the following two operation.

- Which node determine the candidate target cells for LTM

- Which node trigger LTM and the target cell

Above two questions are very essential and they should be determined first before going to the detail procedure for LTM. From our understanding the first question is clear, CU will have a role to determine the candidate target cells for LTM based on the L3 measurement. In Rel-18 feMob, only intra-CU case is considered, there are no further work between the CUs, the CU can determine the candidate target cells for LTM and request the addition/change of LTM configuration to the candidate target cells. Based on the response from the candidate target cells, the CU generates the RRCReconfiguration message by appending any necessary information to the received CellGroupConfig from the DU(s), and construct the list of these RRCReconfiguration messages (i.e., might be called as inner RRCReconfiguration) and its associated ID, and send this in the LTM configuration container in the outer RRCReconfiguration message to the UE.

Proposal 1: CU determines the candidate target cells for LTM based on the L3 measurements and CU generates the RRCReconfiguration message including the candidate cell configurations for LTM.

For the second question, we think there are two possible approaches which node trigger LTM based on the L1 measurement for the target cell.

- Option 1: CU triggers the LTM

- Each Cell/DU needs to transfer the received L1 measurement to the CU.

- Among the collected L1 measurements for all candidate target cells, CU determines the target cell for LTM

- CU indicates the LTM information (e.g. target cell ID, beam info., etc.) to the target DU’s MAC entity.

- Option 2: serving DU’s MAC entity triggers the LTM

- CU and serving DU negotiate the LTM triggering condition for the candidate target cells.

- Amongst the L1 measurements of all candidate target cells, the serving DU determines the target cell for LTM.

- Serving DU’s MAC entity generate the LTM signaling and send it to the UE.

The LTM is triggered based on the L1 measurements of the candidate target cells, serving cell receives these L1 measurements from the UE. RAN2 has agreed that at least MAC CE is used to trigger the LTM, it means that serving DU’s MAC entity has the role to trigger the LTM. It is a good approach to reduce the overall latency of LTM because the additional signaling between CU and DU can be omitted to trigger the LTM. In that sense, we think option 2 has more benefit to reduce the overall latency for LTM.

Proposal 2: Serving DU’s MAC entity triggers the LTM based on the L1 measurements from the UE.

## Beam information for LTM MAC CE

In RAN2#119-e meeting, RAN2 has agreed that L1 measurement is assumed to trigger LTM, and RAN2 has confirmed that the details will be determined by RAN1 decision. In Rel-17 feMIMO WI, inter-cell beam management (ICBM) using unified TCI framework is operated with the enhanced L1 measurement and report procedure which enables to measure/report the SSB/CSI-RS resources associated with PCIs different from the PCI of a serving cell. CSI resources to be measured are configured and list of references to SSB resources used for CSI measurement/report in a CSI-RS resource set is provided.

- CSI-SSB-ResourceSet includes the additional PCI info for CSI resource set.

- The number of additional PCI configured for CSI-RS is at most 7 (i.e. servingAadditionalPCIList-r17).

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17           SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF  ServingAdditionalPCIIndex-r17    OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

CSI report is configured in ServingCellConfig and following entities are configured:

- Report type (periodic, semi-persistent for PUCCH, semi-persistent for PUSCH, aperiodic)

- Report quantity

- Etc.

We assume this L1 measurement/report procedure would be the baseline of Rel-18 LTM as well. Based on L1 measurement results reported by the UE, the NW can know which beam associated with the target cell is suitable for the UE. Therefore, when triggering the cell switch by L1/L2 signaling (e.g., MAC CE), the NW can indicate TCI state(s) associated with the target cell to be activated upon cell switch. In this way, instead of further refinement of beam selection based on L1 measurement after L3 HO execution, the UE can directly apply the indicated TCI state(s) in LTM for a better performance.

Proposal 3: the MAC CE triggering LTM can indicate TCI state(s) to be activated for the target cell.

By inter-cell beam management (ICBM), a UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the PCI of a serving cell. In RAN2#119-e meeting, RAN2 has agreed that ICBM is one scenario considered for L1/L2 mobility. Thus, the cell with additional PCI configured for ICBM should be a candidate cell for LTM.

Proposal 4: The cell with additional PCI for ICBM can be configured as a candidate cell for LTM.

For a UE under ICBM, i.e., receiving or transmitting UE dedicated channels/signals via a TRP associated with a cell with additional PCI, LTM can be triggered for cell switch to a target cell, where the target cell can be the cell with additional PCI for ICBM. In this case, the NW can send the L1/L2 signaling via the dedicated channels from the cell with additional PCI. By this operation, UE first under the ICBM on the serving cell connection, but NW indicate HO this UE to the target cell by LTM based on L1 measurement/report. It means two step operation of ICBM and LTM should be supported as well as one step LTM with beam indication.

Proposal 5: For the UE under ICBM, the NW can send L1/L2 trigger signaling (i.e. LTM) via the cell with additional PCI.

As LTM is triggered based on L1 measurement without L3 filtering, the advantage of LTM is that the UE can switch to the target cell with a better beam in a shorter interruption time, while the drawback is the ping-pong effect. In order to take the advantage of fast cell/beam switch and mitigate the ping-pong effect, one way is that the UE first applies ICBM as a transition phase and then triggers LTM if certain conditions are fulfilled. The NW can configure LTM execution conditions in RRC pre-configuration for candidate cells. The following options can be considered for UE-triggered LTM.

- Option 1: Timer-based LTM triggering

- The UE starts a timer upon receiving a TCI state switch that requests UE to use a beam from a candidate cell.

- The UE re-starts the timer upon receiving a TCI state switch that requests UE to use a beam from a different candidate cell.

- The UE stops the timer upon receiving a TCI state switch that requests UE to use a beam from the serving cell.

- At the expiry of the timer for a candidate cell, the UE executes handover to that candidate cell.

- Option 2: Counter-based LTM triggering

- UE counts the number that L1-RSRP of candidate cell is better than the serving cell.

- UE triggers the LTM when the counter reaches the configured max value.

- Option 3: NW indication based LTM triggering

- UE just operate based on NW signaling i.e. if LTM is required NW will send the LTM signaling.

- In this case, mitigating ping-pong effect is up to NW implementation.

Proposal 6: For the UE under ICBM, consider how UE initiates LTM to mitigate ping-pong effect.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: CU determines the candidate target cells for LTM based on the L3 measurement and CU generates the RRCReconfiguration message by appending the candidate cell configurations for LTM.

Proposal 2: Serving DU’s MAC entity triggers the LTM based on the L1 measurement from the UE.

Proposal 3: the MAC CE triggering LTM can indicate TCI state(s) to be activated for the target cell.

Proposal 4: The cell with additional PCI for ICBM can be configured as a candidate cell for LTM.

Proposal 5: For the UE under ICBM, the NW can send L1/L2 trigger signaling (i.e. LTM) via the cell with additional PCI.

Proposal 6: For the UE under ICBM, consider how UE initiate LTM to mitigate ping-pong effect.

# References

Chairman notes for RAN2#119bis-e meeting	RAN2.