3GPP TSG-RAN2 Meeting #120-e	R2-2212604

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3718 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to support repetition on PDSCH time domain resource allocation for DCI format 1-2 (Option 1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core, NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the current RRC specification, the single-DCI inter-slot TDM multi-TRP PDSCH repetition scheme for DCI format 1-2 is not supported. However, PDSCH time domain resource allocation for DCI format 1-2 should support the use of repetitionNumber based on RAN1 response in [1] R2-2006057. 3) PDSCH time domain resource allocation PDSCH-TimeDomainResourceAllocation can be configured with repetitionNumber. Meanwhile, pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 was introduced in PDSCH-Config. Q3-1) Can the PDSCH time domain resource allocation for DCI format 1-2 support the use of repetitionNumber? [Answer]: Yes. Q3-2) If the answer to Q3-1 is yes, can repetitionNumber be configured in the PDSCH time domain resource allocation for DCI format 1-2 if it is not configured in the time domain resource allocation for other DCI formats (and vice-versa), or should it be configured in the PDSCH time domain resource allocation for all DCI formats or for none? [Answer]: Yes. The configuration of the repetitionNumber in pdsch-TimeDomainResourceAllocationList and in pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 is independent.. However, the conditional presence in PDSCH-TimeDomainResourceAllocation-r16 describes that it is not possible to configure the repetition number for PDSCH time domain resource allocation for DCI format 1-2. |
| Summary of change: |  | Change the description of the conditional presence for repetitionNumber-r16 (i.e. Formats1-0and1-1) to support configuring the repetition number for PDSCH time domain resource allocation for DCI format 1-2. - Remove the pdsch-TimeDomainAllocationListDCI-1-2 in the absent condition. In pdsch-TimeDomainResourceAllocationList-r16, this field is optionally present, Need R. In pdsch-TimeDomainAllocationListDCI-1-2, this field is absent. Impact analysis Impacted 5G architecture options: EN-DC, SA, NE-DC, NR-DC Impacted functionality: Support repetition on PDSCH time domain resource allocation for DCI format 1-2 Inter-operability: If the UE is implemented according to the CR while the network is not, there are no inter-operability problem. If the network is implemented according to the CR while the UE is not, the UE might have different understanding on the configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The single-DCI inter-slot TDM multi-TRP PDSCH repetition scheme for DCI format 1-2 is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================================================== CHANGE BEGIN==============================================================

### 6.3.2	Radio resource control information elements

#### –	PDSCH-TimeDomainResourceAllocationList

The IE PDSCH-TimeDomainResourceAllocation is used to configure a time domain relation between PDCCH and PDSCH. The PDSCH-TimeDomainResourceAllocationList contains one or more of such PDSCH-TimeDomainResourceAllocations. The network indicates in the DL assignment which of the configured time domain allocations the UE shall apply for that DL assignment. The UE determines the bit width of the DCI field based on the number of entries in the PDSCH-TimeDomainResourceAllocationList. Value 0 in the DCI field refers to the first element in this list, value 1 in the DCI field refers to the second element in this list, and so on.

PDSCH-TimeDomainResourceAllocationList information element

-- ASN1START

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START

PDSCH-TimeDomainResourceAllocationList ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation

PDSCH-TimeDomainResourceAllocation ::=   SEQUENCE {

k0                                      INTEGER(0..32)                                                  OPTIONAL,   -- Need S

mappingType                             ENUMERATED {typeA, typeB},

startSymbolAndLength                    INTEGER (0..127)

}

PDSCH-TimeDomainResourceAllocationList-r16 ::=  SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation-r16

PDSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k0-r16                                     INTEGER(0..32)                                              OPTIONAL,   -- Need S

mappingType-r16                            ENUMERATED {typeA, typeB},

startSymbolAndLength-r16                   INTEGER (0..127),

repetitionNumber-r16                       ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                OPTIONAL,   -- Cond Formats1-0and1-1and1-2

...

}

-- TAG-PDSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP

-- ASN1STOP

PDSCH-TimeDomainResourceAllocation field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1) When the field is absent the UE applies the value 0.

mappingType PDSCH mapping type. (see TS 38.214 [19], clause 5.3).

repetitionNumber Indicates the number of PDSCH transmission occasions for slot-based repetition scheme in IE RepetitionSchemeConfig. The parameter is used as specified in 38.214 [19].

startSymbolAndLength An index giving valid combinations of start symbol and length (jointly encoded) as start and length indicator (SLIV). The network configures the field so that the allocation does not cross the slot boundary (see TS 38.214 [19], clause 5.1.2.1).

| Conditional Presence | Explanation |
| --- | --- |
| Formats1-0and1-1and1-2 | In pdsch-TimeDomainResourceAllocationList-r16, this field is optionally present, Need R. |

=================================================   END OF CHANGES   =========================================================