3GPP TSG RAN WG2#120	R2-2212614

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	8.7.4.1

Source:	CMCC

Title:	Discussion on NTN-TN reselection and reselection for earth moving cell

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions about cell reselection enhancements and some agreements were reached as following [1]:

Agreements:

- For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to  UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered)

- To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available.

Agreements:

- System information is the basic means for providing necessary parameters to assist UE to estimate when the serving cell stops providing coverage at the present UE location.

- UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage.

- The method of detecting the transmission energy or SIB presence to determine the NTN coverage when a UE currently camps on a TN cell is not pursued.

- In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

In this contribution, we would like to provide some analysis for further reselection enhancements.

Discussion

## 2.1 NTN-TN cell reselection

To reduce UE power consumption due to unnecessary TN neighbor cell search and measurements beyond its reachable scope, it is beneficial that the network delivers available TN neighbor cell information to the UE. And according to the last meeting agreements, we have agreed that UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. However, regarding the assistance information for UE to identify an area where TN network is available is controversial and postponed to discuss. In the email discussion [2], there are some options proposed as follows:

1.	The cell center and cell radius of TN neighbour cells, or in other terms, the reference location and a distance threshold of TN neighbour cells

2.	The boundary line between TN area and NTN area

3.	For quasi-earth fixed cells, TN coverage is described by a distance range from the cell center and an angle range based on a reference direction

4.	An indication could be included in system information to indicate NTN cell’s coverage overlaps with terrestrial TN cell’s coverage

5.	NTN cell can be divided to several virtual areas based on certain criteria. The virtual areas and the corresponding TN frequency information are broadcast as assistance information to help UE perform more accurate TN measurements.

6.	Introduce a parameter using the polygon shape captured in TS 23.032 to describe the coverage area of a TN neighbour cell.

From our perspective, option 1 could be the starting point for discussion. However, the UE does not now the appropriate occasion to perform TN neighbour cells measurement only with the assistance information in option 1. And relative position information(e.g. direction and distance to the current NTN serving cell. ) between TN neighbour cells and the current NTN serving cell could also be provided to UE to help UE choose a suitable TN neighbor cell.

Proposal 1: Regarding the assistance information for UE to identify an area where TN network is available, option 1(The cell center and cell radius of TN neighbour cells, or in other terms, the reference location and a distance threshold of TN neighbour cells) could be the starting point for discussion.

Proposal 2: In addition to option 1, relative position information(e.g. direction and distance to the current NTN serving cell. ) between TN neighbour cells and the current NTN serving cell could also be provided to UE to help UE choose suitable a TN neighbor cell.

## 2.2 Cell reselection for earth moving cell

In RAN2 119b-e meeting, it is agreed that in Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

For the FFS issue about how to provide the reference location of serving cell, the following options were discussed in email discussion [2] without convergence:

1.only location coordinates of current cell center, and network is supposed to update the values every time when it is provided for Earth-moving cell.

2.multiple reference locations and its time information of Earth-moving cell.

3.based on the sub-satellite point derived by satellite ephemeris and the broadcasted location offset between sub-satellite point and the cell reference location

4.cell type (fixed or moving), reference location coordinates with a time stamp, and the velocity of reference location

5.cell type, reference location corresponding to the epochTime (reuse the existing epochTime).

As known, the reference location in a earth moving cell would change along with the satellite movement. The cell type(fixed or moving) could be deduced implicitly by the current mechanism(i.e. whether t-service is provided by NW). And for earth moving cell, reference location with corresponding time information at different point is important for real-time reference location estimation. While the moving trajectory of the satellite composed by satellite trajectory point with corresponding time information is able to be predicted with the ephemeris already. Therefore, the real-time reference location of earth moving cells could be estimated with the existed information and no additional information is required.

Proposal 3: The real-time reference location of earth moving cell could already be estimated with the existed ephemeris information and no additional information is required.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on cell reselection enhancements and have the following proposals:

Proposal 1: Regarding the assistance information for UE to identify an area where TN network is available, option 1(The cell center and cell radius of TN neighbour cells, or in other terms, the reference location and a distance threshold of TN neighbour cells) could be the starting point for discussion.

Proposal 2: In addition to option 1, relative position information(e.g. direction and distance to the current NTN serving cell. ) between TN neighbour cells and the current NTN serving cell could also be provided to UE to help UE choose suitable a TN neighbor cell.

Proposal 3: The real-time reference location of earth moving cell could already be estimated with the existed ephemeris information and no additional information is required.

Reference

- RAN2-119bis-e - NR-NTN-IoT-NTN_EOM

- R2-2210860 Report of [AT119bis-e][117][NR NTN Enh] cell reselection enhancements (Intel)