3GPP TSG-RAN WG2 Meeting #120	                                                         R2-2212634

Toulouse, France, November 14-18, 2022

Agenda Item:	8.3.3

Source:	CMCC

Title:	Discussion on SSB/SIB1/Paging-less NES solution

Document for:	Discussion

Introduction

In last RAN2 meeting, the following agreements are achieved related to the SSB/SIB-less solution.

Agreements:

- There is a need to allow NES cells to prevent legacy UEs from camping. FFS the definition of NES cells.

- Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell.   Options to bar UEs to be considered are 1) UseIntra/InterFreqExcludedCellList (FFS on the exact mechanism and spec impact) and 2) use cellBarred or cell reservation fields in MIB/SIB.

- The network should be able to configure NES capable UEs to (de)prioritize NES cells.  mechanism such as can be considered for both frequency and cell levels cell selection/reselection (de)prioritization.  FFS on whether the existing mechanism is sufficient.

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

In this contribution, we further share our considerations on the SSB/SIB/Paging-less solutions.

Discussion

SSB/SIB/Paging-less solution

A typical network deployment scenario is that anchor cell and several NES cells are co-located and overlapping deployed, as shown in Figure 1. Anchor cell and NES cells deployed on different frequencies.

Figure 1: Multicarrier Scenario, where anchor cell and NES cells are co-located

Energy saving benefits

Energy saving for not broadcasting the always on SSB and SIB1 signal on NES cells. Especially when NES cells are in the low load, broadcasting the always on SSB and SIB1 is not energy efficiency.

Energy saving for not broadcasting paging message on the NES cells. As shown in figure 1, for multiple carriers overlapping deployed scenario, the paging message would be the same for all the cells on these overlapping carriers. This is also beneficial for spectrum efficiency, since the radio resources are alleviated and no need to transmit the same paging message for multiple times.

In addition, considering path loss for high frequency carrier is higher than low frequency carrier, the coverage for lower frequency carrier is better than high frequency. Deploying the anchor cell on the low frequency, and deploying NES cells on the higher frequencies, can guarantee the system information for anchor cell and NES cells are well received by the UEs.

Observation 1: SSB/SIB1/Paging less NES is beneficial for both energy saving and spectrum efficiency.

To avoid any measurement on NES cell, one of the requirements should be that the anchor cell and NES cells shall be co-located and synchronized. So that the measurement on the anchor cell can be utilized for RACH resource selection on NES cells.

Proposal 1: Requirement for SSB/SIB/paging less solution includes, anchor cell and NES cells shall be co-located and synchronized.

Network broadcast info

Anchor cell broadcast SIs for NES cells. The SIs on the anchor cells containing the necessary information to access to the NES cells, including cell bar info, initial BWP configuration, RACH configuration, etc. The impact on spec mainly on the providing cell access info SI for NES cells.

UE behaviour and potential specification impacts

Idle mode UE camping on the anchor cell, receiving SIs from anchor cells. Idle mode UE also receives paging message from the anchor cell. While UE camping on the anchor cell, UE receives the valid initial BWP configuration of NES cells as well as RACH configuration of NES cells from the anchor cell. UE can perform random access to either anchor cell or NES cells.

Therefore, idle mode UE can receive and keep valid for all the necessary information for camping on the anchor cell and ready to perform RACH procedure for NES cell. There are some impacts on idle mode UE behaviour and we think the impacts are manageable.

RACH procedure

UE performs SSB measurements on anchor cell. When RACH procedure is triggered, the UE can perform RF retuning to non-anchor cell. UE selects RACH occasion & preamble and perform random access to the NES cell, according to the NES cell PRACH configuration received on the anchor cell.

Proposal 2: UE camps on and perform SSB measurements on the anchor cell in IDLE mode. When RACH procedure is triggered, UE is allowed to select RACH occasion & preamble and perform random access to the NES cell, according to the NES cell PRACH configuration received on the anchor cell.

Conclusions

In this contribution, we further discussed SSB/SIB1/Paging-less NES solutions, the following are our observations and proposals:

Observation 1: SSB/SIB1/Paging less NES is beneficial for energy saving and spectrum efficiency.

Proposal 1: Requirement for SSB/SIB/paging less solution includes, anchor cell and NES cells shall be co-located and synchronized.

Proposal 2: UE camps on and perform SSB measurements on the anchor cell in IDLE mode. Then UE is allowed to select RACH occasion & preamble and perform random access to the NES cell, according to the NES cell PRACH configuration received on the anchor cell.

References

- R2_119bis-e_meeting_report

Text Proposals for TR 38.864

-------------------------------------------------------TR 38.864---------------------------------------------------------------

6	Techniques to improve network energy savings

Editor's note: simulation results to be captured under this section.

Editor's note: RAN2 and RAN3 related aspect to be provided by using separate sections like 6.X when applicable.

## 6.1	Techniques in time domain

### 6.1.1	Technique A-1 Adapting transmission/reception of common channels/signals

#### 6.1.1.1	Description of technique

Editor's note: potential need of UE assistance is also to be described here.

A typical network deployment scenario is that anchor cell and several NES cells are co-located and overlapping deployed, as shown in Figure X. Anchor cell and NES cells deployed on different frequencies.

Figure X: Multicarrier Scenario, where anchor cell and NES cells are co-located

Anchor cell broadcast SIs for NES cells. The SIs on the anchor cells containing the necessary information to access to the NES cells, including cell bar info, initial BWP configuration, RACH configuration, etc.

Idle mode UE camping on the anchor cell, receiving SIs from anchor cells. Idle mode UE can also receive paging message from the anchor cell.

While UE camping on the anchor cell, UE receives the valid initial BWP configuration of NES cells as well as RACH configuration of NES cells from the anchor cell. UE can perform random access to either anchor cell or NES cells.

UE perform SSB measurements on anchor cell. Then UE performs RF retuning to non-anchor cell. UE select RACH occasion & preamble and perform random access to the NES cell, according to the NES cell PRACH configuration received on the anchor cell.

#### 6.1.1.2	Analysis of performance and impacts

Editor's note: potential impact on UE side is also to be included here.

Performance

Energy saving for not broadcasting the always on signals, i.e., SSB and SIB1 signal, on NES cells. Especially when NES cells are in the low load, broadcasting the always on SSB and SIB1 is not energy efficiency.

Energy saving for not broadcasting paging message on the NES cells. As shown in figure 1, for multiple carriers overlapping deployed scenario, the paging message would be the same for all the cells on these overlapping carriers. This is also beneficial for spectrum efficiency, since the radio resources are alleviated and no need to transmit the same paging message for multiple times.

#### 6.1.1.3	Specification impacts

Editor's note: potential need of UE assistance that may have RAN2 impact is also to be provided here, preferably using a separate paragraph for RAN2 easy reference.

Impact on system information: Anchor cell broadcast SIs for NES cells. The SIs on the anchor cells containing the necessary information to access to the NES cells, including cell bar info, initial BWP configuration, RACH configuration, etc.

Impact on paging message: Only anchor cell broadcast paging message. NES cell doesn’t need to broadcast paging message. It seems no specification change is needed.

Impact on idle mode UE behaviour: Idle mode UE camping on the anchor cell, receiving SIs and paging message from anchor cells. While UE camping on the anchor cell, UE receives the valid initial BWP configuration of NES cells as well as RACH configuration of NES cells from the anchor cell. UE can perform random access to either anchor cell or NES cells.

Impact on RACH procedure: UE performs SSB measurements on anchor cell. When RACH procedure is triggered, UE performs RF retuning to non-anchor cell. UE selects RACH occasion & preamble and perform random access to the NES cell, according to the NES cell PRACH configuration received on the anchor cell.