3GPP TSG-RAN WG2 Meeting #120	R2- 2212638

Toulouse, France, November 14-18, 2022                  (revision of R2-2210504)

Agenda item:	8.8.2

Source: 	Huawei, HiSilicon

Title: 	Further discussion on UAV measurement enhancements

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #119b, the following agreements have been reached regarding NR support for UAVs [1]:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

- Continue to study height-depending scaling, triggering and combinations

- As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

As we can see, there are some FFS that remain to be analysed. This contribution a revision of R2-2210504. Compared to the previous version we analysed a few more technical aspects due to be resolved in RAN2 (i.e. FFSs).

- Discussion

2.1 Discussion on the flight path report

The flight path was agreed to be reported to the network for NR UAV in the last meeting. However, the format of waypoint and time information, which are included in the flight path, has not been defined. Furthermore, we note that such information has been defined in LTE. The yellow part as follows is included in one waypoint, and the maximum number of waypoints is 20.

WayPointLocation-r15 ::=			SEQUENCE {

wayPointLocation-r15						LocationInfo-r10,

timeStamp-r15							AbsoluteTimeInfo-r10		OPTIONAL

}

The waypoint is indicated by the IE LocationInfo, and the timestamp is indicated by the IE AbsoluteTimeInfo which indicates an absolute time in the format YY-MM-DD HH:MM:SS and using BCD encoding.

Observation 1: In LTE, waypoints and timestamps that are included in the flight path are indicated by the IE LocationInfo and the IE AbsoluteTimeInfo, respectively.

In NR, the IE CommonLocationInfo similar to the LTE, and the IE AbsoluteTimeInfo indicates an absolute time. As for NR UAV, we think such IE can be used to indicate the expected location and time corresponding to the waypoints and time information, respectively, as in LTE, because we expect that there is nothing that needs to be changed in the use of waypoints compared to LTE and actually use the same format could save implementation effort.

Proposal 1: Waypoints and the time information for NR UAV that are included in the flight path can be indicated by the IE commonLocationInfo and the IE AbsoluteTimeInfo, respectively.

As we all know, the drone’s speed can be very high, and the success rate of handovers is inversely proportional to the drone’s speed, i.e., as the drone’s velocity increases, the success rate of handover decreases. Moreover, the network may need to adjust the mobility configuration based on the expected speed of the drone at the waypoint. Thus, the velocity of the drone corresponding to the waypoint can also be included in the flight path report.

Proposal 2: Besides waypoints and timestamps, the velocity corresponding to the waypoints can be included in the flight path report.

The flight path update was also agreed upon in the last meeting. As for how to update the flight path, we think it should be controlled by the network, because it might cause a heavy signalling overhead if the flight path updating is up to the UE's will, due to the flight path information being relatively large and the number of drones potentially high. Besides, the network might not need the updated flight path to update network configuration when the update flight path is available at the drone's side. At this time, the drone's sending the updated flight path to the network will waste the network resource. Thus, we think the drone needs to indicate to the network when the flight path update is available.

Proposal 3: The UAV should indicate to the network if a flight path update is available.

Furthermore, the flight path update available indication information can be one bit of indication and included in the UEAssistanceInformation message.

Proposal 4: The flight path update available indication information can be included in the UEAssistanceInformation message.

The network can follow the flight path report mechanism defined in LTE to obtain the updated flight path, i.e., a request-response procedure. In particular, after receiving the flight path update available, the network can request the UAV to report the updated flight path when the network needs the newest flight path, i.e., the network can request the updated flight path via the UEInformationRequest message as in LTE. Then, the drone will report the updated flight path via the UEInformationResponse message.

Proposal 5: The NW can follow the LTE mechanism to obtain the updated flight path, i.e., the UEInformationRequest and UEInfomationResponse.

2.2 Discussion on the height-depending scaling, triggering and combinations

In order to avoid the measurement reporting being excessively affected by the interference cell, a Time To Trigger (TTT) was introduced. It requires the cell quality, which is measured by the UE, to always be over the threshold for a certain time, i.e., TTT. Besides, speed-dependent TTT has been specified in LTE. When the speed of UE changes, the TTT will be scaled accordingly to adjust the reporting delay. This mechanism works well for the terrestrial UE. However, the aerial UE’s communication environment is different to the terrestrial UE's due to more LOS paths in the air. If we follow the legacy mechanism, i.e., reconfigure the TTT when the aerial UE exceeds a certain height, the configured TTT may not be suitable due to the communication delay and the drone’s high mobility. Thus, the TTT, which is configured for the terrestrial UE, is not suitable for the aerial UE when the aerial UE is at a different altitude. As a result, a "scalable" TTT is needed; for example, an aerial UE can be configured with a set of TTT for different altitudes. The aerial UE could, for example, select a normal TTT when it is on the ground, and when the altitude is higher than the base station, the aerial UE selects a larger TTT to avoid frequent reporting.

Proposal 6: A height adaptive TTT should be considered for NR UAV.

Fig.1 Illustration of the vertical mobility issue

The received signal strength of a terrestrial UE is dependent on the distance and obstructions between the terrestrial UE and the base station. However, such mobility management for the terrestrial UE is not suitable for the aerial UE when the drone is ascending or descending. On one hand, almost all of the transmission paths between the aerial UE and the base station are LOS paths; on the other hand, the distance between the aerial UE and the base station changes a little when the drone is vertically moving. At this time, if the vertical mobility of the drone UE is still dependent on the legacy mobility management, e.g., event A5, i.e., SpCell becomes worse than threshold1 and neighbour becomes better than threshold2, it might cause RLF due to too late handover.

We would give an example to explain this issue, and for convenience, we assume that the strength of the signal can be ignored at the edge of the beam. As shown in Fig.1, first, we can see that the drone UE enters the range (A, B) and it establishes a connection with gNB A. Then, the drone UE enters the range (B, C). Even though the aerial UE can receive the signal from gNB B, it is still connected to gNB A at this time. This is because the distance between the gNB A and the aerial UE changes only a little, making the signal strength change only slightly, resulting in the entering condition of event A5 not being fulfilled. Afterwards, the drone UE enters the range (C, D): at this time, RLF might occur. This is because the network does not handover the aerial UE from gNB A to gNB B in advance when the signal of gNB A at C point will decrease extremely quickly.

In order to handle the mentioned issue, we think the combination of the event A4 and the events H1/H2 is needed. In particular, if the height of the aerial UE exceeds a certain altitude and the quality of the neighbouring cell is better than a certain threshold, the network should handover the aerial UE to the target gNB when the aerial UE is ascending or descending.

Proposal 7: The combination of events A4 and H1/H2 should be considered for the NR UAV to handle vertical mobility.

2.3 Discussion on the numberofTriggeringCells

The measurement mechanism of LTE is different compared to NR due to the fact that in NR the base station uses beams to connect with aerial UEs. The UE needs to measure the beams to derive the cell quality, and the final measurement report is at cell level. In the last meeting, some companies suggested to introduce numberOfTriggeringBeams similar to numberOfTriggeringCells. We found that there were different opinions on the role of numberOfTriggeringBeams. Some companies said it was needed for mobility, and others said it was for measurement reporting. If it is for mobility, it is not needed due to the fact that it will delay the measurement report further, especially when numberOfTriggeringCells is confiugured. If it is for measurement reporting, it is also not needed because we already have the numberOfTriggeringCells to limit measurement reporting. Thus, the need for the introduction of the numberOfTriggeringBeams in NR should be clarified.

Proposal 8: Clarification on the intention to introduce the numberOfTriggeringBeams for NR UAV is needed.

For the leaving condition, there is a parameter reportOnLeave defined for each measurement report event in LTE, which can be set as True or False. If we set reportOnLeave as True, any cell in the cellsTriggeredList that fulfills the leaving condition, e.g., RSRP is lower than threshold, will trigger the UE to send MR to the gNB. It is not reasonable that the aerial UE sends the Measurement Report (MR) “mainly” relying on reportOnLeave. For example, let’s assume that the numberOfTriggeringCells is 3. When the numberOfTriggeringCells equals 3, the UE sends the MR to the NW. At this moment, we assume that cells 1, 2, and 3 are in the cellsTriggeredList . After that, let’s assume that a new cell 4 is added to the cellsTriggeredList, i.e., we have cells 1, 2, 3, 4 in the cellsTriggeredList. Based on the LTE mechanism, the UE would not send MR to the NW after cell 4 is added to the cellsTriggeredList. However, if the NW sets the reportOnLeave as true, the UE will send MR to the NW when cell 4 leaves, even though a MR of cell 4 was never sent to the NW. Obviously that is not reasonable, because it is an overhead with no use, that on the contrary could possibly confuse the network. Thus, if the leaving cells have never been reported to the NW beforehand, a MR should not be reported to the NW even if reportOnLeave is set to true.

Besides, some other views in the last meeting were proposed, i.e. to apply multi-cell trigger mechanism to the leaving condition. This mechanism implies  that the drone UE that would like to send the MR to the network should instead wait for a number of cells to leave the cellsTriggeredList before doing so. We think that this mechanism will introduce a new issue, i.e. the network may handover the drone UE to the wrong cell if the aerial UE waits for a certain number of cells to leave the cellsTriggeredList. For example, we assume that cells 1, 2, 3, and 4 are in the cellsTriggeredList, and the drone UE sends the MR should wait for two cells to leave the cellsTriggeredList. At this moment, if cell 3 leaves the list, the drone UE would not send the MR to the network. Unfortunately, handover failure will occur if the network decides to handover the drone UE to cell 3 based on the previous MR.

Proposal 9: The UE sends the MR to the NW only when the cell which is leaving the cellsTriggeredList, has been reported to the NW beforehand.

- Conclusion

Observation 1: In LTE, waypoints and timestamps that are included in the flight path are indicated by the IE LocationInfo and the IE AbsoluteTimeInfo, respectively.

Proposal 1: Waypoints and the time information for NR UAV that are included in the flight path can be indicated by the IE commonLocationInfo and the IE AbsoluteTimeInfo, respectively.

Proposal 2: Besides waypoints and timestamps, the velocity corresponding to the waypoints can be included in the flight path report.

Proposal 3: The UAV should indicate to the network if a flight path update is available.

Proposal 4: The flight path update available indication information can be included in the UEAssistanceInformation message.

Proposal 5: The NW can follow the LTE mechanism to obtain the updated flight path, i.e., the UEInformationRequest and UEInfomationResponse.

Proposal 6: A height adaptive TTT should be considered for NR UAV.

Proposal 7: The combination of events A4 and H1/H2 should be considered for the NR UAV to handle vertical mobility.

Proposal 8: Clarification on the intention to introduce the numberOfTriggeringBeams for NR UAV is needed.

Proposal 9: The UE sends the MR to the NW only when the cell which is leaving the cellsTriggeredList, has been reported to the NW beforehand.

- Reference

- R2_119bis-e_Skeleton_v3

- R2-1708544 Analysis and Simulation of Handover Issues for Drones Huawei, HiSilicon