3GPP TSG-RAN WG2 Meeting #120	 										    R2-2212654

Toulouse, France, 14th - 18th November 2022

Source: 	Xiaomi

Title:  	Discussion on RRC configurations of LTM

Agenda Item:	8.4.2.2 RRC

Document for:	Discussion and Decision

Introduction

According to RAN2#119-e and RAN2#119bis-e meeting discussion [1] [2], we consider that the following agreements could have impact on RRC for LTM:

RAN2#119-e： Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios). The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. ICBM is one scenario considered for L1L2 mobility, but is not the only one, and is not a prerequisite for using L1L2 mobility. RAN2 to consider preparation of target cell configurations capable of dynamic switching without need for full configuration. R2 will initially focus on PCell mobility. R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - - b) FFS the target PCell is a current SCell - - c) FFS the target SCell is the current PCell. DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS). Current options on the table: to configure a L1/L2 inter-cell mobility candidate cell: - - a. One RRCReconfiguration message for candidate target cell - - b. One CellGroupConfig IE for each candidate target cell - - c. One SpCellConfig IE for each candidate target cell RAN2#119bis-e： No security update support in Rel-18 with L1/L2 based mobility. FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration. FFS if this need to be specified. For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command: - MAC/RLC reset (when configured) - RF retuning (e.g. needed for inter-frequency), baseband retuning R2 assumes that the following items may be discussed by RAN1 and RAN4 (and may be scenario specific): - - Whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. R2 assumes this is feasible at least for the case that the target cell is already an active serving cell. - - Whether to support of performing TRS tracking and CSI measurement of candidate/target cell before/by cell switch command L1L2 based mobility supports the following CA scenarios: - PCell change without SCell change - PCell change with SCell change Support NR-DC scenario in L1L2 based mobility, at least for the PSCell change without MN involvement case, i.e. intra-SN. A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. - a. Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration - b. Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration RAN2 to use “LTM” as term for the L1/L2-triggered mobility. Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between. RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release). For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration. For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates. FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

In this contribution, we provide our understandings for RRC modelling and other RRC aspects of LTM.

Discussion

In RAN2#119bis-e meeting, the following agreement has been reached for the RRC modelling [2]:

RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details.

- a.	Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration

- b.	Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

For Model 1, the RRCReconfiguration message needs 3 levels in order to provide a candidate cell configuration:

- Message level configuration: RRCReconfiguration

  - Cell group level configuration: masterCellGroup/secondaryCellGroup containing CellGroupConfig

    - Cell level configuration: spCellConfig

For Model 2, the CellGroupConfig configuration needs 2 levels in order to provide a candidate cell configuration:

- Cell group level configuration: CellGroupConfig

  - Cell level configuration: spCellConfig

According to the email discussion of [Post119-e][048][feMob] Candidate target configurations for L1/L2 mobility (Ericsson) [3], pros and cons for Model 1 and Model 2 are as follows:

| Model | Pros | Cons |
| --- | --- | --- |
| Model 1 | - Full flexibility - Support of all targeted scenarios - Similarities with the existing CHO framework | - Since only intra-CU scenario is considered, there may be no need to provide all configurations and field within the RRCReconfiguration message. - Existing RRC procedures may heavily impacted (specification efforts may not be minimal). - Delta signalling may be needed (and needs to be discussed how to achieve it). - Potentially longer latency due to the execution of some RRC procedures (e.g., radio bearers, security, L1/L2 processing). |
| Model 2 | - Support for all targeted scenarios - Smaller signalling overhead compared to e.g., model 1. - Potentially reduced interruption time due to less time spent by the UE to execute non-necessary RRC procedures. | - How to perform L2 reset needs to be clarified - A new procedure for L1/L2 mobility may be needed (but some companies do not consider this necessarily a con). - One CellGroupConfig for each L1/L2 mobility target configuration - Configuration outside the CellGroupConfig may require a subsequent RRCReconfiguration message after the switch has happened. - Delta signalling may be needed (and needs to be discussed how to achieve it). |

Since the two Models are able to support all targeted scenarios, we think that RAN2 should consider the extra overheads or efforts spent on each model, while selecting the RRC model for the candidate configuration.

As given in the above table, Model 2 requires less signalling overhead, as lots of fields within the RRCReconfiguration message are not needed for the cell switching. And Model 2 would also provide less interruption.

Regarding the extra drawbacks of “a new procedure for L1/L2 mobility” and “clarification for L2 reset” listed for Model 2, we think that those are essential standard efforts also required for Model 1. As the L1/L2 mobility for cell switching is different from the legacy mobility procedure (e.g. legacy handover and legacy CHO), RAN2 would anyway need to define new procedures for the L1/L2 mobility for cell switching and discuss whether L2 resetting can be avoided, for both Model 1 and Model 2. Regarding the drawback of using extra signalling for providing configurations outside the CellGroupConfig, we think that the configurations (e.g. measConfig) outside of the CellGroupConfig in Model 1 are not specific for cell switching. Since the configurations (e.g. measConfig) outside of the CellGroupConfig are not associated to a specific candidate configuration, the gNB can provide those configurations (e.g. measConfig), while providing the candidate cell configurations.

Regarding the extra standard efforts for Model 1, when the UE applies the RRCReconfiguration message at cell switching, the UE would need to apply the RRC reconfiguration procedure. Then RAN2 needs to discuss which steps are required or not. This would cause lots of standard effort. For example, if the UE follow the legacy handover procedure for Model 1, we need to consider how to deactivate one candidate configuration and activate another candidate configuration, and this would require new activation/deactivation procedures in both RRC and MAC. For Model 2, since the candidate configuration is at the cell group level, the Rel-17 SCG activation/deactivation procedure can be reused for SCG candidate configurations, and the Rel-17 SCG activation/deactivation procedure can be used as the baseline for the MCG candidate configuration.

Observation 1: For Model 1, new activation/deactivation procedure for candidate configuration at the message level may be needed in both RRC and MAC.

Observation 2: For Model 2, the Rel-17 SCG activation/deactivation procedure can be used as the baseline for MCG candidate configuration, and the Rel-17 SCG activation/deactivation procedure can be reused for SCG candidate configurations.

Thus, we consider that Model 2 should be selected for LTM.

Proposal 1: Model 2 (i.e. one CellGroupConfig IE for each candidate target configuration) is selected for LTM.

For RRC configurations of LTM, the following RRC aspects also need to be discussed:

- When to perform ASN.1 decoding and validity/compliance check of candidate cell configuration.

- Whether some optimization is needed for candidate configuration modification/release.

- Whether the reference configuration for delta configuration is a separate reference configuration or e.g. the current configuration.

In previous meeting, the first issue has been discussed, some companies think ASN.1 decoding and validity/compliance check are performed upon reception of the candidate cells configuration. Others prefer it should be left for UE implementation.

If UE performs ASN.1 decoding upon reception of the cell switch command, it will result in extra interruption for cell switch. Hence, UE should perform ASN.1 decoding of candidate cell configuration upon reception of the candidate cells configuration. In general, no specification change is need for ASN.1 decoding.

Proposal 2: UE performs ASN.1 decoding of candidate cell configuration upon reception of the candidate cells configuration. No specification change is needed.

For CHO and CPAC, it is up to UE implementation whether the compliance check for an RRCReconfiguration received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO, CPA and CPC execution (when the message is required to be applied) [4].

Because UE can perform compliance check and apply the candidate cell configuration simultaneously, there is no extra interruption in the processing. Therefore, the similar principle for conditional reconfiguration can be reused for LTM. It is up to UE implementation whether to perform compliance check of candidate cell configuration upon the reception of the candidate cells configuration or upon cell switch.

Proposal 3: It is up to UE implementation whether the compliance check for candidate cell configuration is performed upon the reception of the candidate cell configuration or upon cell switch.

For the second issue, RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release) in last meeting. Some optimizations have be provided, e.g. UE autonomously releases candidate cell configuration after cell switch or based on a timer/conditions. UE-based methods for release might cause that UE and network have different understanding of candidate cell configuration stored in UE. Hence, RAN2 should confirm that candidate cell configuration can only be modified / released by Network.

Proposal 4: RAN2 should confirm that candidate cell configuration can only be modified / released by Network.

In last meeting, RAN2 agreed to support that candidate configurations are delta configurations on top of a reference configuration. However, there is no conclusion for the definition of reference configuration and following options have been captured as FFS:

Option 1: The reference configuration is the current configuration

Option 2: The reference configuration is a separate reference configuration

For Option 1, current configuration may carry different explanations, e.g. the current configuration is the serving cell configuration when UE received the cell switch command (Configuration A) or the serving cell configuration when UE received the delta configuration (Configuration B). Similar to legacy handover, if the reference configuration is Configuration A, UE can apply the candidate configuration directly. However, Configuration A is changing after cell switch. So, to support subsequent LTM, network might need to update all candidate cell configuration to match the new reference configuration. It is inconsistent with the criterion that subsequent LTM is performed between candidates without RRC reconfiguration. For Configuration B, it can be used to support the delta configuration in LTM. However, it is required that the UE store a long list of Configuration B to support delta configuration.

For Option 2, UE only needs to store the separate reference configuration which is provided by network, all candidate configurations are delta configurations on top of the separate reference configuration. Hence, Option 2 may be a better way. To reduce signalling overhead, the initial source cell configuration can be the separate reference configuration.

Proposal 5: The reference configuration is a separate reference configuration for all candidate configurations.

Conclusions

According to the analysis given above, we have the following observations and proposal:

Observation 1: For Model 1, new activation/deactivation procedure for candidate configuration at the message level may be needed in both RRC and MAC.

Observation 2: For Model 2, the Rel-17 SCG activation/deactivation procedure can be used as the baseline for MCG candidate configuration, and the Rel-17 SCG activation/deactivation procedure can be reused for SCG candidate configurations.

Proposal 1: Model 2 (i.e. one CellGroupConfig IE for each candidate target configuration) is selected for LTM.

Proposal 2: UE performs ASN.1 decoding of candidate cell configuration upon reception of the candidate cells configuration. No specification change is needed.

Proposal 3: It is up to UE implementation whether the compliance check for candidate cell configuration is performed upon the reception of the candidate cell configuration or upon cell switch.

Proposal 4: RAN2 should confirm that candidate cell configuration can only be modified / released by Network.

Proposal 5: The reference configuration is a separate reference configuration for all candidate configurations.

Reference

[1] R2-2209301, “Report of 3GPP TSG RAN WG2 meeting #119-e”

[2] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #119bis-e”

[3] R2-2210329, Ericsson, “[Post119-e][048][feMob] Candidate target configurations for L1/L2 mobility”

[4] TS 38.331 V17.2.0, 3GPP, “NR; Radio Resource Control (RRC) protocol specification”