3GPP TSG-RAN WG2 Meeting #120		R2-2212670

14th November – 19th November 2022

Source:	Qualcomm Incorporated

Title:	Discussion on SON/MDT enhancements for Non-Public Networks

Agenda Item:	8.13.7 SON/MDT enhancements for Non-Public Networks

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PSCell change report

- Successful Handover Report (e.g., inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

This paper will discuss SON/MDT enhancements for non-public networks further.

# Discussion

- SON/MDT enhancements for NPN

3GPP in Rel-16 introduced support for Non-Public Networks (NPN), including SNPN and PNI-NPN. To manage an SNPN, the standalone SNPN management system needs a dedicated NPN identifier. The combination of a PLMN ID and Network Identifier (NID) is used to uniquely identify an SNPN.

SON/MDT has been so far only supported for PLMNs. In RAN2#119bis-e [2], RAN2 made the following agreements on SON/MDT enhancements for NPN:

Agreements:

1	SNPN ID (e.g., NID ID) checking is needed before sending the availability indication for the corresponding SON and MDT report. The details can be discussed case by case. FFS PNI-NPN ID checking.

2	Include the NPN ID into SON/MDT report, whether SNPN ID or PNI-NPN ID-related info should be included can be discussed per use case.

3	RAN2 prioritizes the use cases of RLF report and logged MDT enhancement for NPN.

2.1.1 	SON/MDT enhancement for PNI-NPN

Note that PN and PNI-NPN cells may be deployed by the same operator. Therefore, for PNI-NPN SON/MDT report, the SON/MDT reports may be retrieved by both CAG and non-CAG cells if the registered PLMN of the cell lies within the PLMN identity list in the UE variable for SON/MDT report.

Observation 1: PN and PNI-NPN cells may be deployed by the same operator.

Proposal 1: CAG ID checking is not required before sending the availability indication at the UE before reporting SON/MDT reports.

2.1.1.1	RLF report enhancements for PNI-NPN

To uniquely identify the cell identities in the SON/MDT report for PLMN, UE reports global cell identity (i.e., cell identity and PLMN identity) and optionally the tracking area code. UE uses CGI-Info-Logging to report the global cell identity. We can have similar solutions for reporting the global cell identity of NPN cells. To represent the global cell identity of a PNI-NPN cell, UE can include the CAG-ID in the existing CGI-Info-Logging.

Observation 2: For the unique identification of cell identity in the PLMN, UE reports global cell identity (i.e., cell identity and PLMN identity) and optionally the tracking area code in CGI-Info-Logging.

Proposal 2: To represent the global cell identity of a PNI-NPN cell, UE can include the CAG-ID in the existing CGI-Info-Logging.

2.1.1.2	Logged MDT report enhancements for PNI-NPN

In rel-16/rel-17, logged MDT report is supported for PN. For configuring, logged MDT at the UE, the network sends LoggedMeasurementConfiguration which may include a cell list or a frequency list for area-specific filtering when logging the measurement. In rel-18, as the PNI-NPN cells are deployed by the same PN operator, the network can configure UE to measure and report both CAG and non-CAG cells. The network may have the following options to configure logged MDT measurement logging for CAG and Non-CAG cells,

- PLMN-wide measurement logging, irrespective of CAG or Non-CAG cells

- PLMN-wide measurement logging on a subset of CAG and Non-CAG cell/frequency list

- Measurement logging only on CAG cell

- Measurement logging on a subset of CAG cell/frequency list, but not from non-CAG

- Measurement logging only on non-CAG cell

- Measurement logging on a subset of non-CAG cell/frequency list, but not from CAG

Observation 3: The network may have the following options to configure logged MDT measurement logging for CAG and Non-CAG cells,

- PLMN-wide measurement logging, irrespective of CAG or Non-CAG cells

- PLMN-wide measurement logging on a subset of CAG and Non-CAG cell/frequency list

- Measurement logging only on CAG cell

- Measurement logging on a subset of CAG cell/frequency list, but not from non-CAG

- Measurement logging only on non-CAG cell

- Measurement logging on a subset of non-CAG cell/frequency list, but not from CAG

Observation 4: Rel-16/rel-17 logged MDT supports measurement logging on non-CAG cells, and optionally configuration for measurement logging on a subset of non-CAG cell and/or frequency lists.

Proposal 3: In Rel-18, PLMN-wide measurement logging (irrespective of CAG or Non-CAG cells) is supported by existing LoggedMeasurementConfiguration.

Proposal 4: In rel-18, for supporting configuration for measurement logging on a subset of CAG and non-CAG cell and/or frequency lists, the area configuration may include both CAG and non-CAG specific cells and interFrequencyTarget list.

Proposal 5: In rel-18, for supporting measurement logging on only CAG-Cell, an indication can be used in area configuration.

Proposal 6: In rel-18, for supporting configuration for measurement logging on a subset of CAG cells and/or frequency lists, the area configuration may include only CAG-specific cell and interFrequencyTarget list.

Proposal 7: in rel-18, for supporting measurement logging on only non-CAG-Cell, an indication can be used in area configuration.

Proposal 8: In rel-18, for supporting configuration for measurement logging on a subset of CAG cells and/or frequency lists, the area configuration may include only non-CAG specific cell and interFrequencyTarget list.

Proposal 9: Maximum number of cells remains 32 irrespective of whether the network configures CAG only, non-CAG only, or both CAG and non-CAG cells in area configuration.

Proposal 10: Maximum number of cells in InterFreqTargetInfo remains 32 irrespective of whether the network configures CAG only, non-CAG only, or both CAG and non-CAG cells in area configuration.

Proposal 11: Maximum number TrackingAreaCode and TrackingAreaIdentityList remains at 8 irrespective of whether the network configures CAG only, non-CAG only, or both CAG and non-CAG cells in the area configuration.

2.1.2 	SON/MDT enhancements for SNPN

2.1.2.1	RLF report enhancements for SNPN

To represent the global cell identity of an SNPN cell, UE can include the NID in the existing CGI-Info-Logging

Proposal 12: To represent the global cell identity of an SNPN cell, UE can include the NID in the existing CGI-Info-Logging.

2.1.2.2	Logged MDT report enhancements for SNPN

As the SNPN operator may be different operators than PN, therefore, both PN and SNPN cell measurements cannot be configured together. Furthermore, measurements from one SNPN cannot be reported to another SNPN, therefore, a single SNPN identity is configured for measurement logging in the LoggedMeasurementConfiguration.

Proposal 13: A single SNPN is configured in LoggedMeasurementConfiguration for SNPN.

Proposal 14: UE will record SON/MDT reports only if SNPN check holds. UE suspends logging if the registered NPN is not within the SNPN-identity store at UE.

Proposal 15: UE suspends logging the measurement if UE moves to PN.

Proposal 16: UE clears SNPN logged MDT report and configuration if a new configuration is received from another SNPN or PN.

# Conclusion

Observation 1: PN and PNI-NPN cells may be deployed by the same operator.

Proposal 1: CAG ID checking is not required before sending the availability indication at the UE before reporting SON/MDT reports.

Observation 2: For the unique identification of cell identity in the PLMN, UE reports global cell identity (i.e., cell identity and PLMN identity) and optionally the tracking area code in CGI-Info-Logging.

Proposal 2: To represent the global cell identity of a PNI-NPN cell, UE can include the CAG-ID in the existing CGI-Info-Logging.

Observation 3: The network may have the following options to configure logged MDT measurement logging for CAG and Non-CAG cells,

- PLMN-wide measurement logging, irrespective of CAG or Non-CAG cells

- PLMN-wide measurement logging on a subset of CAG and Non-CAG cell/frequency list

- Measurement logging only on CAG cell

- Measurement logging on a subset of CAG cell/frequency list, but not from non-CAG

- Measurement logging only on non-CAG cell

- Measurement logging on a subset of non-CAG cell/frequency list, but not from CAG

Observation 4: Rel-16/rel-17 logged MDT supports measurement logging on non-CAG cells, and optionally configuration for measurement logging on a subset of non-CAG cell and/or frequency lists.

Proposal 3: In Rel-18, PLMN-wide measurement logging (irrespective of CAG or Non-CAG cells) is supported by existing LoggedMeasurementConfiguration.

Proposal 4: In rel-18, for supporting configuration for measurement logging on a subset of CAG and non-CAG cell and/or frequency lists, the area configuration may include both CAG and non-CAG specific cells and interFrequencyTarget list.

Proposal 5: In rel-18, for supporting measurement logging on only CAG-Cell, an indication can be used in area configuration.

Proposal 6: In rel-18, for supporting configuration for measurement logging on a subset of CAG cells and/or frequency lists, the area configuration may include only CAG-specific cell and interFrequencyTarget list.

Proposal 7: in rel-18, for supporting measurement logging on only non-CAG-Cell, an indication can be used in area configuration.

Proposal 8: In rel-18, for supporting configuration for measurement logging on a subset of CAG cells and/or frequency lists, the area configuration may include only non-CAG specific cell and interFrequencyTarget list.

Proposal 9: Maximum number of cells remains 32 irrespective of whether the network configures CAG only, non-CAG only, or both CAG and non-CAG cells in area configuration.

Proposal 10: Maximum number of cells in InterFreqTargetInfo remains 32 irrespective of whether the network configures CAG only, non-CAG only, or both CAG and non-CAG cells in area configuration.

Proposal 11: Maximum number TrackingAreaCode and TrackingAreaIdentityList remains at 8 irrespective of whether the network configures CAG only, non-CAG only, or both CAG and non-CAG cells in the area configuration.

Proposal 12: To represent the global cell identity of an SNPN cell, UE can include the NID in the existing CGI-Info-Logging.

Proposal 13: A single SNPN is configured in LoggedMeasurementConfiguration for SNPN.

Proposal 14: UE will record SON/MDT reports only if SNPN check holds. UE suspends logging if the registered NPN is not within the SNPN-identity store at UE.

Proposal 15: UE suspends logging the measurement if UE moves to PN.

Proposal 16: UE clears SNPN logged MDT report and configuration if a new configuration is received from another SNPN or PN.

# Reference

[1] RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

[2] RAN2#119bis-emeeting report

[3] TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17)”

[4] TS 37.340, “Multi-connectivity; Stage 2 (Release 17)”

[5] TS 38.300, “NR and NG-RAN Overall description; Stage 2 (Release 17)”