# 3GPP TSG RAN WG2 Meeting #120                     	    	    R2-2212693 
Toulouse, France, 14th – 18th November 2022

Agenda item:	6.15.3

Source:	Qualcomm Incorporated

Title:	Discussion on remaining issues

WID/SID:	NR_SL_enh-Core – Release 17

Document for:	Discussion and Decision

# Introduction

In this contribution, we further discuss remaining issues.

# Discussion

## IUC with non-preferred resource set

RAN1 made agreement on the reply LS to RAN2 on IUC with Non-preferred Resource Set at RAN1 #110bis-e meeting.

Agreement The draft LS reply in R1-2210482 is endorsed. Final LS reply to RAN2 LS in R1-220578 on IUC with non-preferred resource set in R1-2210582

R1-2210582 contains the following responses from RAN1.

As it’s confirmed by RAN1 that it’s a possible scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g., UE B performs mode 2 random resource selection, etc.

RAN1 cannot conclude whether or not UE-B performs resource exclusion based on non-preferred resource set and asks RAN2 to take this into account for RAN2’s future works on this possible scenario.

When UE B receives IUC Scheme 1 non-preferred resource set from UE A and UE B does not perform sensing in the resource pool associated with the non-preferred resource set (e.g., UE B performs mode 2 random resource selection, etc.), there is no candidate resources provided from PHY and MAC has to select resources randomly from the resource pool. In this case, MAC may consider excluding the non-preferred resource set received from UE A.

Proposal 1. RAN2 may discuss if the non-preferred resource set is excluded by MAC for resource selection when UE B receives IUC Scheme 1 non-preferred resource set from UE A and UE B does not perform sensing in the resource pool associated with the non-preferred resource set.

## Default CBR for Release 16/17 normal pool

RAN2 made the following agreement for default CBR at RAN2 #119bis-e meeting [2].

For case 3, we don’t see a good reason to treat Release 16 normal pool and Release 17 normal pool differently. Therefore, it should be consistent between Release 16 normal pool and Release 17 normal pool with supporting or not supporting default CBR.

Proposal 2. RAN2 may decide either supporting or not supporting default CBR for both Release 16 normal pool and Release 17 normal pool.

# Conclusion

In this contribution, we discussed the remaining issues, and concluded with the following proposals.

IUC with non-preferred resource set

Proposal 1. RAN2 may discuss if the non-preferred resource set is excluded by MAC for resource selection when UE B receives IUC Scheme 1 non-preferred resource set from UE A and UE B does not perform sensing in the resource pool associated with the non-preferred resource set.

Default CBR for Release 16/17 normal pool

Proposal 2. RAN2 may decide either supporting or not supporting default CBR for both Release 16 normal pool and Release 17 normal pool.

# References

[1] R1-2210582 “Reply LS to RAN2 on IUC with Non-preferred Resource Set”, RAN1 #110bis-e meeting, October 10th – 19th, 2022.

[2] RAN2 NR SL-enh session chair final notes, “RAN2-119bis-e_Rel17&18_NR SL-enh_EoM”, RAN2 #119bis-e meeting, October 10 – 19, 2022.