3GPP TSG-RAN WG2 Meeting #120	R2-2212727

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On coverage enhancements for NR NTN

WID/SID:	NR_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In the WID for Rel-18 NR NTN enhancement [1], the objective of coverage enhancements is defined, in which RAN2 is expected to study the need for NTN-specific coverage enhancements for Rel-18 on top of the coverage enhancement WI. Specifically, it is expected to support coverage enhancement for VoIP and low-data rate services for commercial handset terminals. For example, how to improve performance of low-rate VoNR codec including RAN protocol overhead reduction.

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). <omit some text> The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. <omit some text> Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the RAN2 study. - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec.

In this contribution, we would like to provide our view on how to support VoNR in NTN, as well as Msg3 coverage enhancements.

# 2	Discussion

## 2.1	VoIP Frame Aggregation

In RAN2-119bis meeting, RAN2 agreed that a UE may use application layer frame aggregation by implementation. While it is not clear whether RAN needs to know whether UE is using frame aggregation in the voice packet.

Agreements:

- RAN2 thinks a UE may use application layer frame aggregation by implementation (no RAN2 spec impacts). (RAN2 can further discuss whether RAN needs to know whether UE is using frame aggregation in the voice packet)

As indicated in [2], application layer frame aggregation can achieve 1.5dB of SNR gain (i.e., coverage extension) since it can reduce packet overhead (e.g., the RTP/PDCP/RLC/MAC headers can be reduced) and enable more than 20 PUSCH repetition when aggregation level >1.

Table 1: Coverage gain for application layer voice frame aggregation

| Codec (kbps) | Inter tx time (ms) | Application aggregation (frames/packet) | Voice Payload (bits) | Protocol overhead (bits) | TBS (bits) | Repetitions | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.75 | 20 | 1 | 96 | 80 | 176 | 16 | -5.7 |
| 4.75 | 40 | 2 | 192 | 80 | 272 | 32 | -7.2 |

To achieve the coverage gain, NW needs to know the aggregation level hence it can decide whether more than 20 PUSCH repetition can be scheduled for this UE.

Observation 1: NW can enable more than 20 PUSCH repetitions for VoIP packets if RAN knows the aggregation level information for the packets.

Furthermore, different aggregation level means different packets transmission periodicity and TBS requirement. For example,  assuming the packet aggregation level is 1/2/3, the required TBS to accommodate the payload and overhead should be 192/304/416 bits. Correspondingly, the packets transmission periodicity is 20/40/60 ms.

Table 2: TBS and packets transmission periodicity for application layer frame aggregation

| Codec (kbps) | Application layer aggregation | Inter tx time (ms) | Payload (bytes) | RoHC (bytes) | PDCP (bytes) | RLC (bytes) | MAC (bytes) | TBS (bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.75 | 1 | 20 | 14 | 5 | 2 | 1 | 2 | 192 |
| 4.75 | 2 | 40 | 14*2 | 5 | 2 | 1 | 2 | 304 |
| 4.75 | 3 | 60 | 14*3 | 5 | 2 | 1 | 2 | 416 |

The frame aggregation may impact the Configured Grant(CG) a lot due to the changed TBS requirement and packet transmission periodicity. For Configured Grant, the UE is pre-configured with the UL resource with fixed TBS periodically via RRC and/or single PDCCH grant. Generally, the fixed TBS and CG occasions are configured based on traffic properties. The ideal case is the traffic TBS requirement and periodicity are aligned with CG TBS and occasions hence packets can be delivered on time.

However, with variable frame aggregation, the TBS requirement and packet transmission periodicity of aggregated voice packets may not aligned with CG allocated fixed TBS and periodicity (since RAN has no idea when and whether frame aggregation will happen).  It may cause the allocated TBS cannot accommodate the traffic (which means additional dynamic scheduling should be scheduled by NW) and/or the allocated CG occasions are wasted due to no packets (or not enough payload) to be transmitted on the occasions . It may also cause the CG feature useless from PDCCH saving point of view due to additional PDCCH should be used for dynamic scheduling.

Observation 2: The Configured Grant cannot work efficiently (e.g., with PUSCH and/or PDCCH resource waste) if UE applies the frame aggregation while RAN is not aware that.

Based on above observations, we think RAN needs to know UE’s frame aggregation information in a voice packet.

Proposal 1: To support VoNR in NTN, RAN needs to know UE’s frame aggregation information in a voice packet.

## 2.2	VoIP codec adaptation

Per the agreement reached in RAN1-109 meeting, the VoIP coverage performance is evaluated with AMR codec 4.75 Kbps.  This means when UE is in a link budget limited situation (i.e., poor RF conditions), smallest codec data rate (with smallest TBS requirement) should be selected by UE to extend the coverage which can avoid packet RLC segmentation.

Agreement For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

However, according to TS 26.114, a UE offering speech communication may support AMR, AMR WB and EVS in which different codec modes are supported with different bitrate. A higher AMR voice codec rate may provide a higher-definition voice call and accordingly a better user experience. Hence, when UE’s radio conditions are better than before, UE may reselect a higher codec rate for a better user experience of the VoNR call. Similarly, when UE’s radio conditions are worse than before, UE may reselect a lower codec rate to extend coverage. For NTN, due to satellite movement, the UE experienced radio conditions may change frequently. So, codec bitrate adaptation based on radio conditions is mandatory.

Table 3: Codec modes with different codec bitrate for AMR and AMR-WB

|  | Codec mode with different codec bitrate (in kbps) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AMR | 4.75 | 5.15 | 5.9 | 6.7 | 7.4 | 7.95 | 10.2 | 12.2 |  |
| AMR-WB | 6.6 | 8.85 | 12.65 | 14.25 | 15.85 | 18.25 | 19.85 | 23.05 | 23.85 |

Observation 3: To support VoNR in NTN, the VoIP codec adaptation should be supported due to satellite movement.

In legacy, the voice codec selection and adjustment were discussed in enhancement of VoLTE in SID [3] and WID[4], RAN2 agreed that the RAN-assisted Codec adaptation mechanism in which NW can send codec bitrate recommendation to UE via DL MAC CE based on UE’s radio conditions and system load status. Furthermore, UE may query the recommended codec bitrate from NW via UL MAC CE. The same design was inherited in NR as specified in 38.321 section 5.18.10 and 6.1.3.20.

Observation 4: The RAN-assisted voice codec bitrate adaptation mechanism is supported in TN, in which NW can send codec bitrate recommendation to UE via DL MAC CE and UE may query the recommended codec bitrate from NW via UL MAC CE.

For NR NTN, according to WID[1], the VoIP performance is evaluated in LEO-1200km scenario. The legacy MAC-CE based codec bit rate adaptation may not work well since the UE-gNB RTT delay will be 42ms which is quite larger than TN. For example,  given UE generate VoIP packet every 20ms, the time distance between the point where UE wants to trigger the codec adaptation due to detected coverage change and the point where UE applies new codec can reach up to 80ms. As illustrated in Figure1, when the UE triggers codec rate increase (e.g., it may be triggered by UE’s detection of better RF condition or requested by peer UE of the call), UE has to query the recommended codec bitrate from NW via UL MAC CE and then wait DL RBR (Recommended Bit Rate) MAC CE to trigger further codec bitrate increase. The time distance between UE triggers the codec bitrate increase and UE applies new codec can reach up to 80ms. The time delay on application of higher codec bitrate means the delay to provide a better user experience (a higher-definition voice call) which is not good for system performance. The case seems happen frequently due to satellite movement in high speed (e.g. 7.5km/s) where the pathloss between UE and gNB will change accordingly.

Figure 1: Time delay of codec adaptation for bitrate increase

Observation 5: Due to high UE-gNB RTT, the legacy MAC-CE based codec bitrate adaptation may delay the voice codec adaptation.

In the study report TR36.750 for VoLTE enhancement SID [4], there are several candidate solutions captured. In our view, the proposed solutions can be the baseline for further enhancements for NTN. For example, the broadcast-based bit rate adaptation may need to be further studied for NTN to avoid high latency issue. In this approach, NW broadcast a mapping relationship between recommended bit rates and channel conditions via system information. When initiating VoIP call or during an on-going call, UE can determine the recommended bit rate corresponding to the measured channel conditions and negotiates with the peer UE based on SIP procedure. Obviously, this approach is helpful to save RTT delay in-between the MAC CE exchange.

Proposal 2: The VoIP codec adaptation mechanism should be enhanced for NR NTN.

## 2.3	Msg3 repetition for NTN

In Rel-17 NR NTN, it was agreed Msg3 repetition defined in Coverage Enhancement WI should be supported for NTN, while any further enhancement should be studied in Rel-18.

3. Msg3 repetition is supported in Rel-17 NTN. The text proposal in R2-2206207 is adopted as baseline and included in the TS 38.321 Rapporteur CR. No need to send an LS to RAN1 on this.

The PUSCH of Msg3 was found to be a coverage bottleneck for an NR system. For this reason, repetitions of Msg3 were introduced in Rel-17, to increase reliability of Msg3 detection and decoding. The signal repetition framework was kept invariant compared to PUSCH repetitions in RRC connected mode, but new signalling and methods for indication of the number of repetitions for Msg3 were specified.

The main agreements for enabling of Msg3 repetitions include:

- UE requests Msg3 repetitions if the measured SS-RSRP is below an RSRP threshold configured through SIB1

- Request occurs through choice of specific preamble and/or RO in Msg1

- gNB indicates the number of Msg3 repetitions by using two repurposed bits of the MCS field of the RAR UL grant carried in Msg2

The framework developed in Rel-17 seems to work fine for an NTN setting. A specific set of preambles or RO can be reserved and configured by gNB to allow UEs at low RSRP to communicate a request for Msg3 repetitions. However, the need for specific enhancements for optimization of Msg3 repetitions framework in NTN could be further investigated, especially considering specific NTN characteristics.

As mentioned above, Msg3 repetitions are requested and scheduled only if the SS-RSRP measured by the single UE is below a configured RSRP threshold. Typically, setting of such RSRP threshold is based on an estimate of the UL link budget when the DL RSRP is equal to the configured threshold. In other words, if the DL RSRP is below the configured threshold, the UE is likely to deliver an UL RSRP below the necessary RSRP (and SNR) for correct detection of the Msg3 content, hence such a UE may benefit from Msg3 repetitions.

In NTN, due to satellite movement, the average SNR (or DL RSRP) experienced by the single UEs strongly depends on the satellite position compared to Earth, with a larger average SNR when the satellite is at nadir (large elevation angles) and lower average SNR when the satellite is at horizon (small elevation angles). This means that, for a certain configured RSRP threshold for requesting of Msg3 repetitions, the probability that the DL RSRP of a UE is below the threshold is larger for satellite at small elevation angles, compared to satellite at larger elevation angles. Therefore, when the satellite is at small elevation angles, a higher request for Msg3 repetitions is to be expected and hence a larger probability of PRACH collisions for a fixed preamble and RO set configured for request of Msg3 repetitions.

Observation 6: Due to satellite movement, the probability of PRACH collision for a fixed preamble and RO set configured for requesting Msg3 repetitions is larger when the satellite is at small elevation angles.

Since the majority of the path loss will come from the satellite-to-earth propagation, with the elevation angle change, many UE may be “in similar conditions” at most of the time which may cause UEs to request Msg3 repetition at the same time. So, the risk of overloading the preamble and RO sets may increase the RACH collisions. A larger probability of PRACH collisions creates undesired delays in initial access that need to be avoided and addressed in Rel-18. Methods for configuration of a more dynamic set of resources for Msg1 transmissions requesting Msg3 repetitions could for example be discussed to alleviate the issue and maintain the collision probability of Msg1 invariant regardless of satellite position.

Proposal 3: RAN2 to discuss methods for configuration of PRACH resources for requesting of Msg3 repetitions for NR NTN.

## 2.4	Blind Msg3 retransmission for initial Msg3 transmission

For Msg3 coverage enhancement, an alternative solution is to support blind Msg3 retransmission. It is up to NW scheduling strategy on whether the blind retransmission will be scheduled for a UE based on e.g., the NW resource status and the UE’s service requirement for latency. Compared to the coverage enhancement based on Msg3 repetitions, blind Msg3 retransmission can be applied to all UEs without special UE capability requirement.

However, in Rel-17 NTN, blind Msg3 retransmission is supported for Msg3 retransmission while it is not supported for initial Msg3 transmission.

Agreement: Blind Msg3 retransmission is not supported for initial Msg3 transmission in Rel-17 NTN.

As explained in [5], due to long UE-gNB RTT, it was agreed to delay the start of ra-ContentionResolutionTimer by UE-gNB RTT after initial Msg3 transmission (in red in Figure2). Hence it is not possible for NW to schedule blind Msg3 retransmission for initial Msg3 transmission since UE will not monitor PDCCH during that period.

Figure 2: ra-ContentionResolutionTimer (re)start

Observation 7: Blind retransmission for initial Msg3 transmission is not supported due to the UE-gNB RTT delay before starting the ra-ContentionResolutionTimer.

In the email discussion [6], majority companies would like to support blind Msg3 retransmission for initial Msg3 transmission for NTN and some companies propose to study it in Rel-18 as part of coverage enhancement objective. But there are different options provided by companies on how to address the issue, for example:

- Option1: Do not stop the ra-ResponseWindow after initial RAR reception

- Option2: Start ra-ContentionResolutionTimer immediately after initial Msg3 transmission

- Option3: Introduce a new timer (and short timer) for blind Msg3 retransmission after initial Msg3 transmission

- Option4: Whether the UE will additionally monitor for a blind Msg3 retransmission grant may be configurable by the network

In our understanding, it is reasonable to further investigate the Pros and Cons for different options and support blind Msg3 retransmissions for initial Msg3 transmission.

Proposal 4: RAN2 to address the issues on how to support blind Msg3 retransmission for initial Msg3 transmission for NR NTN.

# 3	Conclusion

This document has made the following observations:

Observation 1: NW can enable more than 20 PUSCH repetitions for VoIP packets if RAN knows the aggregation level information for the packets.

Observation 2: The Configured Grant cannot work efficiently (e.g., with PUSCH and/or PDCCH resource waste) if UE applies the frame aggregation while RAN is not aware that.

Observation 3: To support VoNR in NTN, the VoIP codec adaptation should be supported due to satellite movement.

Observation 4: The RAN-assisted voice codec bitrate adaptation mechanism is supported in TN, in which NW can send codec bitrate recommendation to UE via DL MAC CE and UE may query the recommended codec bitrate from NW via UL MAC CE.

Observation 5: Due to high UE-gNB RTT, the legacy MAC-CE based codec bitrate adaptation may delay the voice codec adaptation.

Observation 6: Due to satellite movement, the probability of PRACH collision for a fixed preamble and RO set configured for requesting Msg3 repetitions is larger when the satellite is at small elevation angles.

Observation 7: Blind retransmission for initial Msg3 transmission is not supported due to the UE-gNB RTT delay before starting the ra-ContentionResolutionTimer.

And proposed the following:

Proposal 1: To support VoNR in NTN, RAN needs to know UE’s frame aggregation information in a voice packet.

Proposal 2: The VoIP codec adaptation mechanism should be enhanced for NR NTN.

Proposal 3: RAN2 to discuss methods for configuration of PRACH resources for requesting of Msg3 repetitions for NR NTN.

Proposal 4: RAN2 to address the issues on how to support blind Msg3 retransmission for initial Msg3 transmission for NR NTN.

# Reference

[1] RP-222654 NR NTN (Non-Terrestrial Networks) enhancements

[2] R2-2209709 Frame aggregation for coverage enhancements, Qualcomm

[3] RP-160563 New SI proposal: Study on enhancement of VoLTE

[4] RP-161856 New Work Item on Voice and Video Enhancement for LTE

[5] R2-2208561 On Msg3 blind retransmission and UE behaviour upon validity timer expiry, Nokia, Nokia Shanghai Bell

[6] R2-2208751 Report of [AT119-e][101][NTN] UP corrections, Interdigital