3GPP TSG-RAN2 Meeting #120	R2-2212743

Nov 14th – 18th, 2022 Toulouse FR

Source: 			ZTE Corporation, Sanechips

Title:           	Further Consideration on the IDC FDM solutions

Agenda item:	8.10.2

Document for: 	Discussion and Decision

- Introduction

In the last meeting, some agreements were achieved for the IDC FDM solutions.

=> The Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP should be considered for the FDM enhancement in Rel.18. => The Adjacent channel interference between SN (NR) of MR-DC and non-3GPP should be considered for the FDM enhancement in Rel.18. => NE-DC is not considered; We will work on NR freq as SA NR case. => We will not consider the enhancements on E-UTRA freq for EN-DC scenario. FFS, on signaling details; => The IMD interference from simultaneous Tx in EN-DC to non-3GPP should be considered for the FDM enhancement in Rel.18. => The IMD interference from simultaneous Tx in NR-DC to non-3GPP should be considered for the FDM enhancement in Rel.18. Note: the solution (on freq granularity) for adjacent can be reused for IMD, we will not invent new solution on freq granularity for IMD. FFS on signaling details. => Granular indications of the affected NR frequency reported for IDC issue needs to consider both serving and non-serving frequency as in the legacy FDM solution.

About the candidate FDM solutions, there is also a post meeting email discussion. Besides, in the last meeting, there are also some discussions on MN-SN coordination and SRB3 related issues. In this paper, we’d like to share our further thinking on these issues.

- Discussion

In this chapter, we first discuss the candidate solutions based on the email discussion summary, then share our preliminary thinking on the MN-SN coordination, at last we will further analyze whether the SRB3 should be involved for the IDC reporting.

2.1 Candidate Solutions

According to the summary of the post email discussion, the solution 1/2/2a were preferred by majorities.

Proposal 1 The Rel-18 IDC solution should allow for more granular IDC indications both on serving and on non-serving frequencies. Proposal 2 Only one single new finer granularity report is introduced, that applies for both serving and non-serving frequencies. Proposal 3 For LTE, problematic frequencies are indicated by indicating measurement object IDs Proposal 4 RAN2 down select one of solution 1, 2 or 2a. Option 1: Central frequency + Bandwidth of the actual affected frequency range Option 2: Starting frequency + Ending frequency of the actual affected frequency range Option 2a: starting frequency + Bandwidth of the actual affected frequency range

However as commented by the most companies, the main concern of the option 1/2/2a is about the signaling overhead with below 4 reasons:

- 22 bits may be needed for each ARFCN;

- For one frequency, it may have multiple affected frequency ranges

- Each IMD IDC interference includes multiple affected frequency ranges

- The UE need to report IDC on both serving and non-serving frequency which may leads more frequency range combinations for the IMD IDC interference reporting

Observation 1: The main concern of the option 1/2/2a is about the signaling overhead with below 4 reasons:

- 22 bits maybe needed for each ARFCN;

- For one frequency, it may have multiple affected frequency ranges

- Each IMD IDC interference includes multiple affected frequency ranges

- The UE need to report IDC on both serving and non-serving frequencies which may leads more frequency range combinations for the IMD IDC interference reporting

Based on this observation, RAN2 shall consider to reduce the signaling overhead from 2 directions:

- Direction 1: For the non-serving cell, only report adjacent IDC interference;

- Direction 2: Do some signaling optimization to reduce the signaling overhead.

With direction 1, it doesn’t need to report IMD interference that involve the non-serving frequencies, so it would reduce the signaling overhead significantly, however it seems against the original intention of the option 1/2/2a. Thus, RAN2 can take the direction 2 as a start point, e.g. reduce the signaling bits of the ARFCN or reduce the frequency range repetition in the different IMD frequency range combinations.

Proposal 1: RAN2 to discuss how to reduce the signaling overhead from 2 directions:

- Direction 1: For the non-serving cell, only report adjacent IDC interference

- Direction 2: Do some signaling optimization to reduce the signaling overhead.

Proposal 1a: Take the Direction 2 (Do some signaling optimization to reduce the signaling overhead) as start point.

2.2 MN-SN Coordination

As in legacy, the MN would indicate the MR-DC related IDC interference information to the SN, as below

CG-Config: The SN provides the frequencies of candidate serving cells to the MN, then the MN includes it in the other config together with the MN candidate frequencies. CandidateServingFreqListNR ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueNR candidateServingFreqListNR Indicates frequencies of candidate serving cells for In-Device Co-exsistence Indication (see TS 36.331 [10]).. CG-ConfigInfo: The MN indicates the MR-DC related IDC interference information to the SN MRDC-AssistanceInfo ::= SEQUENCE { affectedCarrierFreqCombInfoListMRDC SEQUENCE (SIZE (1..maxNrofCombIDC)) OF AffectedCarrierFreqCombInfoMRDC, ...} AffectedCarrierFreqCombInfoMRDC ::= SEQUENCE { victimSystemType VictimSystemType, interferenceDirectionMRDC ENUMERATED {eutra-nr, nr, other, utra-nr-other, nr-other, spare3, spare2, spare1}, affectedCarrierFreqCombMRDC SEQUENCE { affectedCarrierFreqCombEUTRA AffectedCarrierFreqCombEUTRA OPTIONAL, affectedCarrierFreqCombNR AffectedCarrierFreqCombNR } OPTIONAL}

Then in the legacy capability coordination information, the MN would also indicate the selectedBandEntriesMNList,

ConfigRestrictInfoSCG ::= SEQUENCE { allowedBC-ListMRDC BandCombinationInfoList ...} BandCombinationInfoList ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationInfo BandCombinationInfo ::= SEQUENCE { bandCombinationIndex BandCombinationIndex, allowedFeatureSetsList SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex } selectedBandEntriesMNList SEQUENCE (SIZE (1..maxBandComb)) OF SelectedBandEntriesMN SelectedBandEntriesMN ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandEntryIndex

As an implementation method, the SN can further determine the available band entries based on the MR-DC IDC interference information and the selectedBandEntriesMNList, For example, if the UE report an IDC that caused by f1 and f2, and the f1 was included in the selectedBandEntriesMNList, the SN will not include the cells on the f2 in the SCG.

Observation 2: For the legacy carrier based scheme, the SN can further determine the available band entries based on the MR-DC IDC interference information and the selectedBandEntriesMNList.

In R18, if the UE report more granular FDM information, as analyzed in [1], the MN and SN may also need to take these granular FDM information into consideration.

For example MN: F1: PRB Range1, PRB Range2 SN: F2: PRB Range1, PRB Range2 IMD 1: F1 PRB Range1+ F2 PRB Range2 IMD 2: F1 PRB Range2 + F2 PRB Range1 If the MN indicates the SN that F1 was selected (e.g. in the selectedBandEntriesMNList), but doesn’t indicate the PRB Range based information to the SN, the SN would take all frequency ranges on the F2 as invalid.

Thus, RAN2 need to have further discussion on whether and how to do FDM enhancement on the MN-SN coordination.

Proposal 2: Ran2 to discuss whether and how to do FDM enhancement on the MN-SN coordination.

However, the detail solution also depends on which granularity would be selected, thus this can be done once the reported granularity was determined.

2.3 SRB3 Involved or Not’

About the SRB3, it can be discussed with lower priority, e.g. once the reported granularity was determined. In this chapter, we give us preliminary thinking on it for the NR-DC and EN-DC respectively.

For the NR-DC, normally, the SN would indicate a candidate frequency list to the MN, then the MN would send the candidate frequencies from both the MN and SN side to the UE. The UE would report both adjacent IDC interference and the IMD interference among these candidate frequencies to the MN. With SRB3 involved, it means that the SN can configure and receive the IDC report independently. However, it also means that there would be overlap with the IDC reporting to the MN especially for the IMD IDC reporting, e.g. IMD IDC among SN and MN. To avoid such kind of overlap for the NR-DC, there is no need to support SRB3 for the IDC reporting.

Proposal 3: For the NR-DC, no need to support SRB3 for the IDC reporting.

For the EN-DC, it’s a little different from the NR-NR DC. With SRB3, the SCG can require the UE to report the IDC among the NR frequencies separately. While the IMD interference among the Eutra and NR frequencies can be reported to the MN. The details can be further considered.

Proposal 4: For the EN-DC, Ran2 to discuss whether to support SRB3 for the IDC reporting or not.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: The main concern of the option 1/2/2a is about the signaling overhead with below 4 reasons:

- 22 bits maybe needed for each ARFCN;

- For one frequency, it may have multiple affected frequency ranges

- Each IMD IDC interference includes multiple affected frequency ranges

- The UE need to report IDC on both serving and non-serving frequencies which may leads more frequency range combinations for the IMD IDC interference reporting

Proposal 1: RAN2 to discuss how to reduce the signaling overhead from 2 directions:

- Direction 1: For the non-serving cell, only report adjacent IDC interference;

- Direction 2: Do some signaling optimization to reduce the signaling overhead.

Proposal 1a: Take the Direction 2 (Do some signaling optimization to reduce the signaling overhead) as start point.

Observation 2: For the legacy carrier based scheme, the SN can further determine the available band entries based on the MR-DC IDC interference information and the selectedBandEntriesMNList.

Proposal 2: Ran2 to discuss whether and how to do FDM enhancement on the MN-SN coordination.

Proposal 3: For the NR-DC, no need to support SRB3 for the IDC reporting.

Proposal 4: For the EN-DC, Ran2 to discuss whether to support SRB3 for the IDC reporting or not.

- Reference

[1] R2-2207844	Discussion on FDM solution for in-device co-existence interference avoidance	Samsung