3GPP TSG-RAN WG2 Meeting #120                                   R2-2212760

Toulouse, France, November, 2022

Agenda Item	: 8.7.2 (NR_NTN_enh-Core)

Source	: LG Electronics Inc.

Title	: Discussion on the coverage enhancement in NTN

Document for	: Discussion and Decision

Introduction

In RAN2#119bis e-meeting, how to reduce the L2 header was discussed to enhance the coverage enhancement and the following agreements were made.

1. RAN2 thinks a UE may use application layer frame aggregation by implementation (no RAN2 spec impacts). (RAN2 can further discuss whether RAN needs to know whether UE is using frame aggregation in the voice packet) 2. RAN2 understands that it is up to network implementation to decide whether to configure SDAP header and integrity protection for a VoNR DRB to reduce the protocol overhead (no RAN2 spec impacts)

In this contribution, we show our views on whether the L2 header reduction should be studied or not.

Discussion

In order to discuss the L2 header reduction, we should look into each layer.

1. PDCP header

The PDCP header is composed of the D/C field and the PDCP SN field. The D/C field is used to indicate whether the PDCP PDU is the PDCP control PDU or the PDCP data PDU. Thus, the D/C field shall be included in the PDCP header.

The PDCP COUNT value is used to perform the security operation, i.e., integrity protection and ciphering. Considering that the PDCP COUNT value is composed of the HFN and the PDCP SN, the PDCP SN field should be included in the PDCP header.

Observation 1. The D/C field and the PDCP SN field should be included in the PDCP header.

In order to reduce the PDCP header, the only way is to introduce the small PDCP SN, i.e., 7bit PDCP SN, as in LTE. However, introducing the small PDCP SN would lead to the window stalling due to window size and HFN de-synchronization.

Observation 2. Introducing the small PDCP SN would lead to the window stalling and HFN de-synchronization.

2. RLC header

In order to reduce the RLC header, the RLC TM may be considered. Unlike RLC UM and RLC AM, the RLC TM is used to carry the RLC PDU without RLC header. In other words, L2 header can be reduced if RLC TM is used for the transmission of VoNR.

However, if the RLC TM is used, it will make the scheduling less flexible and less efficient because the segmentation is not supported in RLC TM. To avoid RLC segmentation, the network should always provide UL grant with enough size to contain the VoNR traffic without segmentation.

Observation 3. Introducing RLC TM for VoNR would make the scheduling less flexible and less efficient.

3. MAC header

The MAC subPDU header is composed of the LCID field and L field. The LCID field is used to support the multiplexing. Considering that multiple MAC SDUs from different LCHs and multiple MAC CEs can be multiplexed in the MAC PDU, the LCID field should be included in a MAC PDU. The L field indicates the length of a MAC SDU. It means that the L field can be removed only when the MAC SDU size is fixed.

However, in ROHC, the length of the packet compressed by the ROHC is not fixed. This is because the ROHC compressor periodically transmits the full header information to the ROHC de-compressor in order to update ROHC context even if the ROHC context is synchronized between the ROHC compressor and de-compressor. As long as the MAC SDU size is not fixed, the L field cannot be removed.

Observation 4. The LCID field and the L field should be included in the MAC sub-header.

In summary, based on the above observations, there is no room to reduce the L2 header for VoNR. In addition, during the 119bis#103 offline discussion, the majority view is not to study the L2 header reduction because the L2 header reduction has a large impact on the specification and the benefit is marginal.

Proposal 1. RAN2 do not study the L2 header reduction in NTN.

Conclusion

In this contribution, we show our views on whether the L2 header reduction should be studied and the following observations and proposal are made.

Observation 1. The D/C field and the PDCP SN field should be included in the PDCP header.

Observation 2. Introducing the small PDCP SN would lead to the window stalling and HFN de-synchronization.

Observation 3. Introducing RLC TM for VoNR would make the scheduling less flexible and less efficient.

Observation 4. The LCID field and the L field should be included in the MAC sub-header.

Proposal 1. RAN2 do not study the L2 header reduction in NTN.