3GPP TSG-RAN WG2 Meeting #120	R2-2212780

# Toulousse, France, 14th- 18th November, 2022

Agenda Item:	8.19

Source:	Ericsson

Title:	Discussion on long eDRX cycle support for RRC_INACTIVE

Document for:	Discussion, Decision

# 1	Introduction

SA2 has sent an LS to RAN2 and RAN3 on Rel-18 RedCap study "FS_REDCAP_Ph2" regarding the support of long eDRX cycles, i.e., > 10.24sec, in RRC_INACTIVE state. SA2 concluded the following for the normative phase [1]:

SA2 thanks RAN3 and RAN2 for the answers provided in S2-2208117 (S2-2208098/R3-225119) and S2-2208145 (R2-2209243). Based on the answer provided by RAN3 and RAN2, SA2 has concluded to support the CN based MT communication handling for UE in RRC_INACTIVE with eDRX (>10.24s) as following. - It is agreed to support MT data and signalling handling within the CN when the UE is unreachable due to long extended DRX in RRC inactive. - The gNB sends an indication to the CN to handle MT communication while the UE is in RRC_INACTIVE state and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state). This allows the CN to apply the HLcom functionality (e.g. data buffering, notifications to other network and application functions etc) based on the unreachability information the gNB provided (as described in CR3705 and CR3555). - If the gNB has indicated the UE has entered RRC_INACTIVE to the CN, the gNB also notifies the CN about the RRC State transition back to RRC_CONNECTED (as described in CR3555). NOTE 1: If the indication of UE transition to RRC_INACTIVE is not sent (or sent after UE has entered RRC_INACTIVE) by the gNB then until CN receives it the CN cannot apply HLcom functionality and other NFs will not be aware of the UE reachability, and certain HLcom related services provided to the AF via NEF would not be available. Downlink data transmitted from the UPF to RAN might be discarded and not delivered to the UE. SA2 is currently further discussing if the conditions for gNB to send request for CN based MT communication handling can be implementation based (e.g, CN based MT communication handling is always triggered when gNB decides to apply long eDRX for RRC_INACTIVE, or gNB may decides not to trigger such in certain conditions, e.g., if UE is static and has uplink data only). SA2 has agreed the attached CRs for the support of eDRX for RRC_INACTIVE state with MT communication handling in CN. In the 23.502 CR, there are new flows introduced with impact on NGAP messages (see ENs).

In this contribution we discuss the outcome of the discussion in SA2 from RAN2 standpoint.

# 2	Discussion

## 2.1	General

The figure below [3] presents the SA2 solution with NG-RAN initiated connection:

- Figure 4.8.1.1a-1: NG-RAN initiated Connection Inactive procedure with CN based MT communication handling

One can observe that SA2 solution has impacts on procedures/signaling between AMF and NG-RAN., e.g., in steps 0, 2 and 3, which requires further discussion in RAN3. For example, a new IE is needed in the NGAP message from AMF to the NG-RAN which indicates that CN supports the Rel-18 solution to handle the MT communication when the UE is in RRC_INACTIVE state. This is to assist NG-RAN to distinguish between legacy AMF’s and AMF’s that support CN buffering.

SA2 solution has impacts on procedures/signaling between AMF and NG-RAN which requires further discussion in RAN3.

Another aspect is the need to inform AMF about the eDRX cycle NG-RAN has configured in RRC_INACTIVE. AMF is currently informed about the RRC state of the UE, but there seems to be a need to provide information regarding the eDRX cycles NG-RAN has configured along with maybe the size of the PTW for eDRX cycles longer than 10.24 sec.

There seems to be a need to provide information regarding the eDRX cycles NG-RAN has configured along with maybe the size of the PTW for eDRX cycles longer than 10.24 sec.

Discuss whether size of the PTW for eDRX cycles longer than 10.24 sec should be provided to the CN.

Based on such information from NG-RAN, AMF can deduce for how long a UE would be not reachable when configured with a long eDRX cycle to enable data buffering at UPF. NG-RAN should also receive an acknowledgment from AMF regarding the eDRX information, and therefore when it would not be able to reach the UE. Otherwise, NG-RAN may release the UE to RRC_INACTIVE state without knowing if HLcom benefits will apply.

# 3	Conclusion

In this contribution we discussed the outcome of the discussion in SA2 from RAN2 standpoint. We made the following observations:

Observation 1	SA2 solution has impacts on procedures/signaling between AMF and NG-RAN which requires further discussion in RAN3.

Observation 2	There seems to be a need to provide information regarding the eDRX cycles NG-RAN has configured along with maybe the size of the PTW for eDRX cycles longer than 10.24 sec.

Based on the discussion in the previous section we propose the following:

Proposal 1	Discuss whether size of the PTW for eDRX cycles longer than 10.24 sec should be provided to the CN.

# References

R3-226170, LS On long eDRX support for RRC_INACTIVE, SA2

S2-2209675, Support of RRC_INACTIVE with long eDRX, CR to TS 23.501

S2-2209583, Support of RRC_INACTIVE with long eDRX, CR to TS 23.502

R3-224800, Propositions for RAN3 feedback to SA2 on Rel-18 RedCap enhancement study (Ericsson)

R3-225401, RRC_Inactive with eDRX Notification for RedCap Ph2 (Qualcomm India Pvt Ltd)