3GPP TSG-RAN2 Meeting #120	R2-2212783

Toulouse, France, 14th - 18th November 2022

Title:	draft Reply LS on XR and Media Services on Network exposure

Response to:	R2-2211041 (S2-2209979) on LS on XR and Media Services

Release:	Rel-18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	Xiaomi (to be RAN2)

To:	SA2

Cc:	RAN1, RAN3

Contact Person:

Name:	Yanhua Li

E-mail Address:	liyanhua1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for their XR and Media Services review.

RAN2 would like to provide the following feedback on the points raised by SA2:

- SA2 point #1: In KI#3 (Network exposure), SA2 has been studying what information is useful for the purpose of enablement of rate adaptation at application and how that can be exposed by 5GS to the server and agreed the conclusions in TR 23.700-60 clause 8 (see pCR S2-2209977 and S2-2209978). The purpose of rate adaptation is to reduce the influx of data to keep the buffer/queue length level low which gives low latency.

Two variants of L4S marking are considered: (1) L4S marking in the NG-RAN node and (2) L4S marking by the PSA UPF based on information provided by NG-RAN. SA2 would like to ask RAN2 and RAN3 feedback on the following questions:

  - Q1: whether it is feasible for RAN to estimate congestion information per QoS flow, per DRB in downlink and uplink directions.

  - Q2: whether it is feasible for RAN to estimate congestion information per QoS flow in UL, per DRB in UL without UE impacts.

RAN2’s feedback on point #1:

For Q1, RAN2 think that it is feasible for RAN to estimate congestion information per QoS flow, per DRB in downlink and uplink directions.

For Q2, regarding UL congestion detection, RAN2 think that it is feasible for RAN to estimate congestion information per QoS flow, per DRB with the “remaining time” reported from UE, which is already agreed to be further studied in RAN2 119bis-e meeting (the agreements are shown below in bold), and RAN2 is discussing the details on this question.

Delay information consists of at least “remaining time”.

2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

2. Actions:

To SA2 group.

ACTION: 	RAN2 asks SA2 to take the above feedback into account.

3. Date of Next RAN2 Meetings:

RAN2#121                                       27th February - 3rd March 2023		Athens, Greece

RAN2# 121-bis-e                             17th Apr – 26th Apr 2023, 			e-Meeting