3GPP TSG-RAN WG2 Meeting #120	R2-2212795

Toulouse, France, 14th-18th November 2022

Agenda item:	8.14.2

Source:	China Telecom

Title:	Discussion on QoE measurements in RRC_IDLE and RRC_INACTIVE

Document for:	Discussion and Decision

- Introduction

In last few meetings, RAN2 and RAN3 have discussed the QoE configuration and reporting in RRC_IDLE and RRC_INACTIVE state for MBS, and made some progress. Based on it, this paper aims to analyse the remaining issues regarding to the QoE measurements and provides some considerations.

- Discussion

  - QoE configuration for MBS services in RRC IDLE/INACTIVE

As for QoE configuration for MBS services in RRC_IDLE and RRC_INACTIVE, the following consensuses have been made in RAN2 #119e meeting:

- 2: The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service. - FFS if configuration can be done in IDLE/INACTIVE states. - FFS how does gNB determine which UEs can be configured with MBS QoE measurements.

In RAN3 #117e meeting, the following agreements have been reached:

- Support MBS broadcast service INACTIVE/IDLE QoE first. - UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE. - If the UE receives the configuration in RRC connected state, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast service for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline.

As depicted above, RAN3 has already agreed that a common MBS-related QoE measurement configuration can be used for all states. Therefore, it makes sense to reuse the current QoE measurement configuration for RRC_CONNECTED state to RRC_IDLE/INACTIVE state. When the UE receives the QoE configuration (such as RRCReconfiguration message) in RRC connected state, the UE stores the QoE configuration and uses it in all RRC state.

Proposal 1: The QoE configuration received in RRC connected state can be used in all RRC state.

  - QoE Measurement Reporting for MBS services

In RAN2#119-e meeting, how the UE reports the Inactive/Idle QoE reports to gNB was discussed and left the following issue.

- 1: The baseline principles for QoE measurement collection for MBS services in RRC_INACTIVE and RRC_IDLE states are: 3) FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons.

In our understanding, QoE data is not time sensitive and high priority task. If UE needs to setup/resume RRC connection just for QoE reporting, frequent QoE reporting may cause extra power consumption. Therefore, it is suggested that UE can only report the IDLE/INACTIVE QoE reports when it moves to RRC_CONNECTED state due to other reasons.

Proposal 2: UE can only report the IDLE/INACTIVE QoE reports when it moves to RRC_CONNECTED state due to other reasons.

  - Buffering of QoE reports

The issues pertaining to buffering of QoE reports for MBS service have been discussed and reached the following agreement:

- 4: For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 should discuss at least the minimal memory size requirement. FFS if AS layer is responsible for storing the QoE reports (as in Rel-17).

When the UE receives the configuration of QoE measurements for MBS broadcast service, UE AS layer forwards the QoE configuration to the UE APP layer, and the APP layer performs the QoE measurements. In Rel-17, when QoE measurement collection is paused, the UE temporarily stores the APP layer measurement reports in AS layer with 64KB buffer size.

To reduce complexity, the QoE reports generated in RRC IDLE/INACTIVE state can also be buffered in AS layer. If the QoE reports exceed 64KB, there are two alternatives to handle the QoE data:

- Alt 1: AS layer discards the QoE data

- Alt 2: APP layer is responsible for storing the QoE data

Proposal 3: The QoE reports generated in RRC IDLE/INACTIVE state can be buffered in AS layer with 64KB buffer size. If the QoE reports exceed 64KB, RAN2 can discuss the following two alternatives:

•	Alt 1: AS layer discards the QoE data

•	Alt 2: APP layer is responsible for storing the QoE data

- Conclusions

The paper analyses remaining issues regarding the QoE measurements in RRC_IDLE and RRC_INACTIVE, and concludes the following proposals:

Proposal 1: The QoE configuration received in RRC connected state can be used in all RRC state.

Proposal 2: UE can only report the IDLE/INACTIVE QoE reports when it moves to RRC_CONNECTED state due to other reasons.

Proposal 3: The QoE reports generated in RRC IDLE/INACTIVE state can be buffered in AS layer with 64KB buffer size. If the QoE reports exceed 64KB, RAN2 can discuss the following two alternatives:

•	Alt 1: AS layer discards the QoE data

•	Alt 2: APP layer is responsible for storing the QoE data

- References

- RP-221803: Enhancement on NR QoE management and optimizations for diverse services